CHAPTER 1

GENERAL

Version:A

Key words: Breadth 27.5m, Gearless, 6S60ME-C10.5 Tier II, 220 Reefer Plugs, Open loop scrubber

CONTENT

CHAPTER 1 GENERAL	3

1.1      GENERAL PROVISIONS	3

1.1.1	GENERAL	3

1.1.2	design AMBIENT TEMPERATURE	5

1.1.3	MATERIAL AND WORKMANSHIP	7

1.1.4	STANDARD	8

1.1.5	MEASUREMENT UNIT AND LANGUAGE	8

1.1.6	BUYER'S SUPPLY	9

1.2	   DESCRIPTION OF THE VESSEL	12

1.2.1	GENERAL ARRANGEMENT	12

1.2.2	intended cargo	13

1.2.3	principalDIMENSIONs	15

1.2.4	DECK HEIGHT, SHEER AND CAMBER	15

1.2.5	DEADWEIGHT, AND CAPACITY	16

1.2.6	COMPLEMENT	17

1.2.7	main engine, propellerAND SPEED	18

1.2.8	m/e FUEL CONSUMPTION	18

1.2.9	CRUISING rANGE	19

1.2.10	FREEBOARD	19

1.3      RULES AND REGULATIONS	20

1.3.1	CLASSIFICATION	20

1.3.2	FLAG	20

1.3.3	RULES AND REGULATIONS	20

1.3.4	guidelines and recommendations	22

1.3.5	CERTIFICATEs	23

1.4      PROPERTIES OF THE VESSEL	27

1.4.1	MODEL TEST	27

1.4.2	deadweight and lightweight	27

1.4.3	TRIM AND STABILITY	28

1.4.4	VIBRATION AND NOISE LEVEL	29

1.5	   INSPECTION, TESTING AND TRIAL	31

1.5.1	GENERAL	31

1.5.2	INSPECTION AND TEST OF CONSTRUCTION	31

1.5.3	SHOP TESTS	32

1.5.4	MOORING TEST and lifting test of container	32

1.5.5	INCLINING TEST AND lightWEIGHT MEASUREMENT	32

1.5.6	QUAY TRIAL and sea trial	33

1.6	   DOCUMENTATION	37

1.6.1	PLANS FOR APPROVAL	37

1.6.2	FINISHED DRAWINGS	37

1.6.3	EXHIBITION MODEL	38

1.7	    DRY DOCKING	38

1.8      LOAD instrument	39

CHAPTER 1 GENERAL

1.1      GENERAL PROVISIONS

1.1.1	GENERAL

It is the intent of this Specification and accompanying plan(s) to describe and set forth the design, performance, capacities, construction, equipment and material required for the construction of a Container Vessel (hereinafter called the “Vessel”).

Following accompanying plan(s) shall form an integral part of this Specification:

|  | # General Arrangement Plan | # Drawing No. HP4102-100-04 |
| --- | --- | --- |
|  | # Midship Section Plan | # Drawing No. HP4102-110-03 |
|  | # Maker's List No.GW4001-014-03 | # Drawing No. HP4102-014-03 |

This Specification with above mentioned accompanying plans shall form an integral part of the Building Contract of the Vessel.

Details in design, construction, installation, inspection, test and workmanship not covered by this Specification and the accompanying plan(s) shall be performed under the working plans and Builder’s practice. List of Builder’s standards and practices to be submitted to the Buyer for reference at the time of plan approval.

The Builder shall furnish the Vessel with all items required by the Rules and Regulations as specified in section 3.3 “Rules and Regulations” and 3.4 “Guidelines and Recommendations”, except items specifically stated herein as shall be furnished and supplied by the Buyer.

Any items repeated by mentioning twice or more in this Specification shall be supplied and/or applied only once. Any items, which are not called for explicitly in this Specification, shall not be furnished nor supplied by the Builder. Any items missing but required by Classification Society or Regulatory Bodies concerned which set force at the date of Contract signing, shall be supplied and/or equipped by the Builder.

Any requirement of the Buyer, which is in excess of the requirements of Rules and Regulations as specified in Section 3.3 “Rules and Regulations” and 3.4 “Guidelines and Recommendations” and/or the content of this Specification, shall be subject to the adjustment to the Contract price, scheduled delivery date, contract speed, deadweight, homogeneous intake and any other items which may be affected.

If any inconsistency is found between the Building Contract and this Specification and/or this Specification and accompanying plans, the former shall prevail in any case.

The description in CHAPTER 1 “General” shall be applied to all parts of this Specification, whereas in the other parts the descriptions shall be applied the respective parts, if not stated otherwise. In the event of any conflict or inconsistency between the parts of this Specification, the hull part shall prevail in respect of hull items, the outfitting part shall prevail in respect of outfitting items, the machinery part shall prevail in respect of machinery items and the electrical part shall prevail in respect of electrical items.

In the event of any inconsistency between the description in the Specification and Rules and/or Regulations listed in the Specification, Rules and/or Regulations shall prevail.

If during approval of plans, both parties agree to a design which differs from the Specification, it is understood that terms in the Specification concerning these alterations shall be no longer applicable, and consequently in these cases the approved drawings shall be supplementary to the Specification.

Any alterations of Rules and Regulations as specified in Part 1, Section 3.3 and Section 3.4 of this Specification coming into effect between the date of Contract signing and ship’s delivery may be carried out subject to adjustment to the Contract price, Contract guaranteed technical data (Speed, Deadweight, Homogeneous intake, etc.), delivery date and any other item(s) which may be affected.

Any amendments and/or modifications in this Specification and/or Contract Plan(s) to be treated similar to above.

If the Buyer desires to apply the said amendment and/or modification to the Vessel, the Buyer will inform the Builder in written document, and the Builder shall submit a written document about their effect on the Vessel's Contract price, Contract guaranteed technical data (Speed, Deadweight, Homogeneous intake, etc.), delivery date and any other item(s), which may affect the Contract.

The amendments and/or modifications shall come into effect after agreement has been reached between the Buyer and the Builder and a written consent has been signed between both parties.

Amendments and/or modifications shall be recorded in a memorandum or a written document signed by the Buyer and the Builder. These documents shall be a supplement to the Contract Specification.

Whenever the terms "or equal" or “or equivalent” is used after a brand or a type of machinery equipment or component, the term "or equal" or “or equivalent” means that the substitute shall be of equivalent standard and quality and should not affect the contracted performance or compliance of the revised system. The Buyer must agree to such a change.

Wherever the terms such as “or”, “and/or”, “similar”, “where necessary”, “if any”, “may” and other similar expressions are mentioned in this Specification, application of alternative item(s) shall be respected to opinion of the Builder. And the result will be

informed to owner.

Unless otherwise stipulated by the Contract and/or specifically described in this Specification, all figures stated in this Specification shall be understood as approximate ones and determined in accordance with the design development except those which are of countable nature.

Spare Parts, tools and accessories shall be furnished as per rules of Classification Society and Regulatory Bodies concerned and in accordance with the Makers’ normal standard or as listed throughout these Specifications. Any additional spares required by the Buyer shall be supplied by the Builder on the Buyer’s account unless otherwise agreed or stated herein.

Spare parts, Tools and accessories shall be properly protected and stored in a way which guides easy access and control depending on its type and size. However, heavy components of main engine to be arranged under overhead crane working range. Steel cabinets, shelves and drawers shall be used for storing spare parts in accordance with the Builder’s practice and recommendations of the Makers.

During the tests and trials, the Builder may use some of the supplied spare parts, tools and accessories to the Vessel if necessary subject to Buyer’s approval, and the Builder shall furnish them back to the Vessel upon ship’s delivery if deemed critical by Buyers.

The Builder may make changes and improvements to the Specification, if found necessary in good condition, provided that the Builder shall first obtain the Buyer's approval which shall not be unreasonably withheld.

1.1.2	design AMBIENT TEMPERATURE

If not otherwise stated, the equipment and machinery to be installed and operated in accordance with following ambient conditions:

| Outside air temperature | –19oC ~ + 35oC |  |
| --- | --- | --- |
| Sea Water temperature | 0oC ~ + 32oC |  |
| Reference conditions for machinery equipment: | Max. air temperature in engine room: + 45oC L.T. cooling F.W. temperature: + 36oC Barometric pressure: 0.1 MPa Relative humidity: 60% |  |
| Reference condition for HVAC – plant for accommodation: | Summer cycle Outside atmosphere: Inside atmosphere: Winter cycle Outside atmosphere: Inside atmosphere: | +35oC;70% R.H. +27oC; 50% R.H. –20oC; 50% R.H. +20oC; 50% R.H. |

Note:

1.R.H.: Relative humidity

2. For the materials of hull steel of structures exposed to low air temperatures shall be based on the design temperature tD (the lowest mean daily average air temperature in the area of operation) of -10℃. (refer to IACS UR S6) (Rev 9 July 2018)

1.1.3	MATERIAL AND WORKMANSHIP

Materials supplied and used for construction of the Vessel shall be of such quality as approved by the Classification Society where required.

Ordering of materials, machinery and equipment shall comply with this Specification and Maker List.

All structural steel used for the construction of the vessel including forgings and castings to be of the shipbuilding and marine engineering quality, tested, inspected and certified as required by the Classification Society. In addition, steel plates certificates to be provided to the Site Team before inspection by group. the first group will submit before steel cut.

For any materials, machinery and equipment specified in the Maker List, selection of the

final Makers shall be as per the signed Maker List.. For other materials, machinery and equipment not specified in the Maker List, the Builder shall have the right to decide the Maker and/or supplier according to Builder’s own knowledge and experience.

Manufacturer’s standard type, size and material of machinery, equipment and fittings shall be adopted unless otherwise described in this Specification.

The manufacturer or Maker of equipment has the right to improve or update the production after the contract has been made between the Maker and the Builder which shall not be unreasonably withheld by the Buyer.

The Builder may employ sub-contractors for carrying out works of the Vessel at the

Builder’s responsibility and the Builder shall guarantee the work / quality of subcontractors.

If any materials and equipment described in this Specification is not available, the Builder shall substitute equivalent materials and equipment capable of satisfying the

requirements of Classification Society and relevant Regulations and subject to prior

acceptance of the Buyer.

If any of the specific description for the machinery or equipment in this Specification is not consistent with the finally selected manufacturer’s specifications, such description shall be readily corrected to follow the manufacturer’s specification.

Wherever stainless steel is mentioned in the specifications and unless otherwise specified,

this means quality of SUS304, or corresponding Maker’s standard may be applied. However, SS 301 shall not be used. In case of maker standard, quality shall be SUS 304 or above.

Workmanship shall be carried out in accordance with the descriptions in this Specification, China Shipbuilding Quality Standard (CSQS) (GB/T34000-2016) and Builder's Engineering Standard.

Asbestos, Refrigerant (R 12 / R 22), HALON, Lead-based paint and Tin-based antifouling paint shall not be used.

All wood to be well seasoned and free from defect.

1.1.4	STANDARD

The following standards shall be applied to the construction of the Vessel.

1)	Chinese Industrial Standard (GB, GB/T, YB)

2)	Chinese Shipbuilding Standard (CB, CB/T, CBM)

3)	China Shipbuilding Quality Standard (CSQS)(GB/T34000-2016)

4)	Builder's Engineering Standard and Builder's standard practice or Manufacturer’s standards unless otherwise described in the Specification.

5)  ISO standard

6)  IACS Rec.47 Shipbuilding and Repair Quality Standard(Part A)

7)  JIS or DIN standard

The connection dimension of flange shall be in accordance with ISO standard except those flange attached on the imported equipment, which shall be as per Maker's standard.

The grease nipples shall be of flat type (CB/T 3762-1996 eqv ISO7824-1986, Material: brass) or the Maker's standard which is provided by the equipment supplier as a package.

In addition to above, other standards may be applied for the items or equipment which is described hereinafter in this Specification except those grease nipples attached on the imported equipment, which will be as per Maker's standard.

1.1.5	MEASUREMENT UNIT AND LANGUAGE

Metric system shall be adopted for design and construction of hull, machinery and equipment unless specially specified in the Specification.

All measuring units such as power, pressure gauge, thermometer, volume gauge, and tank scales, etc., shall be of metric system. However, some conventional units are also to substitute the corresponding SI units or metric units as common customs in shipbuilding and shipping metric threads to be generally applied to the Vessel, however, the British measurement also may be applied where ISO is not adopted. For pressure, MPa or bar shall be used throughout.

Types, sizes and materials of fittings, equipment and machinery to be in accordance with ISO or equivalent standard. Where no applicable ISO or national standard exists, the Builder's standard and/or Manufacturer's standard may be used.

The draught of the Vessel specified in this Specification shall be of moulded figure unless specifically stated.

All necessary documents and drawings, calculations, instruction manuals, etc. to be written in English or in both English and other language (English part shall prevail) for approval by the Buyer, Classification Society and Authorities. All final plans and drawings shall be in English or in both English and other language.

All plans, booklets, name plates, caution/identification plates and whatever documents required for implementation of the Specification shall be in English.

If any language other than English is required by any reason whatever, the text of translation shall be provided by the Buyer.

1.1.6	BUYER'S SUPPLY

Following items including their accessories and spares to be furnished and supplied by the Buyer at Buyer’s own expense, and installed on the Vessel by the Builder, at the time designated by the Builder to meet the Builder’s construction schedule of the Vessel.

The Buyer shall furnish the necessary specifications, plans instruction books, manuals, test reports, test certificates and the manufacturer’s service engineer(s) etc. of the listed items as required by the Builder, which shall form an integral part of the Buyer’s supplies.

The supervision fees related to final commissioning, inspection and certifications of the Buyer’s supply items shall be paid by the Buyer. The Buyer shall be responsible for the adjustment, inspection and certifications of the Buyer’s supply items with Builder’s assistance.

The Builder shall be responsible for storing and handling of Owner's supply items after delivery to the Builder's yard and shall store those items in the Vessel.

The Buyer’s supply items shall be as follows:

- Charts in general and all nautical books (nautical almanacs, international signal books, ITU books, register tide table, etc.).

- Electronic charts for ECDIS.

- Flags, signals and nautical instruments in excess of those specified in the Specification.

- Hobby equipment such as T.V, broadcasting video, etc. in excess of those specified in Specification.

- Steel wire, mooring ropes, towing rope and ETA fittings in excess of Class Rules and Flag Authority and those specified in the Building Specification.

- Personal computers and printers with peripheral devices for computer network system. Computer network cables and sockets to be provided by Builder.

- Painting and pictures including frames and other decorations. (Builder to frame and position).

- Suez Canal searchlight excluding bridge wing search light (the foundation, power socket and manual operated davit for Suez Canal search light shall be installed by the Builder).

- All office machine including copy machine, fax machine and stationery including pencils, rulers, papers, etc.

- All gymnastic apparatus.

- All medicine and medical equipment.

- Galley and pantry utensils including pans, cutlery, crockery, and glassware.

- Linen including blanket, carpets, bed sheets, pillows, towels and table cloth, etc.

- Bedding except mattresses.

- Brooms, brushes, vacuum cleaners, and waste baskets, etc. for accommodation and all portable hoses and clean devices not mentioned in the Specification.

- Boatswain’s and carpenter’s tools and inventory other than those specified in the Specification.

- Fuel oil (HFO,LSHFO / ULSFO/MGO) for actual service after delivery at Builder yard.

- Lubrication oil, greases and operating oils of all machinery and equipment for actual service after ship’s delivery at Builder’s shipyard. However lubrication oil, greases and operating oils shall be purchased by the Buyer on behalf of the Builder prior to the tests and trials, and the Builder shall calculate and pay back the cost based on the quantity consumed during the tests and trials in accordance with the Contract.

- Consumable stores including sanitary and all chandlery (soap, etc.).

- Boiler feed water test kit and chemical compounds for the treatment of feed water and cooling fresh water. And test kit for fuel oil and lubrication oil, hydraulic oil.

- Cable for power feeding circuits between reefer container and receptacle.

- Container loose fittings

- Container spreader, if necessary.

- Garbage compactor.

- Cable for electrical power shore connection.

- Test kit for Class notation on shaft condition monitoring.

- Portable atmosphere testing instrument for measuring concentration of oxygen, flammable gases or vapors, hydrogen sulphide and carbon monoxide.

- Portable toilet, potable water, cable and antenna of the Iridium radio for the anti-piracy citadel.

- Fire ropes for Suez Canal

- Ship’s local area network

- All hoses, etc., other than those required by the Rules, or mentioned in these specifications.

- All other spares, store and equipment in excess of Rule guidelines, Maker’s standard and those listed in the Specification.

- VSAT equipment (Builder to install).

The Buyer to supply following documents approved by Classification Society with the Builder’s assistance:

- Training Manual

- Fire fighting fixed and portable system manual and maintenance manual, and train manual

- -Garbage Management Plan and Garbage Record Book

- - Safety Management Certificate (SMC) and Document of Compliance (DOC) for deck, engine and cabin stores supplied by the Buyer.

- -	Minimum Safe Manning Document

- -	Container Securing Manual(CSM)

- Ship Energy Efficiency Management Plan (SEEMP) (Builder to assist Buyer with necessary plans)

- International Ship Security Certificate (ISSC) and Ship Security Plan (SSP) covering the requirement of ISPS Code except Automatic Identification System (AIS), ship security alert System (SSAS) and Ship’s Identification number (IMO number)

- -	Fuel Oil Changeover Procedure and Logbook

- -	Ship-specific Plans and Procedures Recovery Persons from the Water

- -	Pilot Card

- -	Maneuvering Booklet

- -	Muster List

Emergency Towing Procedure and Maneuvering Booklet, Shipboard Oil Pollution Emergency Plan (SOPEP), Inventory of Hazardous Materials (IHM) to be provided by builder with Buyer’s assistance. The buyer provided a sample.

1.2	   DESCRIPTION OF THE VESSEL

1.2.1	GENERAL ARRANGEMENT

The Vessel to be designed and constructed as a full cellular container carrier for unrestricted world-wide ocean going service (except NOx ECA Tier III area and other areas which are stated or limited by the rules and regulation in section 1.3.3, the design and equipment condition of the specification), suitable for carrying dry cargo containers, reefer containers and specified dangerous cargo in closed containers within the scope of the Specification.

The vessel to be an ocean-going gearless container vessel with a single-acting two stroke turbo charged diesel engine, a fixed pitch propeller and a semi-spade rudder. .

The vessel is not service for NOx ECA (Tier III) area and do not comply with the rules and regulations regarding NOx Tier III. The vessel shall be built on the basis of Tier II regulation with the certificate of International Air Pollution Protection Certificate (IAPP) and Statement of compliance with the NOx technical code for main & aux. engine (EIAPP) in compliance with Tier II criterion.

Open loop scrubber to be provided to satisfy the requirements of Global Sulfur Cap 2020 (option requirement of SECA).

HFO (< 3.5% Sulphur), LSHFO (< 0.5% Sulphur)/ULSFO (< 0.1% Sulphur) and MGO(< 0.1% Sulphur) to be loaded on board.

ULSFO (< 0.1% Sulphur), MGO (< 0.1% Sulphur) shall be adopted for on SOx Emission Control Area (SECA).

The vessel to have a continuous main deck with a detached forecastle, eight (8) tiers of deck house.

The propulsion machinery and living quarters including navigation bridge shall be located semi-aft as shown on the General Arrangement Plan (hereinafter called as the "G.A. plan").

Containers, cargo holds, ballast water tanks, fuel oil tanks, fresh water tanks, bow thruster room, steering gear room and engine room etc. shall be arranged as shown on the G.A. plan.

The Vessel shall have four (4) cargo holds. each cargo hold have two (2) hatches each.

The double hull and double bottom space shall be arranged as ballast water tanks or void space. Fore peak tank shall be designed as ballast tank. Aft peak tank shall be designed asballast tank. One pair of anti-heeling tanks to be arranged at midship area port and starboard sides as shown on GA plan.

Double hull construction shall be provided for all cargo holds. A double bottom shall be fitted extending from the collision bulkhead to the aft peak bulk head .

Cargo holds to be designed to have access from upper deck to cargo hold walkways and bilge wells in the aft end. Stringer to be provided as walkway location for each 8’6’’ standard container tiers in hold.

One (1) pipe duct shall be provided in double bottom from aft end of No.2 cargo hold compartment to fore end of engine room. Small hatch covers in place in fore and aft end of pipe duct.

Pipe duct to be fitted with trolley and sufficient lighting to be also provided.

One (1) set of bow thruster to be installed in bow thruster room..

Mooring facilities shall be arranged as shown on the G.A. plan.

1.2.2	intended cargo

The Vessel shall be designed for carrying container in all cargo holds and on deck.

The types and dimensions of dry cargo containers to be shown as below:

Type of container

| Type | Height ft-inches | Width ft-inches | Length ft-inches |
| --- | --- | --- | --- |
| ISO-1AA | 8 - 6 | 8 | 40 |
| ISO-1CC | 8 - 6 | 8 | 20 |
| ISO-1AAA | 9 - 6 | 8 | 40 |
| ISO-1EEE | 9 - 6 | 8 | 45 |

Dimension of container

| Type | Height |  | Width |  | Length |  | Weight |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (mm) | Tole-rance (mm) | (mm) | Tole-rance (mm) | (mm) | Tole-rance (mm) | Gross Max. (MT) |
| ISO-1AA | 2,591 | +0 –5 | 2438 | +0 –5 | 12,192 | +0,-10 | 30.48 |
| ISO-1CC | 2,591 |  |  |  | 6,058 | +0,-6 | 30.48 |
| ISO-1AAA | 2,896 |  |  |  | 12,192 | +0,-10 | 30.48 |
| ISO-1EEE | 2,896 |  |  |  | 13,716 | +0,-10 | 30.48 |

Dangerous goods

Dangerous goods can be carried in closed freight containers in hold and on deck as qualified as follows.

| Loading space IMO Class | In cargo hold No.1 *1) | In cargo hold No.2 ~ No.4 *2,4) | On hatch covers *3) | Above E/R |
| --- | --- | --- | --- | --- |
| 1.1 to 1.6 explosives | X | X | P *4)7) | X |
| 1.4S | C | C | C | C |
| 2.1 including hydrogen and hydrogen mixtures | X | X | C | X |
| 2.1 excluding hydrogen and hydrogen mixtures | C | X | C | X |
| 2.2 |  | C | C | C | X |
| 2.3 flammable | X | X | C | X |
| 2.3 non-flammable | X | X | C | X |
| 3 F.P.<23℃ | C | X | C | X |
| 3 23℃≤F.P.≤60℃ |  | C | C | C | X |
| 4.1 |  | C | C | C | X |
| 4.2 |  | C | C | C | X |
| 4.3 liquids F.P.<23℃ | X | X | C | X |
| 4.3 liquids F.P.≥23℃ |  | C | C | C | X |
| 4.3 solids |  | C | C | C | X |
| 5.1 |  | C | C | C | P*5) |
| 5.2 | X | X | C | X |
| 6.1 liquids F.P.<23℃ | C | X | C | X |
| 6.1 liquids 23℃≤F.P.≤60℃ | C | C | C | X |
| 6.1 liquids non-flammable | C | C | C | C |
| 6.1 solids |  | C | C | C | C |
| 8 liquids F.P.<23℃ | C | X | C | X |
| 8 liquids 23℃≤F.P.≤60℃ | C | C | C | X |
| 8 liquids non-flammable having a subsidiary risk class 6.1 | C | C | C | C |
| 8 solids |  | C | C | C | C |
| 9 including goods evolving flammable vapour | X | X | C | X |
| 9 except goods evolving flammable vapour *6) | C | C | C | C |

Notes:

F.P.: Flash point

C: Intended for carriage

X: Not intended

P *x): Intended for carriage subject to *x)

*1) For cargo hold No.1, explosion-proof type fans to be of IIB-T4 grade or equivalent. If any dangerous cargoes require explosion-proof grade higher than IIB-T4, such cargoes cannot be loaded (i.e. hydrogen, hydrogen mixture, ethyl nitrite, ethyl nitrate, etc.).

*2) For cargo hold No.2~ No.4, fans to be of non-explosion-proof type.

*3) Reefer containers on weather deck to be stowed 3m away from the mechanical

ventilation outlet of dangerous cargo hold (No.1) where dangerous cargo is loaded.

*4) Stow 3m horizontally away from the machinery space boundaries and not be

stowed within a horizontal distance of 6 m from potential sources of ignition in all case.

*5) Except goods stowage away from sources of heat.

*6) On examination into nature of dangerous goods and equipment of this vessel, judge

whether is able to stow or not.

*7) Class 1 (except 1.4S) not to be stowed within a horizontal distance of 6 m from

potential sources of ignition in all case.

1.2.3	principalDIMENSIONs

Length over all (L.O.A.)	max.172.00m

Length between perpendiculars (L.B.P.)	approx.168.70 m

Breadth, moulded		     27.50m

Depth, moulded		     14.60 m

Design draught, moulded		8.50 m

Scantling draught, moulded		     10.00m

Total height (from baseline to top of antenna)	approx.49.00 m

Note: The length between perpendiculars may be adjusted according to hull form optimization results.

1.2.4	DECK HEIGHT, SHEER AND CAMBER

1.2.4.1	Height between Decks

| Upper deck to A deck | : | Approx. 3.30 m |
| --- | --- | --- |
| A deck to B deck | : | Approx. 3.10 m |
| B deck to bridge deck | : | Approx.5X2.90 m |
| Bridge deck to Compass deck | : | Approx. 3.00 m |

1.2.4.2	Sheer of Deck

Upper deck	Nil

Deckhouse	Nil

Forecastle deck	Approx. 0.15 m

1.2.4.3	Camber of Deck

| Deck | Camber |
| --- | --- |
| Upper deck | 100mm |
| Compass deck | Nil |
| Forecastle deck | Nil |
| Exposed part of Navigation bridge deck | Nil |
| Platform deck(s) in engine room | Nil |

1.2.4.4	Rise of Floor

Nil

1.2.5	DEADWEIGHT, AND CAPACITY

1.2.5.1	Deadweight

The deadweight of the Vessel shall be approx.23940 tons at scantling draught 10.0m, with even keel, with the vessel afloat in sea water with specific gravity of 1.025.

1.2.5.2	Container Capacity

| Capacity of Maximized Container |  |  |  |
| --- | --- | --- | --- |
|  | 20ft | 40ft | Total (equivalent to TEU) |
| On deck | 1245 |  | 1245 |
| In hold | 644 | - | 644 |
| Total | 1889 |  | 1889 |

| Capacity of Self-contained Reefer Container (ISO-1AAA type) |  |  |
| --- | --- | --- |
|  | Cooling type | 40ft high cube |
| On deck | Air cooled | 220 |
| In hold | Air cooled | 0 |
| Total |  | 220 |

- IMO visibility requirement at the scantling draught with even keel is considered in the above mentioned container capacity.

- Containers shall be stowed in maximum five(5) tiers by ten (10) rows in holds and maximum seven(7) tiers by eleven(11) rows on hatch covers and maximum seven(7)tiers by eleven (11) rows on upper deck.

- 45ft container (45’ x 8’ x 9’6”) could be stowed on top of 40ft container above hatch

covers where available as shown on the G.A. plan.

- Maximum clear height of cargo hold shall be based on three (3) tiers of 9’6” high cube container and two (2) tiers of 8’6” container.

- Reefer slots shall be provided in two (2) tiers on hatch covers and on upper deck as shown on the G.A. plan

- Number of tiers in the outmost rows of containers on hatch cover / upper deck may be adjusted according to Maker's lashing force calculation result.

- The final stack weight and weight distribution of the containers on upper deck /hatch covers shall be determined according to lashing force calculation result.

- Above mentioned number of containers may be decreased/increased in the course of the design development according to final hull lines and G.A. plan.

1.2.5.3	Container Homogeneous Intake

The number of loadable containers of homogeneous 14MT / TEU (ISO 20’ x 8’ x 8’6’’) at scantling draught 10.0m with moderate trim by stern (not more than 1.0% L.O.A.) shall be about 1332TEU with 100% consumables at departure and VCG of 45% container height.

1.2.5.4.	Tank Capacity

| Heavy fuel oil /low sulpher heavy fuel oil/Ultra low sulphur fuel oil tanks (including settling, service tanks) | approx.1,100 m3 |
| --- | --- |
| Marine Gas Oil tanks (including service tank) | approx.200m3 |
| Fresh Water tanks | approx.200m3 |
| Ballast Water tanks (including anti-heeling tanks) | approx.8300 m3 |

Notes:

- All the volumes of tanks in above table to be of geometric net volume without actual filling ratio considered.

- The volume of water ballast tanks specified above means only total geometric volume and some of the specified volume may not be utilized in ballast conditions to achieve proper trim and to avoid any excessive structural stress with the detail design development.

1.2.6	COMPLEMENT

| Dep. Name Class | Deck Dep. | Engine Dep. | Total |
| --- | --- | --- | --- |
| Captain Class | 1-Captain | 1-Chief engineer | 2 |
| Senior officer | 1-Chief officer | 1-Second engineer | 2 |
| Junior officer | 1-Second officer 1-Third officer | 1-Third engineer 1-Electrician 1-Fourth engineer | 5 |
| Petty officer | 1-Boatswain | 1-Chief Motorman | 2 |
| Rating | 5-Seaman 1-Cook | 5-Motorman | 11 |
| Others | 1-Pilot (Junior officer class) |  | 1 |
|  | 1-Owner (Junior officer class) |  | 1 |
| Total |  |  | 24 |
| Suez crew (not included in the complement ) |  |  | 6 |

1.2.7	main engine, propellerAND SPEED

1.2.7.1	Main Engine

| Model | MAN B&W 6S60ME-C10.5 Tier II |
| --- | --- |
| No. of set | One (1) |
| Tuning method | Low Load with EGB |
| M.C.R. | 14940KW x 105rpm |
| S.M.C.R. | 10380KW x 96 rpm |
| C.S.R. (90% S.M.C.R.) | 9342KW x 92.7 rpm |

1.2.7.2	Propeller

| No. of set | One (1) |
| --- | --- |
| Type | Four (4) or five (5) blades, fixed pitch, skew type and keyless |
| Material | Nickel-Aluminum-Bronze |

The propeller shall be designed at the power of ME CSR and revolution with about 4%-7% light running margin at design draught 8.50 m with clean hull and propeller in open calm sea.

Propeller strength shall be designed based on SMCR of M/E.

1.2.7.3	Service Speed

The design service speed is about18.5 knots, based on:

- On design draught 8.50 m

- Main Engine running at CSR (90% SMCR)

- With 15% sea margin

- Clean hull

- In open, deep sea with Beaufort 0, Douglas sea state 0

Energy saving design:

The rudder bulb to be designed optimized together with propeller. The hull lines, propeller and rudder with rudder bulb to be finally decided by towing tank test.

1.2.8	m/e FUEL CONSUMPTION

The designed M/E Daily fuel oil consumption (D.F.O.C.) is about 35.69t，based on:

The Brake Specified Fuel Consumption (B.S.F.C) of the main engine is about 159.0 grams/kWh (excluding M/E supplier’s tolerance) at CSR（9342kW） based on the use of fuel oil with L.C.V. of 42,700 kJ/kg at ISO 3046-1:2002(E) and ISO 15550:2002(E) ambient condition and fulfilling the IMO NOx emission limitation Tier II (ME back pressure max. 60mbar), which shall be determined by shop test.

Only Brake Specified Fuel Consumption (B.S.F.C) of the main engine based on above-mentioned conditions determined by shop test shall be Contract guaranteed.

In case the shop test conditions are different from the above, the results shall be corrected in accordance with the manufacture’s standards.

1.2.9	CRUISING rANGE

About 11,000n.miles, assuming:

-	ISO ambient conditions

-	Main Engine running at CSR (90% SMCR)

-  Service speed of 18.5 knots on design draught

-	Fuel oil consumption for main engine and one (1) set of generator engine power supply at basic hotel load (including tolerance 5% of SFOC)

-	Fuel oil with specific gravity of 1.01 tons/ m3 for HFO, 0.99 tons/m3 for LSHFO/ULSFO, 40,600kJ/kg in L.C.V. and 0.89 tons/ m3 for MGO, 42,707kJ/kg in L.C.V.

-	98% full of fuel oil tanks

-	Including all HFO,LSHFO/ULSFO /MGO storage, settling and service tanks

1.2.10	FREEBOARD

The Vessel shall be designed according to the requirement of the International Convention on Load Lines (ICLL), 1966 revised by protocol 1988 and all latest amendments for type "B" ship.

1.3      RULES AND REGULATIONS

1.3.1	CLASSIFICATION

The Vessel including its hull, machinery, equipment and outfitting shall be built under the supervision of Classification Society (hereinafter called the "Class") and shall be distinguished in the record by the notation of:

NK NS* (CNC, EQ C DG, PSPC-WBT, NC) (IWS) (PSCM) (EA) (CSSA-R) (IHM) (Sox(EGCS)) MNS*(M0)

Installation Character(s): BWM, MPP, LSA, RCF, M0, AFS

Loading computer will be approved by class.

However, the Class rule shall be applied with the latest version at the date of the Contract signing.

1.3.2	FLAG

The Vessel shall be registered under the flag of Marshall .

1.3.3	RULES AND REGULATIONS

The Vessel shall comply with the following Rules and Regulations including amendments which have entered into force upon the date of signing this contract and are compulsorily applied to the vessel.. (except Rules and Regulations regarding NOx Tier III).

- Maritime Regulation of Country or Region of Registry.

- Rules of the Classification Society.

- IACS UR-S11A Longitudinal Strength Standard for Container Ships and UR-S34 Functional Requirements on Load Cases for Strength Assessment of Container Ships by Finite Element Analysis.

- IACS URA1 Rev.6 for Anchoring equipment, URA2 Rev.4 for Towing/Mooring Equipment, URA3 for Windlass.

- International Convention on Load Lines 1966, including Protocol 1988 and Amendments.

- International Convention for the Safety of Life at Sea, 1974, and Protocol 1988,and Amendments.

- International Convention for the Prevention of Pollution from ships, 73/78 (Annex I, IV, V& VI) and Amendments.

(The requirement of NOx Tier III is not applied in the scope of this Specification.)

- International Regulations for Preventing Collisions at Sea (London, 1972) and all Amendments.

- International Convention for Tonnage Measurement of Ships 1969.

- International Convention on the control of Harmful Anti-fouling System on Ships, 2001.

- International Telecommunication Union Radio Regulation, 2008.

- IMO "Code on Alerts and Indicators, 2009". (Resolution A.1021(26))

- International Ship and Port Facility Security Code (Equipment only).

- International Maritime Dangerous Goods Code (IMDG Code).

- IMO Res. MSC.267(85), “International Code on Intact Stability, 2008 (IS code 2008)”(except Ch.1,2 Dynamic stability Phenomena in Waves).

- IMO Resolution MSC.1/Circ.1352, “Amendments to the code of safe practice for cargo stowage and securing (CSS Code)”.

- IMO Resolution MSC.337(91), “Adoption of the code on noise levels on board ships” (only mandatory part) (Except bridge wing).

- IMO Resolution MSC.339(91), “Amendments to the FSS Code”.

- IMO Resolution MSC.365(93), “Amendments to the International Convention for the Safety of Life at Sea (SOLAS), 1974, as Amended.”

- Maritime Labor Convention (MLC) 2006, Title 3, Regulation Standard A3.1 (except swimming pool).

- ILO Convention 152 Occupational Safety and Health (Dock Work) Convention, 1979 Including Register of Lifting Appliances and Items of loose gear - Model Form and Certificates as required by Article 25(2).

- U.S. Coast Guard Regulations Applying to foreign flag vessel operating in navigable waters of the United States (CFRTitle33 – Part151, 155,159and164, CFR 46 – Part 162), with neither certificate nor inspection.

        - CFR title 33 – part 151 and CFR 46- Part 162: Standard for Living Organisms in Ship’s Ballast Water Discharged in US waters (BWDS)

        - CFR title 33 – part 155: Oil Pollution Prevention Regulations of Vessels

        - CFR title 33 – part 159: Marine Sanitation Devices

        - CFR title 33 – part 164: Navigation Safety Regulation only for Section 164.35 Equipment

- U.S. Department of Labor Occupational Society and Health Administration (OSHA) safety and health regulation for Long shoring (CFT, Title 29, part 1918) regarding Containerized Cargo (Container lashing system) with neither certificate / statement of compliance nor inspection.

- Regulation (EU) No. 1257/2013 of the European Parliament and of the Council of 20 November 2013 on ship recycling and amending Regulation (EC) No. 1013/2006 and Directive 2009/16/EC(only Statement of Compliance).

- EU Directive 1999/32/EC, amended by 2005/33/EC and 2012/33/EU with regards to Operation within European Territory in relating to introduce 0.1% sulfur limit for marine fuel.

- California Code of Regulation, Title 13, Section 2299.2, “Fuel Sulfur and Other  Operation Requirement for Ocean-going Vessel within California Waters and 24 Nautical Miles of the California Baseline” of California Air Resources Board (CARB).

- 2013 VGP issued by U.S. EPA, the requirement compulsorily applicable to foreign flag vessel calling US ports, on Environmentally Acceptable Lubricants (EALs) for bow thruster, stern tube (except air seal applied), rudder bearing (expect water lubricated) and stern tube of lifeboat.

- Suez Canal Navigation Rules, including Tonnage Measurement.

- Panama Canal Navigation Rules and Regulation OP NOTICE TO SHIPPING No. N-1-2019 including Tonnage measurement, Panama chock, mooring arrangement, visibility, etc. (Except paragraph 2.f.(b)(f): the requirement of minimum required draught under ballast condition and Pilot ladder accommodation) It is Buyer’s obligation/risk to apply for an exemption of any rules from the ACP Authority which require minimum required draught and pilot accommodation ladder.

- Australian Maritime Safety Authority, Marine Orders, Part 32 “Cargo Handling Equipment” 2016, regarding Cargo gear, cargo Hold/Deck and access to container onboard for loading and unloading and for material handling equipment.

- IMO Resolution MSC 282 (86), “SOLAS V, Reg. 19, requirement of ECDIS”.

1.3.4	guidelines and recommendations

The Vessel to be built in compliance with following Guidelines and Recommendations.

- IEC Publication No.60092, Electrical installation in ships where required by the Classification Society.

- IMO ResolutionA.868 (20), “Guidelines for the control and Management of Ship’s Ballast Water to Minimize the Transfer of Harmful Aquatic Organisms and Pathogens.”

- International Convention for the Control and Management of Ship’s Ballast Water and Sediments, 2004 according to Regulation D-2 of Ballast water Treatment System. Ballast Exchange Standard (Regulation D-1)is not included.

- ISO 6954-2000(E), “Mechanical vibration – Guidelines for the measurement, reporting and evaluation of vibration with regard to habitability on passenger and merchant ships” (except bridge wings).

- IMO Resolution A.601(15), “Provision and display of maneuvering information on board ships”(Only Appendix 1,Appendix 2 and Appendix 3).

- IMO Resolution MSC.137(76), “Standards for ship maneuverability”.

- IMO MSC/Circ.1053, “Explanatory Notes to the Standards for Ship Maneuverability”.

- IMO Resolution MSC.1/Circ.1353, “Revised guidelines for the preparation of the cargo securing manual”.(To be prepared by the Buyer)

- IMO Resolution MEPC.227 (64), “2012 Guidelines on implantation of effluent standards and performance tests for sewage treatment plants” and amendments by Resolution MEPC.284(70).

- IMO Resolution MEPC.231(65),“Guidelines for calculation of reference lines for use with the energy efficiency design index (EEDI)”.

- IMO Resolution MEPC.245(66),“2014 Guidelines on the method of calculation of the attained energy efficiency design index (EEDI) for new ships” and amendments by Resolution MEPC.263(68) and Resolution MEPC.281(70).

- IMO Resolution MEPC.285(70),“2016 Guidelines for the Development of a Ship Energy Efficiency Management Plan (SEEMP)” and Amendments. (to be prepared by the Buyer)

- IMO Resolution MEPC.254(67),“2014 Guidelines on survey and certification of the energy efficiency design index (EEDI)” and Amendments.

- IMO ResolutionA.962(23), “IMO Guidelines on Ship Recycling Convention 2009”, amended by Resolution A.980(24) (Inventory of Hazardous Materials Only).

- IMO Res. MEPC.269 (68), “2015 Guidelines for the Development of the Inventory of Hazardous Materials”.

- Maritime and Port Authorities (MPA)of Singapore, Recommendation on night signal (3 all-round green lights) for vessels crossing the Traffic Separation Scheme (TSS) and precautionary areas in the Singapore Strait during the hours of darkness.

- MSC/Circ.1175 Shipboard equipment, fittings and supporting hull structures associated with towing and mooring for vessels keel laid after 1 July 2007

- ISPS Code – ref security locks of doors and hatches.

- MSC.1/Circ.1321 GUIDELINES FOR MEASURES TO PREVENT FIRES IN ENGINE ROOMS AND CARGO PUMP ROOMS"

- Resolution A.1021(26) Adopted on 2 December 2009 “CODE ON ALERTS AND INDICATORS, 2009

- Maritime Traffic Safety Law of Japan for foreign vessels (flash lamps without certification request).

- ISO/FDIS 14726 (2008)

1.3.5	CERTIFICATEs

The Builder shall obtain the following certificates and deliver to the Buyer at the time of the Vessel's delivery in triplicate, one (1) original and two (2) copies. All certificates issued by classification or authority party to be free of remarks except the exemption items.

| Certificate | Issued by |
| --- | --- |
| Classification Certificate | Classification Society |
| International Load Line Certificate | Classification Society |
| Cargo Ship Safety Radio Certificate | Classification Society (or Authorized party) |
| Cargo Ship Safety Construction Certificate | Classification Society (or Authorized party) |
| Cargo Ship Safety Equipment Certificate | Classification Society (or Authorized party) |
| International Tonnage Certificate | Classification Society (or Authorized party) |
| International Oil Pollution Prevention Certificate (IOPP) | Classification Society |
| International Sewage Pollution Prevention Certificate (ISPP) | Classification Society |
| International Air Pollution Protection Certificate (IAPP) (Tier II) | Classification Society |
| Statement of Compliance of International Prevention of Pollution by Garbage | Classification Society |
| Statement of compliance with the NOx technical code for main & aux. engine (EIAPP) (Tier II) | Classification Society |
| International Ballast Water Management Certificate | Classification Society |
| Certificate of Potable Water | R.G. |
| Panama Canal Tonnage Certificate | Classification Society |
| Suez Canal Tonnage Certificate | Classification Society(or Authorized party) |
| Documents of compliance regarding Suez Canal Authority Regulations (Rules of Navigation) | Classification Society (or Authorized party) |
| Builder's Certificate | Builder |
| Statement of Compliance of Asbestos free | Builder |
| Ship Sanitation Control Exemption Certificate | Chinese Government |
| Inclining Test Report and Lightweight Survey Report | Classification Society |
| Deadweight Certificate | Builder |
| Statement of Compliance MLC2006, Title 3.1 Regulation A 3.1 mandatory requirement only (except swimming pool) | Classification Society |
| Declaration of Compliance foreign vessel Operation in The Navigable Waters of The US | Classification Society |
| International Ballast Water Management Certificate | Classification Society |
| Statement for US VGP (EAL Only) | Classification Society |
| Statement of Compliance with AMSA (Australia) for cargo hold access | Classification Society (or Authorized party) |
| Document of Compliance with the special requirement for Ships Carriage of Dangerous Goods (SOLAS 74, as amended, Regulation II-2/19) | Classification Society |
| Statement of Compliance for Anti-Fouling Systems | Classification Society |
| Test Certificate of Windlass, Anchor, Anchor Chain, Mooring Rope | Classification Society |
| Certificate for Provision Crane, Engine Room Crane and lifeboat Davit | Classification Society |
| International Energy Efficiency Certification including EEDI Technical file | Classification Society |
| Statement of Compliance for Noise | Classification Society |
| Statement of Compliance on Inventory of Hazardous Materials | Classification Society |
| Fumigation statement of Wooden grating | Builder |

R.G.: Registry Government or Authorized party when applicable

The registration of the Vessel to flag state shall be of Buyer's responsibility and all expenses shall be borne by the Buyer.

The Buyer to supply following documents approved by Classification Society with the Builder’s assistance:

        - Shipboard Oil Pollution Emergency Plan (SOPEP)

        - Training Manual

        - Fire Safety Training Manual and Operational Booklet

        - Garbage Management Plan and Garbage Record Book

        - International Ship Security Certificate (ISSC) and Ship Security Plan (SSP) covering the requirement of ISPS Code except Automatic Identification System (AIS), ship security alert System (SSAS) and Ship’s Identification number (IMO number)

        - Minimum Safe Manning Document

        - Safety Management Certificate (SMC)

        - Container Securing Manual(CSM)

        - Emergency Towing Procedure

        - Ship Energy Efficiency Management Plan (SEEMP)

        - Fuel Oil Changeover Procedure and Logbook

        - Ship-specific Plans and Procedures Recovery Persons from the Water

        - Pilot Card

        - Maneuvering Booklet

        - Muster List

Other Certificates not listed in the Specifications, but required by the herewith listed Rules and Regulations as compulsory shall be provided by Builder.

Other miscellaneous certificates including manufacturer’s certificates which are normally issued for machinery, equipment and outfit of the Vessel to be supplied.

However, if formal certificate(s) cannot be obtained at the Vessel's delivery, the Builder shall furnish provisional certificate(s) to the Buyer to substitute the formal certificate(s).

In such case, the Builder shall deliver formal certificate(s) to the Buyer to replace the provisional certificate(s) to the Buyer as soon as practicable after the Vessel's delivery but in any case prior to the expiration of the validity of the provisional certificates.

1.4      PROPERTIES OF THE VESSEL

1.4.1	MODEL TEST

The model tests consist of the following items which shall be carried out at SSSRI (Shanghai Ship& Shipping Research Institute) and / or CSSRC (China Ship Scientific Research Center).

- Resistance test, at the sea trial ballast draught / EEDI draught / design draught / scantling draught (4draughts), six (6) speeds.

- Self-propulsion test with design propeller at sea trial ballast draught / EEDI draught / design draught / scantling draught (4draughts), six (6) speeds.

- Open water test of design propeller

- Stream line paint test for bilge keel at service speed on design draught

- 3-D Wake field measurement at service speed on design draught

- Designed propeller cavitation test with pressure fluctuation measurement at service speed on design draught and trial ballast draught. The rudder cavitation can be observed together, and the cavitation report will  be provided including the rudder cavitation description.

Model test reports shall be submitted to the Buyer for reference only.

EEDI verification shall be carried out according to IMO Resolution MEPC.254(67) and ISO 15016:2015(E).

The Builder will inform the Buyer to attend model test. The fee that Buyer attend model test are on Buyer’s account.

1.4.2	deadweight and lightweight

The deadweight shall be the difference between the lightweight and fully loaded displacement in sea water of specific gravity of 1.025.

“Fully loaded displacement” is the total displacement in metric tons at the draught specified in “1.2.3 Principal Dimensions” in sea water of specific gravity of 1.025 including shell and all appendages, under the condition of even keel, no heel, no deflection of the Vessel.

The following items shall belong to the lightweight:

- Hull, outfitting, machinery and electric weights, including the weight of fittings and equipment as stated in the Specification;

- Spare parts and inventories according to the requirement of Classification Society and other contractual Regulatory Bodies;

- Oil and water inside the machinery, heat exchangers, supplying/circulating piping and valves in engine room directly related to main engine based on the minimum quantity necessary for main engine starting.

- Fixed fire-fighting mediums (CO2, dry chemical powder, foam concentrate, fresh water, etc.) as per IMO MSC.1/Circ.1539

All weight including the following other than those mentioned above shall belong to the deadweight:

- Cargo and/or ballast water (including water in ballast pipe);

- All fuel oil(HFO/LSHFO/ULSFO/MGO), lubrication oil and fresh water including stern tube cooling water;

- Spare parts and inventories in excess of the requirement of the Classification Society and other contractual Regulatory Bodies;

- Equipment, spare parts and tools supplied by the Buyer in excess of the Rules and Regulations;

- Tools and test equipment;

- Crew, luggage and effects on board;

- Provisions;

- Stores of all kinds;

- All loose fittings for containers lashing and securing;

- Cargo lifting beams and grommets;

- All other outfitting items supplied by the Buyer, if any.

1.4.3	TRIM AND STABILITY

The Builder shall carry out trim and stability calculation based on the following criteria and loading conditions.

Criteria (Only for Seagoing conditions):

- 2008 IS Code Part A Chapter 2.2&2.3

- IMO SOLAS II-1, Part B-1 Subdivision and Damage Stability of Cargo Ships

- Trim by stern limited to 1.0%LOA(except ballast conditions)

Loading Conditions:

- Lightship condition.

- Docking condition with zero cargo and arrival bunker.

- Ballast condition with zero cargo, at departure and arrival.

- Ballast condition with cargo for Panama Canal transit, at departure and arrival.

- Homogeneous10, 12, 14, 16, 18 MT/TEU container (8’6’’ height) loading condition, at design draught, at departure and arrival.

- Homogeneous10, 12, 14, 16, 18MT/ TEU container (8’6’’ height) loading condition, at scantling draught, at departure and arrival.

Note:

1) The loadable number of containers shall be limited only by visibility requirements. The restrictions by capability of container lashing and securing system to be not taken into account.

2) Transverse metacentric height after free surface correction (G0M) shall not be less than the minimum GM value required by damage stability.

3) For each calculation, bunker and other consumable supplies shall be as follow

At departure:	100% consumables supplies

At arrival:	Approx. 10% supplies forHFO/LSFO/ULSFO/MGO/FW and approx. 70%~85% supplies for LO of the total amount at departure

4) The vertical center of gravity (VCG) for stability calculations shall be assumed 45% of the container height.

5) Following values shall be used for the trim and stability calculation:

Kind of liquid		Specific gravity	Full filling ratio (%)

| Ballast water: | 1.025 | 100 |
| --- | --- | --- |
| Fresh water: | 1.000 | 100 |
| Heavy fuel oil: | 1.01 | 98 |
| Low Sulphur Heavy fuel oil/ Ultra Low Sulphur fuel oil | 0. 990 | 98 |
| Marine Gas oil: | 0.890 | 98 |
| Lube. Oil: | 0.900 | 98 |

Ballast water may be loaded fully and/or partially to obtain suitable trim and stability within the permissible shear force and bending moments assigned by the Classification Society.

The load conditions in the Trim and Stability Booklet shall show the values for each

read-out frame, given by the SF&BM&TM limit approval letter. Shear force, bending moment and cargo torsional moment to be used with the same unit as used at the Class approval letter (kN for force and kN.m for moment to be used).

The Builder shall furnish the final Trim and Stability Booklet based on the lightship property obtained from inclining test or lightweight test.

If the final trim and stability booklet is under the approval stage by the Class or Authority at the delivery time of the Vessel, the preliminary one shall be onboard for provisional use.

1.4.4	VIBRATION AND NOISE LEVEL

1.4.4.1	Vibration Level

The International standard ISO6954-2000(E) “Mechanical vibration-Guidelines for the measurement, reporting and evaluation of vibration with regard to habitability on passenger and merchant ships” shall be used as criteria for the vibrating evaluation in the accommodation and working area (except bridge wings).

The design target for vibration level shall be below the upper limit of the "stippled zone" on the figure of the mentioned ISO standard as far as possible. The vibration levels in living quarters shall be below 6.0 mm/sec for area classification B (crew accommodation areas) and below 8.0 mm/sec for area classification C (working areas) as defined in ISO 6954-2000(E).

The vibration measurement shall be carried out during the sea trial in condition of smooth water with main engine running at the operating rpm specified in CSR condition (1.2.7) of main engine as per the Builder’s practice according to ISO 6954-2000(E).

The vibrations deriving from abnormal sailing conditions such as bad weather, maneuvering, running astern, etc. shall not be taken into account.

If vibration level exceeds the limit as defined by ISO 6954-2000(E), the Builder shall make necessary improvement to a practical extent mutually agreed between the Buyer and the Builder, but without second sea trial unless Flag requires.

1.4.4.2	Noise Level

Sound insulation and isolation treatment shall be provided as necessary to keep noise levels within the following values at normal sea going condition within the limit as defined by IMO Resolution MSC.337(91)–“Adoption of the code on noise levels on board ships” (only mandatory part) (Except bridge wing).

| Designation of rooms and spaces | dB(A) |
| --- | --- |
| Machinery space | 110 |
| Engine control room | 75 |
| Work shop out of machinery space | 85 |
| Wheelhouse including chart space | 65 |
| Private cabin | 55 |
| Mess room, recreation room and office | 60 |
| Hospital | 55 |
| Galley and pantry (without food processing equipment operating) | 75 |
| Look-out posts including bridge wings and windows | 70 |

The noise level measurement shall be carried out according to IMO Resolution MSC.337(91). During sea trial, the Vessel shall be running ahead steadily at the operating rpm specified at CSR of main engine, and not less than 80% of the specified maximum continuous rating (SMCR), under the following conditions, and the bedding and linen as well as personal goods shall not be removed during these measurements:

- Auxiliary engine(s) shall be operated in normal sea going condition without reefer containers.

- Machinery unnecessary for normal navigation shall be stopped.

- Mechanical ventilation and air-conditioning equipment shall be in normal operation.

- All doors and windows of the rooms shall be closed.

- No intermittent sound and noise shall be included (i.e. from air horn, sanitary flushing, laundry machine, emergency generator, telephone, speaker, foot-step noise in stair and corridor space, etc.)

- The measurement shall be taken at a height of 1.2 m to 1.6 m above deck, platform or walkway according to IMO Regulation.

If the measured noise  level  exceeds  the  specified  value as  required  by  Resolution MSC.337(91) by more than 3 dB(A), the Builder shall make necessary improvement to a practical  extent  mutually  agreed  between  the  Buyer  and  the Builder, but without second sea trial.

1.5	   INSPECTION, TESTING AND TRIAL

1.5.1	GENERAL

The inspection, testing and trial during construction of the Vessel shall be carried out by the Builder and/or the Maker at Maker’s shop or onboard as per the Builder’s and/or the Maker’s practice in compliance with the Rules requirements of the Classification Society under the presence of the Buyer's supervisor(s) and/or the Surveyor of the Classification Society.

The list of items for inspection and testing shall be submitted to the Buyer for approval in advance.

In section schedule shall be submitted to the Buyer's supervisor(s) one (1) working day in advance, if inspection takes place inside shipyard or near shipyard. And seven (7) working days in advance for the inspection outside shipyard in China, and fourteen (14) calendar days in advance for the inspection out of China. Builder will provide the weekly schedule in advance.

Failure of the Buyer's supervisor(s) present, without justifiable reason after due notice as above, shall be deemed as the waiver of the right, and in such case the Buyer shall be obliged to accept the result so such test on the basis of the Builder's and the Classification Society’s reports on the said results.

Any different opinion of inspection or test between the Buyer’s supervisor and the Builder’s supervisor shall refer to the opinion of Classification Society Surveyor as the settlement.

As to the details of testing of hull, machinery and electrical equipment and installations, refer to relevant Parts.

1.5.2	INSPECTION AND TEST OF CONSTRUCTION

Tests and inspection for hull structure shall be conducted in accordance with the Rules requirement of the Classification Society, IACS Rec.47 and the agreed CSQS and/or the Builder’s standard at time of Contract. List of item for inspection to be submitted to owner.

Records with all measurements during inspections, tests and trials (for all hull /machinery / outfitting /electrical fields) to be prepared by the builder and submitted to the Buyer as a single finished quality booklet.

The Builder/ Subcontractor is to ensure safe access for all inspections.

The block inspection to be carried out after completion of hull block steel works. Even if fittings are fitted wholly or partially to the hull blocks, the inspection for the hull blocks shall be carried out without dismantling such fittings unless the block inspection is obstructed by such fittings.

Castings of stern frame and rudder to be tested by magnaflux or equivalent according to rule's requirement.

Up to total 20 X-rays/UT in addition to class requirements will be taken upon request of Buyer's supervisors and will be marked by Buyer's supervisors on the NDT drawing. if any defect area(such as working hole/or repair area), to be done NDT inspection according to class requirement.

List of tanks that will not be hydrostatically tested shall be provided to buyer during plan approval.

Piping tests to be carried out in two steps: Checking of system’s construction and flushing or blowing. Tightness test as per Class and Maker’s requirements and recommendations.

1.5.3	SHOP TESTS

Shop tests for main engine, auxiliary machineries, boiler, deck machineries, provision crane, steering gear, boat davit, control console, etc. to be performed in accordance with the Rule’s requirement and/or the Maker’s standard. The items of shop test to be according to Class’s requirement and Maker's standard. Other machineries also same the M/E+G/E must inform to owner shop test day and test record.

The shop tests of main engine, main generator sets and main switchboard to be attended by the Buyer. The Buyer shall be informed of the shop test schedule at least fourteen (14) calendar days in advance out of China and seven (7) calendar days in advance in China in general of the expected date of the shop tests. The Buyer’s attendance shall be confirmed to the Builder at least seven (7) calendar days in advance out of China and two(2) working days in advance in China of the expected date of the shop test. The list of the Buyer plans to join shop test will be mutually agreed by both sides and submitted later. Other machineries test must inform to owner shop test day and test record.

The detailed descriptions of shop tests shall be also referred to respective Specifications.

The guarantee fuel consumption of the main engine to be certified at the Maker’s shop test.

1.5.4	MOORING TEST and lifting test of container

Items for mooring test to be submitted to the Buyer four (4) weeks ahead of the test. Procedure for mooring test of the Vessel shall be submitted to the Buyer for review.

The container stowage test for cell-guide in cargo holds and container sockets on deck to be carried out for each 20ftand 40ft socket.

Calibrated container to be used and test carried out in the presence of buyer representative. In case the container test for hatch cover is done at factory, the test to be repeated at random on board for not less than 30% of sockets. Others socket will be check by scanner or total station equipment.

1.5.5	INCLINING TEST AND lightWEIGHT MEASUREMENT

When the Vessel is substantially completed except minor items of work, an inclining test and deadweight measurement shall be carried out in order to determine the lightship weight and center of gravity of the Vessel as per the Builder’s practice. Inclining Test Procedure to be prepared by the Shipyard according to Class requirements and in line with IACS REC.031.

1.5.5.1	Inclining Test

The inclining test shall be carried out in the presence of the Buyer's Supervisor(s) and the Classification Society’s Surveyor in accordance with the procedure of inclining test and deadweight measurement.

The inclining test shall be conducted by shifting solid portable weights on upper deck or hatch cover and by measuring the Vessel's heeling angle using U-tubes or pendulums which placed forward and aft of the cargo hold region.

The results shall be submitted to the Classification Society for approval. The report shall be furnished to the Buyer.

For subsequent vessel(s) of this series, the inclining test except lightweight survey shall not be carried out and the results of the first Vessel shall be referred to subject to approval by the Classification Society.

1.5.5.2	Lightweight Survey

For subsequent vessel(s) of this series, the inclining test except lightweight survey shall not be carried out and the results of the first Vessel shall be referred to subject to approval by the Classification Society if the deviation of lightweight of sister vessel(s) with first Vessel is not more than 1%.

Lightweight survey shall be carried out by visual reading the draught of the Vessel, by measuring specific gravity of water and by subtracting the weight temporarily loaded items on board and adding the weight of unfurnished items at the time of lightweight survey in the presence of the Buyer’s Supervisor(s) and the Classification Society’s Surveyor.

Draught of the Vessel shall be read at the port and starboard side in way of fore, midship and aft draught marks.

The displacement of the Vessel shall be determined from the hydrostatic tables based on the measured draughts with due corrections for trim, heel, deflection of the Vessel and specific gravity of water at the measurement according to the Builder’s practice.

1.5.6	QUAY TRIAL and sea trial

1.5.6.1 Quay Trial

Quay trial for the main engine shall be carried out to check the starting and running of main engine with associated auxiliary machinery prior to the sea trials while the Vessel is moored at the Builder's quay according to Builder’s practice.

- Confirmation of starting position

- Turning gear interlock test

- Confirmation of control test

- Safety device test (automatic trip by simulation)

- Minimum revolution test

- Inspection of crankcase

- Test of starting air capacity

Following test shall be carried out during Quay trial or Official Sea trial.

- Anchoring test

- Lifeboat and davit operation test, launching test

- Refrigerating system test

- Air conditioning system test

- Ballast and bilge system test

- Ballast water treatment plant test

- Emergency fire pump test

- Black-out test

1.5.6.2 Sea Trial

When the vessel is substantially completed and ready for sea trial (excluding works which can be made after sea trial), sea trial to be performed by the Builder and mutually agreed program in the presence of the Owner’s representative and the Class surveyor.

Reports of official sea trial shall be submitted to the Buyer.

After sea trial, working parts of main engine and auxiliary machinery to be opened for the Buyer's inspection on Buyer’s request which cylinder to be opened in accordance with the standard of the Builder, approved by Buyer and refitted to working condition. A confirmation dock run after overhaul for 1/2 hours at low load to be carried out before delivery.

Marine diesel oil (MDO) and heavy fuel oil (HFO) of RMK380 available viscosity in

Chinese domestic commercial market shall be used for the sea trial at the Builder’s

decision. MDO (viscosity is 3~6 cSt/40oC (Sulfur content <0.1% m/m) will be used instead of MGO for MGO system function test.

The test condition of Scrubber in sea trial should be done as below:

1) The alkalinity of the sea water: according to the actual sea trail area.

2) The sulphur value of the fuel oil: according to the actual filling fuel oil (2.5~3.5%.).

3) The engine loading to be as following

For ME: 100%SMCR (to be carried out as per engine output power or engine revolution which reach first).

For AE: according to the actual consumption in the sea trial condition.

Duration: One(1) hour

Sea trial shall be carried out at the ballast condition corresponding to model test.

The measured trial speed shall be corrected to no wind and no wave, open, deep and current less sea conditions as per ISO 15016:2015(E) - "Guidelines for the assessment of speed and power performance by analysis of speed trial data".

The service speed at design draught under the conditions defined in Section 2-7.3 shall be obtained by means of extrapolation based on the above corrected speed and model test results.

During the sea trial following test and trial, as well as other items required by the Classification Society and/or the Regulation Bodies concerned shall be conducted.

| Item | M/E Load | 1st Vessel | Subsequent Vessel (s) | Remark |
| --- | --- | --- | --- | --- |
| Test condition | - | Ballast draught | Ballast draught | - |
| Progressive speed |  |  |  |  |
|  | 65% | O | O | *1 |
|  | 75% | O | O | *1 |
|  | CSR | O | O | *1*8 |
|  | SMCR | O | O | *1*8 |
| Endurance test (Duration: two (2) hours at CSR and four (4) hours at SMCR plus 30 minutes at 103.2% normal propeller speed where engine adjustment permits.) | CSR | O | O | *2 |
| Maneuvering test |  |  |  |  |
| - Turning circle test | *3 | O | X | *5 |
| - Zig-Zag test | *3 | O | X | 10/10 |
| - Crash stop astern test | *3 cond. To full astern | O | O | *4 |
| - Inertia test | CSR | O | X | *7, *9 |
| - Williamson turning test | CSR | O | X | *7 |
| Measurement of noise level (during endurance test) | CSR | O | O | *7 |
| Measurement of hull local vibration | CSR | O | X | *7 |
| Steering gear test | SMCR | O | O | *7 |
| Anchoring equipment test | - | O | O |  |
| Bow thruster test | - | O | O |  |
| Tests and calibration or adjustment of navigation equipment and instruments such as magnetic compass, auto pilot, echo sounder, speed log and radar, etc. | - | O | O |  |
| Lowest revolution test of main engine | Lowest RPM | O | O |  |
| Remote control test of main engine such as main engine maneuver, load up and down, slow down, shut down and emergency stop test, etc. | - | O | O |  |
| Fire alarm and general alarm test | - | O | O |  |
| Unattended operation test (Duration: four (4) hours) | CSR | O | O | *7 |
| Exhaust gas section of boiler demonstration | - | O | O |  |
| Fresh water evaporator capacity test | - | O | O |  |
| Torsional vibration measurement in shafting | *6 | O | X |  |
| Main engine overhaul inspection after sea trial *9 | - | O | O |  |
| Dead ship recovery test |  | O | O |  |
| Change over for M/E,G/Es and boiler from HFO-MDO (Statement to be certified by Classification Society) |  | O | O |  |

Notes:

“O” means “do”; “X” means “not do”

*1: Trial running on each load of main engine shall be of at least one (1) double run consisting of one (1) run in a direction on the course and one (1) run in the opposite direction.

Speed tests for final verification of attained EEDI to be during progressive speed tests.

The Vessel's speed shall be measured by means of Differential Global Positioning System (DGPS).

Weather condition of wind velocity and significant wave height shall be in accordance with requirement of ISO 15016:2015(E). The speed trial shall be carried out under the weather condition of maximum Beaufort Scale 5.

The measuring of main engine power of the Vessel shall be done by the Builder’s torsion meter.

*2: Fuel consumption of main engine shall be measured for ten (10) minutes at CSR during the endurance test for reference. The Brake Specified Fuel Consumption (B.S.F.C) of the main engine at shop test to be guaranteed. Endurance trial to be carried to be carried out as per engine output power instead of engine revolution except that Endurance trial at 90%SMCR and 100% SMCR to be carried out as per engine output power or engine revolution which reach first.

*3: The test speed for Maneuvering test is at the operating rpm specified at CSR of main engine and complying with the IMO MSC.137(76).

*4: To stop the Vessel (Until ahead speed becomes zero).

*5: To port and starboard with 35 deg. of rudder angle.

*6: Main engine RPM range for torsional vibration test to be decided according to manufacturer’s recommendation.

*7: Operating RPM at the specified power

*8: As per engine output power or engine revolution at the specified power which reach first.

*9: To stop the Vessel (Until ahead speed to become approx. 6 knots)

1.6	   DOCUMENTATION

1.6.1	PLANS FOR APPROVAL

The plans for Buyer's approval shall be in accordance with the “List of drawing for Buyer's approval” with approximate dates as agreed between Builder and Buyer and submitted to the Buyer for approval after the Building Contract of the Vessel.

Electronic copy (PDF format) of the plans for approval shall be submitted to the Buyer and the plans with the Buyer's approved signature shall be returned to the Builder. And these returned plans should be clearly marked with“approved” or “approved with comments” or “not approved”.

The Builder’s standard Plans and subcontractor’s or Maker’s Plans may be used as working plans or plans for approval.

The plans submitted for approval shall be deemed to have been approved without any comment when the Buyer does not dispatch the approved plan within fourteen(14)  calendar days after receipt the electronic copy plans.

After the mutual agreement of each drawing's/manual's comments, included in the relevant PCF form, a revised drawing called "Final Agreed Drawing", should be submitted to Owners which will include all the modifications agreed during the plan approval stage and all class comments, if any. This "Final Agreed Drawing", shall be the

one to be used during the vessel's construction stage.

During the phase of class plan approval, Builder will provide the Class approval drawing with class’ comments to the Buyer.

Final Approved drawings as clean version shall be submitted in one set of hard copy and

PDF electronic format to the Site office well in advance before onsite inspections. These

drawings to be in English.

For sister vessels, if any, the plans and documents of first vessel shall be used, so no duplication of drawing approval shall be necessary in general.

1.6.2	FINISHED DRAWINGS

Upon delivery of the Vessel, four (4) copies in black and white of finished plans shall be furnished to the Buyer in accordance with “List of finished plans”, one (1) copy to the Buyer’s office and two (2) copies to the Vessel.

The scope and form of the finished plans shall be in accordance with the Builder’s standard and the list of finished plans shall be submitted to the Buyer for approval in due course.

One (1) set of the final plans except Maker’s plans shall be furnished to the Buyer in electronic PDF format (CD-ROM or equivalent) in accordance with “List of finished plans” for the first vessel.

Instruction books shall be furnished for equipment and machinery, as supplied by manufacturer. Electronic manual of M/E and A/E to be provided by Maker.

The following plans, one (1) copy for each, mounted in aluminum alloy frame with glass shall be arranged at suitable places on board as directed by Buyer's supervisor. Except GA Plan and Fire Control and Safety Plan submit three(3) copy for Bridge, A-deck and ECR.

- General Arrangement Plan

- Tank Capacity Plan with Deadweight Scale

- Container Stowage Plan

- Fire Control and Safety Plan in color coding (Class approved)

- Damage Control Plan (1:200)

- Ballast and Bilge Pumping Plan

- Fuel Oil Bunker Procedure

- Fuel Oil Transfer System Plan

- Muster List (Buyer supply)

- Maneuvering Characteristics Diagram with time and distance (comply with IMO Res.A601)

1.6.3	EXHIBITION MODEL

One (1) exhibition model for series vessel in the scale of 1: 200 shall be supplied to the Buyer’s site office.

1.7	    DRY DOCKING

In the event that the period between the launching and the delivery date of the Vessel is more than six (6) months, an underwater survey shall be carried out. The diving company should be certified by Class. If the condition of the underwater survey is acceptable by painting maker, and maker provide 60 months guarantee, then no dry docking will be carried out. Otherwise, docking and hull cleaning at Builder's cost and time is to be carried out. The Paint maker must give a guarantee that vessel can remain 6 months in water prior delivery and 60 month Guarantee is applicable from date of delivery.

.

If the period from launching to delivery exceed more than 8 months, the vessel will be dry docking.

1.8      LOAD instrument

One (1) set of loading computer with software approved by the class with certificate to be supplied by buyer.

UPS to be supplied for loading computer. Loading computer with printer to be provided in ship’s office.

The calculation functions and display of the loading computer shall consist of:

- Displacement

- Trim/draught (fore and aft)

- Draft correction (fore and aft) due to difference of sea water density

- Shear force and bending moment at the prescribed frame points

- Maximum value of shear force and bending moment at the frame point

- Intact stability calculation and check of damage stability(by minimum GM/ maximum KG)

- Torsional strength

- Summary of loading condition

- Trim adjustment by ballast tank (program)

- Navigation bridge visibility

Lashing (if required):

- Container loading calculation by weight

- Lashing strength and racking force calculation

- Container control program

Data of tank level to be automatically transmitted from the ballast and bunkering control system.

CHAPTER 2

HULL

Version:A

Key words: Breadth 27.5m, Gearless, 6S60ME-C10.5 Tier II, 220ReeferPlugs, Open loop scrubber

CONTENT

CHAPTER 2 	HULL	3

2.1      HULL CONSTRUCTION	3

2.1.1	GENERAL	3

2.1.2	MAIN HULL	5

2.1.3	Deck House	10

2.1.4	MISCELLANEOUS	10

2.2     PAINTING CORROSION PROTECTION	13

2.2.1	GENERAL	13

2.2.2	SURFACE PREPARATION	14

2.2.3	PAINTING WORK	15

2.2.4	INSPECTION OF THE FILM THICKNESS	15

2.2.5	control	16

2.2.6	Painting Scheme	16

2.2.7	GALVANIZING	20

2.2.8	CATHODIC PROTECTION	21

CHAPTER 2 	HULL

2.1      HULL CONSTRUCTION

2.1.1	GENERAL

For the materials of hull steel of structures exposed to low air temperatures shall be based on the design temperature tD (the lowest mean daily average air temperature in the area of operation) of -10℃. (refer to IACS UR S6) (Rev 9 July 2018)

2.1.1.1	Scantling

The scantling of hull structure members shall be based on the scantling draught of the Vessel in compliance with the Rules requirements of the Classification Society.

Design stack load in cargo holds to be as follows.

20ft container:     30.5 MT / TEU (152.5 MT/Stack)

40ft container:     30.5 MT / FEU (152.5 MT/Stack)

the design stack load for container on upper deck shall be as follows:

20ft container:      90 MT/Stack

40ft container:     120 MT/Stack

The scantlings, which are not specified by the Classification Society, shall be in accordance with the Builder’s practice.

Finite element analysis and fatigue strength analysis shall be performed according to the Rules requirement of the Classification Society.

The fatigue life of hull structure to be in accordance with Class requirement for 25 years worldwide service.

Still water bending moment and shear force during cargo loading and otherwise must never exceed 92%.

2.1.1.2	Material

Hull structure members shall be of mild steel and high tensile steel approved by the Classification Society. The steel material including casting and forging shall be of qualities complying with the Rules requirements of the Classification Society.

High tensile steel with minimum yield stress of 315N/mm2 and 355N/mm2 shall be used where necessary.

Steel material not specified by the Classification Society shall be of ordinary commercial quality in accordance with Chinese Industrial Standard as per the Builder's practice.

All frames, beams and stiffeners to be of rolled section, bulb bar, flat bar or built-up section.

2.1.1.3	Workmanship

The works for hull construction shall be carried out in accordance with the Builder's practice and under the survey of the Classification Society.

Chinese Shipbuilding Quality Standard (CSQS) (GB/T34000-2016) and/or Builder's and Maker's Quality Standard shall be applied for workmanship of hull construction and outfitting.

Builder’s Standards and Practices will submit to owner for review at the beginning of  detail design stage.

2.1.1.4	Welding

Welding procedure including edge preparation shall be executed in accordance with the Builder's practice and approved by the Classification Society except unimportant or attachment weld which shall be carried out as per Builder's practice.

Welding shall be performed for the entire hull structure by manual, gravity, electro gas or any other automatic welding including CO2 semi-automatic shall be applied as per Builder's practice.

Fillet welding for the entire hull structure shall be double side continuous, except in some special area which the double side fillet welding can not be applied. In accommodation area and ventilation pipe, the intermittent or one side continuous fillet welding can be adopted.

Excessive distortion of hull structures due to welding shall be cured by means of cold press, line heating or spot heating. The IACS Rec.47 Shipbuilding and Repair Quality Standard is applicable as reference.

If needed plate thickness to be increased to meet strength requirement, the treatment shall be according to class requirement.

2.1.1.5	Slot, Scallop and Hole

Slots, scallops, air holes and drain holes shall be provided where necessary in accordance with the Builder's practice and shall be avoided in high stress areas.

In general, non-watertight structural members shall be provided with access and/or lightening holes as per the Rules requirements of the Classification Society and the Builder's practice, provided that such holes are not detrimental to the local strength of structural members.

Lightening holes on horizontal surface shall be provided with protections bars as per IACS REC 47.

2.1.1.6	Access Opening and Temporary Piece

Access opening for Builder’s workers during construction and for future convenience shall be provided for each cellular space in the double bottom and vertical wing spaces as shown on the Midship Section Plan.

Provisional access opening may be opened where required for access, communication and/or ventilation, etc. for the Builder’s workers during construction and shall be closed by welding after completion of the construction work.

Temporary pieces such as staging pieces, lifting lugs and their reinforcements, etc. fitted to the hull structure for construction may remain as per the Builder's practice unless they are inconvenient for the Vessel’s service and/or painting work. If temporary pieces should be removed, they will be cut off near the root and the edge to be properly finished.

Temporary access opening drawing will be provided to owner and the area where needs inspection to be discussed at site.

2.1.1.7	Strength and Leak Test for Hull Tanks

Structural strength test and leak test shall be carried out in compliance with the Rules requirements of the Classification Society and the Builder’s practice.

Leak test shall be carried out by pressurized air using soapy water at assembly or erection stages. However, fillet and erection joints of the tank boundaries may be tested locally by a compressed air or vacuum testing device.

Hydrostatic structural strength test shall be carried out for the tanks that over 20m3 except cofferdam and void space in accordance with the requirements of Classification Society, by filling with water during construction or afloat at Builder’s convenience to demonstrate the structural adequacy. However the hydrostatic test may be replaced by the air test subject to the agreement with the Classification Society and the Buyer for the subsequent Vessel(s).

Weather-tight and/or watertight doors, hatches, etc. shall be tested with water hose or pouring in accordance with requirements of the Classification Society.

Rudder shall be leak tested with air after completion of fabrication.

2.1.1.8	Welding Seam Test

Non-destructive test (NDT) shall be carried out in accordance with the requirements of the Classification Society.

The location and extension of the NDT shall be in compliance with the requirements of the Classification Society and NDT plan approved by Classification Society.

2.1.2	MAIN HULL

2.1.2.1	General

The structural details shall be in accordance with the Builder's agreed practice and shall be approved by the Classification Society.

Adequate drain holes and air holes shall be provided in the internal structural members in tanks to ensure free movement of contents to suctions and of air escape to vents and shall keep away from high stress areas.

The reinforcement for docking shall be carried out in accordance with the requirement of the Classification Society.

The decks under machinery and other heavy concentrated loads shall be reinforced according to the Rules requirements of the Classification Society.

Step and hand grips shall be provided for the access opening of which sill height is over 600 mm according to the requirement of the Classification Society and the Builder’s practice.

Lightening holes on the horizontal stringers shall be protected with steel bars to prevent personnel falling through according to the Builder’s practice.

Flat plate keel shall be provided throughout the Vessel’s length and connected to the stem and stern frame.

All FO tanks with capacity more than 30 m3 shall be protected by double hull structure.

Opening in the shell plating, deck girders, etc. to be rounded and reinforced by snipped end flat bar where ever necessary.

Hand grip for access manhole according to shipyard standard, yard will supply standard for owner approval.

2.1.2.2	Aft Body Construction

The stern shall be of open water type to obtain adequate clearance for the propeller and to support the rudder.

Steering gear room shall be arranged on the steering gear platform above the aft peak void.

Necessary eye plates for lifting rudder, shaft and propeller shall be provided.

The shafting shall be so designed that the propeller can be dismounted without unshipping the rudder, but the steering gear shall be able to turn suitable degree (with dismantle limit stopper/ram pin to release the rudder angle restriction) to enable propeller removal.

The stern frame shall be consist of steel plates, stern tube unit and weldable cast steel partly.

The stern tube unit shall be fixed to hull structure by stern frame bossing-piece by resin chock.

Aft peak bulkhead shall be of plane type with adequate stiffeners.

Stern tube cooling water tank shall be arranged below the aft peak void.

2.1.2.3	Engine Room Construction

Double bottom

The double bottom in the engine room shall be built on transverse framing system with solid floor in general.

Continuous longitudinal girders shall be provided in the double bottom so that the main engine including thrust block can be set directly on the double bottom. Main engine foundation shall be strengthened in accordance with the main engine Maker’s recommendation.

Double bottom knuckling shall be 50mm away from main double bottom floor.

L.O. sump tank below main engine shall be surrounded by cofferdam at sides and bottom to comply with main engine Maker’s recommendation.

Necessary tanks and bilge wells shall be arranged at suitable place in the double bottom.

Decks, side shell and engine casing, etc.

Upper deck and side shell shall be longitudinally framed and side shell shall be transversely or longitudinally in general.

Platform decks shall be arranged in engine room. These decks shall be extended from side to side with opening around main engine. Openings in the engine room decks shall be fitted with coamings of 100mm height.

Bulkheads and engine casing shall be constructed of steel plate with stiffeners and welded to the decks.

Primary structural member shall be arranged as required by the layout of machinery paying special care to minimize transmission of machinery vibration.

Tanks

Storage, settling and service tanks etc. for fuel oil and lubricating oil shall be arranged in the engine room as shown on the G.A. plan.

However, arrangement of small independent tanks such as service tanks for cylinder oil etc. to be shown on the Machinery Arrangement in E/R, which to be submitted during detail design stage.

Engine control room

Engine control room shall be provided with two windows of size 1.2m x 0.8m to get the direct view of Main Engine from ECR.

Funnel

The funnel shall be of mild steel construction with vertical stiffeners and horizontal stringers inside, and with top roof, adequate number of eye plate, scupper pipe, and steel door on it. The funnel top shall be sloped slightly aft to aid drainage to the scuppers aft.

Eye plates on the upper part of the funnel shall be provided in accordance with the Builder’s practice.

2.1.2.4	Cargo Hold Area Construction

The Vessel to have four (4) individual cargo holds divided by transverse watertight bulkheads.

Double bottom

Double bottom shall be arranged throughout cargo holds as shown on the G.A. plan.

Double bottom under the cargo holds shall be built with longitudinal framing supported by solid floor fitted at appropriate spacing. Longitudinal girders shall be arranged at appropriate spacing.

Doublers shall be welded on tank top, and local stiffening shall be provided at container corners under the tank top in accordance with the Builder’s practice.

Two (2) bilge wells shall be provided at the aft corner of every cargo hold, outside of container corner area and accessible.

Side construction

Side shell and longitudinal bulkhead to be longitudinally framed, in general.

The ship side shall be of double hull construction in way of the cargo area.

Watertight bulkhead

The watertight transverse bulkheads shall be arranged as shown on the G.A. plan.

The transverse bulkheads shall be of plane type construction with adequate stiffeners and primary members.

Structural bulkhead

The non-tight grillage shaped structural bulkheads shall be fitted in the middle of each cargo hold between 40ft container bays.

The structural bulkheads shall have sufficient strength to support the container load transmitted by cell guides fitted thereon.

Stringer decks to be provided as walkway for each 8'6" container tier in cargo holds. The structure to be strengthened to allow for flexi loading capability for the high cube containers in hold.

Stiffeners used on bulkhead between ballast tank and fuel oil tank shall be on fuel tank side.

Deck construction

The upper deck shall be connected to the shell plating by the square gunwale with 100mm extension above. Full/Deep penetration weld shall be used for Upper deck to Sheer strake connection (cargo hold).

Upper deck plating outside of hatch openings shall be longitudinally framed and stiffened with transverse webs in way of cargo hold region.

The corner edge of hatch opening in the upper deck shall be reinforced in compliance with the requirements of the Classification Society.

Hatch coaming

Cargo hatch shall be arranged as shown on G.A plan. Each cargo hatch shall have steel coamings of suitable height for stowing of containers.

The hatch coaming to be of steel construction with stays as required by the Classification Society. Hatch end bracket shall be curved with two radius type and not the straight bracket.

Side hatch coamings shall be continuous from forward to aft of cargo hold area and tapered at each ends according to the requirement of the Classification Society.

2.1.2.5	Fore Body Construction

General

The fore body consists of store space(s), fore peak tank and chain lockers as shown on the G.A. plan.

The shell plating shall be increased in thickness at the anchor rubbing areas as per the Builder’s practice.

Framing at forward end to have deep floors, deep web frames, side stringers and breast hooks as required by the Classification Society.

Side frames in the fore peak tank shall be arranged transversely or longitudinally.

The bulkhead of fore peak tank shall be of flat type stiffened by horizontal or vertical stiffeners supported by vertical webs or horizontal stringers fitted at suitable space.

The strength of the forecastle deck shall consider impacts from green water as per Class requirement.

Fore body construction to be designed against bow flare slamming according to the requirement of the Classification Society.

The forecastle to be arranged as shown on the G.A. plan.

Chain locker and Hawse pipe, etc.

Two (2) self-stowing circular or rectangular type chain lockers with sufficient capacity shall be arranged to stow the anchor chains well clear of deck over.

The lockers shall be of watertight steel construction and fitted with perforated false bottom grating with one removable steel cover at a level approx. 800 mm above the bottom of the locker to make a bilge well space underneath.

Chain pipe with bell mouth shall be led vertically from windlass according to the Builder’s practice. Each one (1) set of chain pipe cover shall be provided.

The chain cable shall be secured by quick releasing type cable clench on the side wall of the chain locker. The cable clench shall be provided for the chain locker in easily accessible position accordance with the Builder’s practice.

One (1) access manhole to be arranged on each chain locker wall in bosun store for inspecting locker and cable.

One (1) pair of hawse pipe with cast steel bell mouse shall be provided at the bow and shall allow acceptable housing of the anchor and easy lead of chain to the gypsy wheel.

For the details, refer to “Chapter 4–Outfitting Part” of the Specification.

2.1.3	Deck House

Deckhouse shall be of welded steel construction.

External walls of deckhouse shall be of flat plate type with vertical stiffeners inside, and the internal walls shall be of flat plate type.

The arrangement of internal webs, pillars and steel bulkheads to be considered to minimize vibration.

Decks shall be transversely and/or longitudinally stiffened with beams and girders.

Bridge wings shall be extended to full beam of the Vessel and be suitably supported with open type construction.

Wheelhouse front wall and side wall with windows shall be inclined outward from navigation bridge deck as shown on the G.A. plan.

As far as possible super structure exterior plates must be at least 6 mm.

2.1.4	MISCELLANEOUS

Foundation

Foundation of main engine shall be of welded steel construction in conjunction with bed plate, floors and girders of double bottom structure and shall have adequate strength and rigidity.

Foundation of main engine shall be designed according to the Builder's practice and the manufacturer's recommendation.

Foundation of auxiliary machinery, deck machinery, etc. shall be of welded steel construction and shall be reinforced with girders or carlings underneath as per manufacturer’s recommendation and Builder’s practice.

Main Engine foundation girders and face plates shall be welded with full penetration weld or deep penetration weld if class approval.

Bilge keel

Non-continuous type bilge keel shall be fitted on the bilge strake port and starboard with about 25% of the Vessel’s L.B.P. amidships.

Bilge keel to be built up with flat-bar and bulb plate. The bulb plate is welded to a flat-bar attached onto bilge strake by welding. The butt seams on the bilge keel flat-bars and shell plates to be staggered each other as per the Rule.

The flat-bar to be of continuous type.

Fore and aft ends of the bilge keels shall be gradually tapered and discontinued at block butt joint. Tapering of the bilge keel at both the ends shall be done as per IACS guidelines. The ends detail to be shown in the detailed BILGE KEEL DWG.

Sea chest

One (1) high sea chest and one (1) low sea chest shall be provided to side shell in way of engine room. One (1) separate sea chest shall be provided for emergency fire pump.

For details about drainage, air escape etc., refer to “Chapter 6–Machinery Part” of the Specification.

Nuts and bolts used for sea chest fittings shall be of stainless steel.

Bottom plug

One (1) bottom plug shall be supplied and fitted at bottom ballast water tank. Basically, the bottom plugs for oil tank and side ballast water tanks shall not be provided.

The size (M42), material (plug: SUS316, socket: mild steel) and design of the bottom plug shall be in accordance with the Builder’s practice. Spare parts of SS plugs to be provided according to yard practice.

Bottom plugs shall be fitted as close as possible to the lowest point of each tank and shall be kept clear of points where the keel block is laid beneath in dry dock.

Bottom plug sockets to be full penetration welded to Shell plate .Weld plug procedure will full weld and test for UT+PT or vacuum box(submit information to owner approval)

Two (2) spanner wrenches for bottom plugs shall be supplied for opening/closing of bottom plugs. .

Bulwark

The bulwarks of steel plate, suitably stiffened by stays at moderate space, shall be fitted on the forecastle deck and bridge wings as shown on the G.A. plan.

Knuckled wind-deflector shall be provided on the front bulwarks of the navigation bridge wings.

The bulwark plating shall be reinforced in way of openings for fairleaders, etc., if necessary.

Step and safety handrail shall be provided on the bulwark for the inspection of anchor stowage and on the operation position of mooring ropes.

Breakwater

Breakwater to be provided as shown on the G.A. plan. The height of breakwater shall be determined according to ILLC and Class rule’s recommendation.

2.2     PAINTING CORROSION PROTECTION

2.2.1	GENERAL

Painting works including surface preparation and inspection shall be carried out based on this Specification.

Painting works to be executed in accordance with the Builder's painting standard, process, practice and paint manufacturer's recommendation.

Stripe coats to be either sprayed or applied using brush/roller, will be confirmed at painting kick off meeting.

Following standards shall be performed:

- ISO 8501-1, Preparation of steel substrates before application of paints and related products - Visual assessment of surface cleanliness

Painting works in ballast water tanks shall be carried out in compliance with IMO PSPC requirement as per Resolution MSC.215(82).

Technical supervision shall be provided by the paint manufacturer and the Builder during preparation and application.

Paint shall be applied by airless spray in principle, but air spray, brush or roller may be applied where airless spraying is impracticable.

No paint work shall be applied to the following, unless otherwise specified.

-	Non-ferrous materials, i.e. copper, copper alloy (brass, bronze), aluminum, aluminum alloy and any other non-corrosive metals

-	Polished fittings

-	Galvanized fittings inside (Galvanized fitting on deck area needs to be carried out same as deck specification)

-	Gaskets and packing

-	Wood works in stores

-	Inner surfaces of doubling plates

-	Permanently enclosed space(Closed area needs to apply one coat of primer application 100μ as far as possible)

-	Internal steel surface under deck covering (Under deck covering to be applied with one coat of primer or not as per covering Maker’s requirement)

Galvanized pipe and vent outside to be painted one finish coat as surrounding and inside not painted.

Care to be taken to protect all cables during construction to avoid paint spillage on cables.

The colors of finish coats shall be in accordance with the Buyer’s/Charter’s color scheme and subject to the Buyer’s approval.

All machinery and equipment shall be coated in accordance with manufacturer’s standard and damaged part after installation shall be touched up only in accordance with Builder’s practice. The inside of radar mast and steel vent ducts to be painted with one (1) coat of Builder's standard primer only if not permanently closed.

Painting demarcation line for outside shell shall be as per the Builder’s standard.

Abbreviation:

A/F : Anti-fouling paint

Sa 2.5 : Abrasive blasting to Sa 2.5 grade (ISO 8501-1)

St 2 : Power tool cleaning to St 2 grade (ISO 8501-1)

St 3 : Power tool cleaning to St 3 grade (ISO 8501-1)

2.2.2	SURFACE PREPARATION

2.2.2.1	Primary Surface Preparation

All steel plates and sections equal to or greater than 6 mm in thickness shall be shot blasted

to ISO8501-1:1988 Sa 2.5 or sand blasting to SA 2.5 or pickling or ST3 may be applied instead of shot blasting when shot blasting is considered impractical.

2.2.2.2	Shop priming

Just after primary surface preparation one coat of 15 microns silicate type shop primer to be applied. The shop primer to have good compatibility with the succeeding coats. Different color shop primer to be used for mild and high tensile steel plates and sections.

2.2.2.3	Secondary Surface Preparation

Before the first coat is applied, damages by welding, burning, rubbing, etc. and rusted steel surface shall be prepared in accordance with the following table. Intact shop primer to be sweep blasting or brush disk cleaning.

| Area | In Block Stage | In Pre-erection, Dock & Quay Stage |
| --- | --- | --- |
| Bottom and boot top | Sa 2.5 | ST3 |
| Topsides | Sa 2.5 | ST3 |
| Weather deck and hatch coaming | Sa 2.5 | ST3 |
| Hatch cover | Sa 2.5 (outside) | ST3(outside) |
|  | Sa 2.0 (inside) | ST2(inside) |
| Deckhouse exterior | ST 3 | ST 2 |
| Deckhouse interior | ST 2 | ST 2 |
| E/R interior | ST 2 | ST 2 |
| Water ballast tanks | Sa 2.5 | ST3 |
| Fresh water tanks | Sa 2.5 | ST3 |
| Cargo holds | ST 2 | ST 2 |
| F.O &.,D.O.tanks | No treatment | ST 1 |
| L.O. tanks | No treatment | Wiping with system oil after cleaning |
| Others | SIS ST 2 | SIS ST 2 |

2.2.2.4	Surface Cleaning Before Over-Coating

The surfaces shall be cleaned of oil, moisture, dust, and other foreign materials with thinner, fresh water, wire brush, or compressed air prior to coating.

2.2.3	PAINTING WORK

The painting shall be carried out by airless spray, in general. Where the spraying is impracticable, the brush or roller shall be used.

The first full coat shall be spray, then roller and brush can be used. Roller and brush can be used in difficult and narrow area.

Mixing and thinning of paint material, conditions/methods of application and minimum and maximum over coating time of painting application shall be in accordance with paint manufacturer's recommendation and data sheets.

Free-edges of steel members which have been gas cut and welding slag remains shall be ground in accordance with the Builder's practice, WBT as per PSPC.

After the first coat one (1) stripe coat shall be applied on the edges of holes, scallops and frames in fresh water tanks and water/weather-exposed areas as well as cargo holds behind and between cell guides in accordance with the Builder’s practice. The stripe coat colour same with the full coat colour.

Watertight weld lines by manual welding shall be painted after air test has been finished, but adhesive tape and touch up of shop-primer may be performed during painting application.

2.2.4	INSPECTION OF THE FILM THICKNESS

Paint work will not start unless a block has been accepted or a written waiver is given by the Buyer’s representative.

For the paint, date of manufacture, batch number, shelf life and expiry dates to be provide as per paint maker’s standard.

Paint samples will be taken by Buyer’s site team and sent to paint Maker factory for analysis to confirm Maker’s formula is used.

The checking of final paint dry film thickness shall be carried out by the Builder in the presence of Buyer's representative and paint manufacturer.

Measuring

The cleaning condition and dry film thickness between coats shall be inspected by two (2) parties, the Builder and the paint maker. Builder will provide daily schedule to Buyer’s site office in the morning time. The Buyer will carry out the patrolling inspection.

In general, two (2) separate spot measurements shall be made over every 10 m2 in tanks, and for each 20 m2 in other spaces. The average of five (5) separate spot measurements must be within the specified thickness. For PSPC areas and fresh water and technical water tanks areas, rule 90%-90% shall apply as acceptance criteria for DFT measurement. Total

DFT specified in painting schedule shall be attained in at least 85% of the painted area and balance 15% measured should be not less than 85% of specified DFT.

In any measurement there is a certain level of uncertainty. Two inspectors using the same gage will not necessarily record the exact same number for given spot measurement. To allow for this natural fluctuation, an individual spot measurement is permitted to be below the 85% of specified minimum thickness, however The average of the means(3 points in the same 4cm diameter circle) shall not be less than the specified 85% of minimum thickness.

No measurement shall be made for about 100mm breadth from free edges of painted steel members, machinery, equipment, outfitting, pipes with nominal diameter of 250 mm and below, pipe supports, machinery seats, small fittings, etc.

Dry Film thickness always to be measured at every stage of coating, final confirm will be done after anti-corrsion paint finish.

2.2.5	control

2.2.5.1	Painting Work

Mixing and thinning of paint material, conditions/methods of application and time interval between each coat shall be observed to the paint manufacturer's recommendation.

Paint works shall not be performed in the open air during the periods of rain, snow, fog or mist and also shall not be carried out when the weather conditions may cause condensation i.e., relative humidity above 85% and steel temperature is lower than the dew point by three (3) degrees in centigrade except the case that paint maker has confirmed that a particular paint can be applied at such weather condition.

During coating processes, when dispute happen, the final arbitration shall come from the painting maker.

Any condition deviation from Paint’s MDS for application in relation to RH and Steel temperature must be given by Maker’s Office in writing and stamped. E-Mail correspondence is not acceptable.

2.2.5.2	Dry Film Thickness

In general, two (2) spot measurements for each 10 m2 in tanks and holds, and for each 20m2 in other spaces shall be measured.

2.2.6	Painting Scheme

Painting scheme for the parts and spaces, which are not specified herein, shall be similar to surrounding space or comparable spaces.

Epoxy finishes will chalk and exposed deck to have a Polyurethane finish coat.

Final dry film thickness and painting system shall be as listed below in the specification, changeable according to paint manufacturer’s recommendation subject to the Buyer’s approval.

| Code | Painting Area |  |  | Paint System |  |  | No. of Coat |  |  |  | DFT (micron) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flat bottom (up to bilge keel level) |  |  |  | ME (HB) A/C A/F (tin free) |  |  | According to maker ‘s recommendation 5 year life design |  |  |  | According to maker ‘s recommendation 5 year life design |  |
| Side bottom (from bilge keel to scantling draught) |  |  |  | ME (HB) A/C A/F (tin free) |  |  | According to maker ‘s recommendation 5 year life design |  |  |  | According to maker ‘s recommendation 5 year life design |  |
| Shellplate Scrubber area |  |  |  | Phenolic epoxy |  |  | 2 |  |  |  | 400 Antifouling same as surrounding |  |
| Topside (from scantling draught to upper deck) |  |  |  | ME (HB) A/C PU F/C |  |  | 2 1 |  |  |  | 200 50 250 |  |
| Exposed area and hull outfitting |  |  |  |  |  |  |  |  |  |  |  |  |
| 1) | Weather deck |  |  |  |  |  |  |  |  |  |  |  |
|  | Hatch cover outside only |  |  |  |  | Epoxy A/C PU F/C |  |  | 1 1 |  |  | 100 50 250 |
|  | Other weather deck |  |  |  |  | Epoxy A/C PU F/C |  |  | 2 1 |  |  | 200 50 250 |
| 2) | Deck house |  |  |  |  |  |  |  |  |  |  |  |
|  | Outside wall |  |  |  |  | Epoxy A/C Polyurethane F/C |  |  | 2 1 |  |  | 200 50 250 |
|  | Exposed decks |  |  |  |  | Epoxy A/C PU F/C |  |  | 2 1 |  |  | 200 50 250 |
| 3) | Funnel |  |  |  |  |  |  |  |  |  |  |  |
|  | Outside |  |  |  |  | Epoxy A/C Polyurethane F/C |  |  | 2 1 |  |  | 200 50 250 |
|  | Inside |  |  |  |  | epoxy primer/finish Heat resistance paint |  |  | 1 2 |  |  | 75 25 100 |
| 4) | Handrails, stanchion and outside of steel pipe |  |  |  |  |  |  |  |  |  |  |  |
|  | Galvanized |  |  |  |  | Epoxy A/C Same as surrounding |  |  | 1 1 |  |  | 40 50 90 |
|  | Bare steel |  |  |  |  | Epoxy A/C Same as surrounding |  |  | 1 1 |  |  | 150 50 200 |
| 5) | Vent duct |  |  |  |  |  |  |  |  |  |  |  |
|  | Outside |  |  |  |  | Same as surrounding |  |  |  |  |  |  |
|  | Inside |  |  |  |  | Epoxy primer |  |  | 1 |  |  | 100 |
| Cargo areas |  |  |  |  |  |  |  |  |  |  |  |  |
| 1) | Cargo holds |  |  |  |  |  |  |  |  |  |  |  |
|  | Wall and overhead, hatch cover underside and cell guide |  |  |  |  | Epoxy A/C Epoxy F/C |  |  | 1 1 |  |  | 100 100 200 |
|  | Hold Bottom |  |  |  |  | Epoxy A/C (HB) Epoxy F/C |  |  | 1 1 |  |  | 100 100 200 |
|  | Bilge well |  |  |  |  | Epoxy A/C |  |  | 2 |  |  | 300 |
| 2) | Hatch coaming |  |  |  |  |  |  |  |  |  |  |  |
|  | Outside |  |  |  |  | Epoxy A/C PU F/C |  |  | 2 1 |  |  | 200 50 250 |
|  | Inside |  |  |  |  | Epoxy A/C Epoxy F/C |  |  | 1 1 |  |  | 100 100 200 |
| 3) | Transverse box inside in way of bulkhead |  |  |  |  | Epoxy A/C |  |  | 1 |  |  | 150 |
| Accommodation areas |  |  |  |  |  |  |  |  |  |  |  |  |
| 1) | Accommodation and store |  |  |  |  |  |  |  |  |  |  |  |
|  | Wall and overhead | Bare steel |  |  |  | Epoxy primer/finish |  |  | 1 |  |  | 120 |
|  |  | Behind lining/insulation |  |  |  | Epoxy primer/finish 1 80 |  |  |  |  |  |  |
|  | Floor head | Bare steel |  |  |  | Epoxy primer/finish |  |  | 1 1 |  |  | 80 40 120 |
|  |  | Under covering |  |  |  | No coating (Shop-priming) |  |  |  |  |  |  |
| 2) | Refrigerated provision chamber converter room |  |  |  |  | Epoxy primer/finish |  |  | 1 |  |  | 125 |
| Engine room and steering gear room |  |  |  |  |  |  |  |  |  |  |  |  |
| 1) | Wall and overhead |  |  |  |  | Epoxy primer/finish |  |  | 1 |  |  | 120 |
| 2) | Deck and flat |  |  |  |  | Epoxy primer/finish |  |  | 1 1 |  |  | 80 40 120 |
| 3) | Tank top (below lowest gratings) |  |  |  |  | Epoxy primer Epoxy F/C |  |  | 1 1 |  |  | 100 100 200 |
| 4) | Behind lining |  |  |  |  | Epoxy primer/finish |  |  | 1 |  |  | 80 |
| Tanks |  |  |  |  |  |  |  |  |  |  |  |  |
| 1) | Water ballast tanks |  |  |  |  | as per PSPC Standard with approved coating system |  |  |  |  |  | 320 |
| 2) | Fresh water/drinking water tanks |  |  |  |  | Holding primer 40 at block stage Pure epoxy (Solvent Free) |  |  | 1 |  |  | 300 |
| 3) | Fuel oil tanks |  |  |  |  | Dry and clean with inhibitive oil |  |  |  |  |  |  |
| 4) | Engine cooling water expansion tanks and boiler cascade tank |  |  |  |  | Pure epoxy Pure epoxy |  |  | 1 1 |  |  | 125 125 250 |
| 5) | Lub. Oil tanks |  |  |  |  | Wiping :system oil after cleaning |  |  |  |  |  |  |
| 6) | Bilge tanks in engine room |  |  |  |  | Epoxy |  |  | 2 |  |  | 250 |
| 7) | Related Scrubber tanks |  |  |  |  | Pure epoxy Pure epoxy |  |  | 1 1 |  |  | 150 150 300 |
| Machinery and equipment |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Deck machinery |  |  |  |  | According to Manufacturer’s standard) |  |  |  |  |  |  |
|  | - Engine room machinery |  |  |  |  | According to Manufacturer’s standard |  |  |  |  |  |  |
|  | - Electric equipment |  |  |  |  | According to Manufacturer’s standard |  |  |  |  |  |  |
| Miscellaneous |  |  |  |  |  |  |  |  |  |  |  |  |
| 1) | Void space and cofferdam |  |  |  |  | Epoxy |  |  | 1 |  |  | 150 |
| 2) | Pipe duct inside |  |  |  |  | Epoxy F/C (HB) |  |  | 2 |  |  | 200 |
| 3) | Chain locker |  |  |  |  | Epoxy |  |  | 2 |  |  | 250 |
| 4) | Rudder |  | Outside |  |  | Same as Bottom |  |  |  |  |  |  |
|  |  |  | Inside |  |  | Volatile corrosion inhibitors |  |  |  |  |  |  |
| 5) | Rudder trunk |  |  |  |  | Same as Bottom |  |  |  |  |  |  |
| 6) | Sea chest inside |  |  |  | Same as Bottom |  |  |  |  |  |  |  |
| 7) | Draught mark and freeboard mark |  |  |  | Same as surroundings with or PU. marking paint |  |  |  |  |  |  |  |
| 8) | Vessel’s identification |  |  |  | P.U C.R. F/C or epoxy F/C |  |  |  |  |  |  |  |
| 9) | Void below upper deck |  |  |  | Epoxy A/C (HB) |  |  | 1 |  | 150 |  |  |
| 10) | Other store and workshop |  |  |  | Epoxy primer/finish |  |  | 1 |  | 120 |  |  |
| 11) | Entry guide piece of cell guide head |  |  |  | Finished with fluorescent type paint |  |  |  |  |  |  |  |
| 12) | Anchor and anchor chain |  |  |  | AS per manufacturer’s standard |  |  |  |  |  |  |  |

Notes:

- Dry film thickness of tin free type antifouling paint shall be in accordance with manufacturers’ standard for 60 months life time.

- Non-slip paint shall be applied for upper deck passageway and mooring surrounding, forecastle deck, and for the deck in steering gear room, other anti-slip method may instead.

- Painting scheme to be in accordance with the following specification or equivalent products, from the makers in Maker’s List. The positions not mentioned hereinafter should be painted same as the surrounding area or similar positions. Number of coats and total DFT (dry film thickness) might be adjusted according to paint manufacturer’s recommendation.

- Welding defect such as  blow hole or pore  and  undercut where not affect the structure strengthen will be applied the epoxy filler which recommended by paint maker.

- The paint on the shell outside will be finished before launch, if the period between the launch and delivery is not more than 8 months and if paint manufacturer will give anti fouling system a guarantee for five(5) years, no final dry docking to be carried out for the vessel.

- In any measurement there is a certain level of uncertainty. Two inspectors using the same gage will not necessarily record the exact same number for given spot measurement. To allow for this natural fluctuation, an individual spot measurement is permitted to be below the 85% of specified minimum thickness, however The average of the means(3 points in the same 4cm diameter circle) shall not be less than the specified 85% of minimum thickness.

- Dry film thickness shall be measured after each coat for quality control purpose and the total dry film thickness shall be confirmed after completion of final coat. The quality control shall be carried out by shipyard and paint maker.

- The E/R, accommodation, free board, cargo hold，main deck etc. area only apply one coat,  No DFT measurement for E/R, ACC C/H etc., only one primer as holding primer at block stage. Final DTF to be measured and confirmed at erection/dock/quay stage, Void, WBT , shellplate etc., which will be finished final coat at block stage need to be DFT measurement. Buyers should be informed the final coat before equipment installed on board.

- Time schedule for application of each coat to be decided according to builder's construction schedule, all inspections without disturbing builder's construction schedule. Painting inspection schedule to be submitted to Buyer at the same day morning.

- Some outfitting, some small area painting job which will affect other department schedule will be carried out de-rusting and painting by shipyard self. Shipyard to carry out the final coat for cable tray/cable supports and control panel supports before installation of the cable/panels.

- The intermediate coat and stripe coat to be inspected by Builder QC and Paint Manufacturer's representative , the inspection of final coat/surface preparation are to be done by Builder QC and Paint Manufacturer's representative and owners supervisor .

- The spray dust of adjacent painting may be left as it is where it is considered harmless to the performance or to the appearance.

- When owner chose the Scrubber maker, the discharge PH in tanks should be >5.

- Minor paint contamination need not to be cleaned.

- The fore mast inside to be painted as far as possible (no blasting due to grit can clean inside).

- If maker require, Report providing data of surface roughness will submit as per Anti Fouling maker’s standard after application of anti-fouling paint prior launching. Shipyard will assistant maker if maker required.

- PSPC Tri-Partite Agreement to be developed by the Builder and submitted for the Buyer’s review and acceptance prior to final signing by the Builder, Buyer and Paint Maker.

- Coating Technical File (CTF) will be developed by the yard and sent to the Buyers for review.

- The painting of the exposed structural steel surface such as outsides of shell, deckhouse, funnel, etc., and of living space and passages in living quarters to be free from obvious sags and runs.

- The steel structure surfaces including all welded seams to be painted before the leak test, in general. However, fillet welding joint and erection butt joint forming tank boundary to be painted after joint has been examined either by compressed air test, vacuum test or tank air test as appropriate.

- Pre-erection joint of hull block which is erected at any location, to be painted before leak test and not to be considered as block erection joint.

- Under water hull paint thickness to be checked before and after application of antifouling paint and recorded at dock stage.

- For the F.W.TK ventilation and washing, the Builder will accord paint maker's recommendation. The chemical component analyzes of water sample in these tanks to be carried out by authorized office and the report should be submitted to owner together with the delivery certificates.

- Main sea water filter body internals will be coated with anti-fouling paint.

- Exhaust pipes outside will be coated with temperature resistant paint before placing the insulation, the area under insulation no need to paint.

- PSPC blocks will be treated as PSPC requirements.

- All the bilge well in passageway/cargo hold/Chain locker to be applied 150*2.

- Maker of Anti Fouling to submit a report providing data of surface roughness after application of anti fouling paint if maker has this requirement.

- TSL: outfitting / cable/supports and some small area Owner will be informed the plan before install the equipment.

- Minor/spots paint need not to be removed if under insulation/lining.

2.2.7	GALVANIZING

Galvanizing shall be applied for items as described throughout this Specification and following outfitting shall also be galvanized:

- Handles and clips of steel doors

- Small chains (8mm dia. and below)

- Small shackles and thimbles (8mm dia. and below)

- Hinges, small bolts and nuts under 12mm dia. exposed to weather

- Standing rigging for masts

- Ventilator ducts (thickness below 2 mm) (refer to galvanized thin plate spire ducts, mainly used for accommodation etc.)

- Steps of inclining and vertical ladders on weather deck

- Chain pipe and hawse pipe covers

- outfitting in WBT to be galvanized.

Outfitting may apply one universal epoxy coat according to Builder’s practice.

The galvanizing shall be done by hot dip method in general except bolts, nuts, screws, etc. which are to be electroplated, unless otherwise specified in this Specification. Damaged area of galvanizing part caused by the electric welding, mechanical damage, etc. shall be touched-up by one (1) coat of Zinc Rich or epoxy primer as far as practicable.

In general, the steel outfittings to be painted with one (1) coat of the Builder’s standard epoxy primer.

    - CATHODIC PROTECTION

- The underwater hull and appendages shall be protected by an automatically controlled Impressed Current Cathodic Protection (ICCP) system.

Mean current density:       35 mA/m2for underwater hull

600 mA/m2for propeller

120 mA/m2for rudder

- Aluminum alloy anodes shall be fitted inside sea chests and bow thruster tunnel.

Mean current density:      35 mA/m2

Life time:                five (5) years

Sea chest filter shall be protected by MGPS aluminum and copper anodes.

The size, quantity and fitting method of ICCP system and anodes shall be as per the Builder’s practice and/or Maker’s recommendation.

CHAPTER 3

ACCOMMODATION

Version:A

Key words: Breadth 27.5m, Gearless, 6S60ME-C10.5 Tier II, 220ReeferPlugs, Open loop scrubber

CONTENT

CHAPTER 3	ACCOMMODATION	3

3.1	  COMPLEMENTAND CLASSIFICATION OF ACCOMMODATION	3

3.1.1	COMPLEMENT	3

3.1.2	CLASSIFICATION OF ACCOMMODATION	3

3.1.3	NAME PLATE & IDENTIFICATION	5

3.2     JOINER WORK	6

3.2.1	DIVISIONAL BULKHEAD, LINING AND CEILING	6

3.2.2	FIRE INSULATION, HEAT INSULATION AND SOUND ISOLATION	8

3.2.3	DECK COVERING	9

3.3	WINDOW, DOOR, STAIRWAY, ETC.	10

3.3.1	WINDOW	10

.3.3.2	DOOR	11

3.3.3	STAIRWAY AND STORM RAIL IN ACCOMMODATION QUARTER	15

3.4     FURNITURE AND UPHOLSTERY	16

3.4.1	FURNITURE	16

3.4.2	UPHOLSTERY	20

3.5	COMMISSARY AND SANITARY OUTFIT	21

3.5.1	COMMISSARY EQUIPMENT	21

3.5.2	SANITARY OUTFIT	22

3.5.3	DRINK WATER FOUNTAIN	23

3.6	  STORE	24

3.6.1	REFRIGERATED PROVISION STORE	24

3.6.2	DRY PROVISION STORE	26

3.6.3	MISCELLANEOUS STORES	26

3.7  ELEVATOR	26

CHAPTER 3	ACCOMMODATION

Cabin arrangement shall be as shown on G.A. plan.

The clear height of accommodation spaces between the deck surface covering and the underside of deck head beam or ceiling shall be about 2,100 mm in general, except that to be about 2,030 mm in sanitary, catering spaces, refrigerated provision stores, 2300mm for Gymnasium if possible. However, the protruding parts shall be excluded from the measurement.

In wheelhouse, the clear height between the bridge deck surface covering and the underside of the deck head beam or ceiling shall be at least 2,200 mm, and the lower edge of deck head mounted equipment shall be about 2,100 mm.

3.1	  COMPLEMENTAND CLASSIFICATION OF ACCOMMODATION

3.1.1	COMPLEMENT

| Dep. Name Class | Deck Dep. | Engine Dep. | Total |
| --- | --- | --- | --- |
| Captain Class | 1-Captain | 1-Chief engineer | 2 |
| Senior officer | 1-Chief officer | 1-Second engineer | 2 |
| Junior officer | 1-Second officer 1-Third officer | 1-Third engineer 1-Electrician 1-Fourth engineer | 5 |
| Petty officer | 1-Boatswain | 1-Chief Motorman | 2 |
| Rating | 5-Seaman 1-Cook | 5-Motorman | 11 |
| Others | 1-Pilot (Junior officer class) |  | 1 |
|  | 1-Owner (Junior officer class) |  | 1 |
| Total |  |  | 24 |
| Suez crew (not included in the complement ) |  |  | 6 |

3.1.2	CLASSIFICATION OF ACCOMMODATION

3.1.2.1	Living Space

        - Cabin

| Name of Class | Room | Lavatory |
| --- | --- | --- |
| Captain class | Day room, bed room | Private lavatory with shower |
| Senior officer | Day room, bed room | Private lavatory with shower |
| Junior officer | Single- berthed | Private lavatory with shower |
| Petty officer | Single- berthed | Private lavatory with shower |
| Rating | Single- berthed | Private lavatory with shower |
| Pilot | Single- berthed | Private lavatory with shower |
| Owner | Single- berthed | Private lavatory with shower |
| Hospital | Single- berthed | Private lavatory with bath tub |
| Suez crew | Six(6) steel folding beds | Stevedore’s toilet |

        - Other compartments

| Public space |  | Officer’s mess room Crew’s mess room Recreation room, Gymnasium Meeting room Dispensary and Medicine locker |  |
| --- | --- | --- | --- |
| Office space | Tally office Ship office and ballast control station |
| Sanitary space | Private lavatory Public toilet Stevedore’s toilet Officer’s laundry with drying room Crew’s laundry with drying room changing room |
| Catering space | Galley pantry |
| Corridor space | Corridor Stairway |
| Control space | Wheelhouse (incl. radio space and chart space) Engine control room Fire control station |
| Machinery space | Air conditioning unit room Emergency generator room |
| Working space | Deck workshop Electrical workshop E/R workshop |
| Provision space | Dry provision store Refrigerated provision stores |
| Sundry space | CO2 room Paint store Deck store Bosun store Safety equipment store Rope store Bonded locker Oxygen/Acetylene bottle store SOPEP store Electrical equipment store Battery store Garbage store Bonded store Linen store and other lockers |

Recreation room will use as smoking area with an exhaust equal to about 12 air changes an hour.

      - Anti-Pirate Citadel

A citadel to be provided in steering gear room withpermanently installed ventilation, potable water, steel door with anti-piracy deviceand VSAT cable.

Portable toilet, portable battery lantern, VSAT, medical first aid kit, emergency food and blanket antenna of the Iridium radio which to be supplied by Buyer.

3.1.3	NAME PLATE & IDENTIFICATION

Name plate & identification shall be provided as follows:

| Material | Location | Letter |
| --- | --- | --- |
| Stainless plate | Name plate of room, guide plate (in living quarter) | English |
| Stainless plate | Name plate of room (exposed to weather) | English |
| Plastic plate | In stairway of every deck showing deck name. Various safety marks. | English |

3.2     JOINER WORK

3.2.1	DIVISIONAL BULKHEAD, LINING AND CEILING

The arrangement of all walls, bulkheads, lining and ceilings in living quarters shall be in accordance with Method “IC” as specified in SOLAS.

Corridor bulkheads, except steel wall shall be constructed of “B” class fire retarding panel.

Internal division bulkheads between cabins shall be “C” class panel. Non-combustible ceiling shall be provided in all cabins, public space and wheelhouse and “B” class non- combustible ceiling shall be provided in corridor of living space.

Partition wall, lining and ceiling shall be according to Rule’s requirement.

Watertight coaming with one side continuous welding to be applied for water-tightness in way of wet space and exterior steel boundary bulkhead.

Wall panels to be jointed and installed shall be in accordance with the Maker’s standard. Other detail of securing and construction shall in accordance with the Builder’s practice, unless otherwise required by the Rule and Regulation.

The Builder shall provide color samples for the Buyer’s selection of panel finishing.

3.2.1.1 Partition Bulkhead

The material and thickness of partition bulkhead to be as follows.

| Corridor bulkhead (“B” class) and division bulkhead | Rock wool insulation of 50 mm thickness furnished with galvanized steel sheet of 0.6 mm thickness decorated with PVC film on both sides. |
| --- | --- |
| Partition bulkhead between two adjacent cabins (without neighboring bed), cabin and public space | Rock wool insulation of 50mm thickness furnished with galvanized steel sheet of 0.6 mm thickness decorated with PVC film on both sides. |
| Partition bulkhead between two adjacent cabins with neighboring bed. | Double 25mm thick rock wool board with 30mm clearance, (the total thickness to be 80mm), Rock wool insulation of 25mm thickness furnished with galvanized steel sheet of 0.6 mm thickness decorated with PVC film on visible sides. |

3.2.1.2 Lining and Ceiling

The lining and ceiling to be applied as follows.

| Compartment | Lining | Ceiling |
| --- | --- | --- |
| Living space Public space Office space Control space (only for Wheelhouse and ECR) Corridor space Private toilet Dispensary bath room | Rock wool insulation of 25mm thickness furnished with galvanized steel sheet of 0.6 mm thickness decorated with PVC film on visible side. | Rock wool insulation of 25mm thickness furnished with galvanized steel sheet of 0.6 mm thickness decorated with PVC film on visible side. |
| Catering space | Rock wool insulation of 25 mm thickness furnished with SUS304 sheet of 0.5mmthickness on visible side. | Rock wool insulation of 25 mm thickness furnished with SUS304 sheet of 0.5mmthickness on visible side. |
| Stevedore’s toilet, Public toilet, Changing room, Laundry and drying room | Paint finish in case of steel fabrication | Paint finish in case of steel fabrication |
| Dry provision store Machinery space Fire control station Sundry space (except lockers and store-rooms having area less than 4m2) | Paint finish in case of steel fabrication | Paint finish in case of steel fabrication |

3.2.2	FIRE INSULATION, HEAT INSULATION AND SOUND ISOLATION

Fire insulation, heat insulation and sound insulation shall be applied to accommodation quarter in accordance with relevant Rules and Regulations as follows.

Approximate bulk density of insulation material shall be as follows or according to

Maker’s recommendation:

| Heat and sound insulation | Rock wool (abt.100 kg/m3) or Glass wool (abt.40 kg/m3) |
| --- | --- |
| Fire insulation | Ceramic fiber (as per maker's standard ) |

3.2.2.1	Fire Insulation

Fire insulation shall be provided according to requirements of SOLAS.

3.2.2.2	Heat Insulation

The parts of steel walls and decks of compartment in accommodation space exposed to weather shall be insulated in accordance with the following table:

| Material | Application |
| --- | --- |
| 50 mm Glass wool with 25 mm beam cover. | Deck head where exposed to weather of living space |
| 100 mm Glass wool with 25 mm beam cover | Deck head of wheelhouse |
| 50mm Glass wool with 25 mm stiffener cover. | Bulkhead exposed to weather of living space |

The other spaces not mentioned on the above table shall have no insulation.

As for the spaces partially exposed to weather, the insulation shall be extended by 450mm inside beyond the boundary as far as practicable. The structural members greater than 450mm in depth shall be insulated by 450mm in depth and the remaining parts shall be left bare.

3.2.2.3	Sound Isolation

Sound isolation shall be designed to fulfill the requirement of IMO Res. MSC.337 (91).

About 50mm glass wool covered with glass cloth shall be fitted if necessary. Attachments of beams, girders, stiffeners etc. to be completely insulated with 25 mm glass wool.

Sound insulation shall be provided in Air conditioning unit room, Engine control room and E/R fan rooms.

Detailed solutions of sound isolation shall be decided based on results of noise analysis and prediction during detail design stage.

3.2.2.4	Coverage of Insulation

The surface of insulation material shall be covered with as follows:

| Material | Application |
| --- | --- |
| Glass cloth | Protected insulation by wall/ceiling panels or galvanized steel sheet |
| Glass cloth coated with aluminum foil | Unprotected insulation, which is covered by neither wall/ceiling panels nor galvanized steel sheet |

3.2.3	DECK COVERING

Deck covering shall be applied as per following list:

| Material | Application |
| --- | --- |
| A-60 deck covering | according to SOLAS requirement |
| 5 mm thick full synthetic carpet on 10 mm latex type deck composition, | Captain class bed room |
| 2mm PVC on 10mm latex type deck composition, | Other living space, corridors, public spaces and office space |
| 3mm thick rubber tile including studs (non-slip type) on 10mm latex type deck composition | Wheelhouse and ECR |
| Ceramic tile on 30 mm cement (Anti-slip type) | Galley, pantry, laundry and drying room, all toilets, and lavatories (as per maker’s standard) |
| Paint (steel deck ) | Store and locker space, deck workshop, machinery space, all exposed steel decks and other similar spaces (except mentioned above.) |

Deck covering shall not be applied on all the weather deck surfaces.

The floor top covering or finished coat on latex type deck composition shall not be applied under fixed furniture with baseboard such as bed, wardrobe, locker etc.

Floating floor shall be applied for Engine control room.

The type of floating floor maybe apply according to noise and vibration analyses suggestion.

Gutter ways shall be fitted for galley which formed by stainless steel plate and covered by stainless steel cover.

The space applied with joiner lining and partition shall be provided with approx. 60mm high plastic skirting. The space applied with tile shall be provided with approx. 150mm high tile skirting.

The deck covering for wet space shall slope to scupper.

Non-slip type rubber mat with 1,000 mm breath and 3mm thickness shall be provided to the front and back side of the Main Switchboard (MSB), Transformers, emergency switchboard and local group starter panel. These mats will be certified.

Perforated rubber mat of 15 mm thickness shall be provided on navigation bridge wing area.

  - WINDOW, DOOR, STAIRWAY, ETC.

3.3.1	WINDOW

The quantity of windows for cabins and rooms to be provided according to the following standard.

| Location | Number |
| --- | --- |
| Captain class dayroom (one window size will be 800x700) | 2 |
| Senior officer class dayroom | 2 |
| Captain / Senior officer class bedroom, Junior officer / Petty officer / Rating class cabin | 1 |
| Crew’s mess room, Ship office | 3 |
| Officer’s mess room, Gymnasium Recreation room , Meeting room, Hospital, Galley | 2 |
| pantry | 1 |
| Engine control room | 2 |

The size, shape and material of windows as shown in following table.

| Size(mm) | Shape | Material | Location |
| --- | --- | --- | --- |
| 800 x 700 | Rectangular | Steel | Recreation room, mess rooms |
| 450x630 | Rectangular | Steel | All living space, public space (except above), office space and catering space |
| abt. * 1,900 x 1,200 fixed type | Rectangular | Steel | Wheelhouse front wall |
| abt. * 1,500 x 1,200 fixed type | Rectangular | Steel | Wheelhouse aft and side wall |
| abt. 1,200 x 650 | Rectangular | Steel | Engine control room |

* Size and quantity of wheelhouse window may be adjusted in accordance with the wheelhouse arrangement in detail design stage.

No mosquito screen to be provided.

The glass for windows shall be of safety glass (tempered and or shatter proof). The frame of windows shall be of painted steel and welded to steel wall, in general. Coaming to be steel type. The glass retaining frame to be aluminum alloy. Steel window box to be fitted except that wheelhouse windows..

All opening type windows should be opened inward. Where two windows or three windows adjacent, only one to be of the opening type.

Center line height of windows to be about 1,500 mm from steel deck.

Cleaning stage or access way of about 600 mm width for wheelhouse front windows shall be provided. A door shall be provided for access from navigation bridge wing to cleaning stage respectively for port side and starboard.

Wheelhouse front windows shall be washed by spraying fresh water from the pipe outside the windows.

The sun screens of roller type to be fitted on front windows of wheelhouse.

The ‘Blind Sector’ roller type to be fitted on Placement of chart table and radio station.

Three (3) windows in front of wheel house shall be provided with electric heater.

Two (2) rotary clear view screens to be provided in front of wheel house window.

.3.3.2	DOOR

Doors shall be arranged according to G.A. plan.

Mosquito screen will not be fitted.

3.3.2.1	Steel Door

Steel doors shall be hinged type, except specified, and shall be provided as follows:

| Type | Material | Clear width (mm) | Location |
| --- | --- | --- | --- |
| Weather tight | Steel plate | 800 | - Entrance from exposed deck to corridor （except wheel house） - Entrance from exposed Upper deck to CO2 room - Entrance from exposed deck to Dispensary - Entrance from exposed deck to Bosun store, paint store |
| Weather tight | Steel plate | 700 | - Entrance from exposed Upper deck to Garbage store - Entrance from exposed A deck to Emergency generator room - Entrance to Bosun store, Safety equipment store - Entrance from exposed deck to Refrigerated chamber - Entrance from exposed deck to Gym - Entrance from exposed deck to Steering gear room stair - Entrance from exposed deck to engine room escape |
| Weather tight | Steel plate | 650 | - All other weather tight door form expose deck to cabin. |
| Watertight, | Steel plate | 700 | - Entrance from E/R to steering gear room (subject to Rule requirement), |
| Watertight, | Steel plate | 650 | - Entrance from E/R to side void space, (subject to Rule requirement) |
| Steel door with ventilation grille | Steel plate | 650 | - Oxygen/acetylene bottle store |
| Sliding type, hollow with square window | Stainless steel | 1,000 | - Entrance from exposed Bridge deck to Wheelhouse |
| Wire mesh door | Steel | 700 | - ER spare room, elec. store |

Notes:

*1) watertight door to be provided with open/close indication on wheelhouse.

2) The tightness of the steel doors (weather-tight or watertight) may be adjusted based on requirement of damage stability.

All steel doors shall be of rectangular, hinged type with round corner, except otherwise specified.

Steel hinged single handle weather door with fixed rectangular window shall be provided for entrance door to accommodation corridor, cylinder lock will be install for the entrance door that fulfill with ISPS rules. .

Weather tight steel doors shall be of welding of seal channels continuous with synthetic rubber gasket, and pad lock eye plate( or cylinder lock) and fixing hook and door stopper according to the purpose intended, and shall be provided in accordance with the requirement of the Rule/Regulations and the Builder’s standard.

The height from covering of the deck to the top of steel door opening shall be of 2,000 mm.

Height of weather tight steel door sill shall meet ICLL requirement. Door sill of entrance from weather deck shall be abt. 150 mm high above steel deck where there is no Rule requirement.

Sill height shall be measured from top of deck plate, even where deck covering is applied subject of LL requirement.

Foot step to be installed only to sills with height equal and above 600 mm.

The weather tight door for emergency escape trunk shall be of quick-open type.

3.3.2.2	Internal Accommodation Door

Stairway doors to be held open on magnetic catches and released by the fire alarm system.

| Kind of Doors | Material and Type | Location |
| --- | --- | --- |
| "A-60" class fire protection | "A-60" class approved, steel sash type | In accordance with SOLAS |
| "A-0" class fire protection | "A-0" class approved, steel sash type | In accordance with SOLAS |
| "B" class fire protection | "B" class approved, steel sash type | In accordance with SOLAS |
| Light type (P.V.C. film) | Steel sash hollow door with vent mesh | Private toilet |
| Sound proof type | Pontoon type steel door with sound insulation | Engine control room |

Notes:

    - Self-closing door devices shall be fitted according to rules requirement.

    - Door catcher with cushion shall be fitted on all doors except those with self-closing device.

Clear height of internal accommodation door to be abt.1,950mm above the surface deck covering except door of private toilet and the door between the two spaces that the deck covering's thickness difference is more than 90mm.

Clear width of internal accommodation door opening to be as shown in the following table.

| Clear width (mm) | Location |
| --- | --- |
| 800 | Entrance to corridor, Galley, From Pantry to Officer’s mess room, Dry provision |
| 700 | Stairways, Fire control station, Lobby, Mess rooms, Ship office, Recreation room, Meeting room, escape in ER, Fire control station, entrance door to ER, |
| 600 | Private lavatory |
| 650 | All other internal accommodation doors |

3.3.2.3	Hardware

All hardware shall be of marine type.

3.3.2.3.1	Lock

All door handles shall be of lever type，the springs to be moderately heavy type.

All locks and latches shall have rim or mortise type casing to meet the requirements of particular installations.

Locks shall be furnished so that each lock will have a different key, and all locks except lavatory locks can be opened by a master key.

All living cabins shall have mortise type cylinder lock and latch set with thumb turn.

Public space, catering space, wheelhouse, fire control station, engine control room, laundry, changing room and inside lockers shall have mortise type cylinder lock and latch set.

Weather doors on A to F deck and wheelhouse to have mortise lock.

Public/deck toilet and hospital bathroom door shall have mortise type latch with "vacant" indicator. Bonded store door shall have padlock made of brass.

Weather deck steel doors shall have latch， padlock or cylinder lock where necessary.

General Anti-pirate locking device to be provided for all external doors.

Locking device only openable from inside shall be fitted at steel doors to accommodation area and engine room area from upper deck as per ISPS.

3.3.2.3.2	Key

Three (3) sets of key shall be supplied for each lock set except "vacant" lock.

Keys shall be divided into user’s group in accordance with shipboard practice.

Key of each door to be different, doors of one room to use the same key.

Three (3) sets of master key shall be provided.

All keys shall be supplied with a plastic identification tag. A list of keys identifying the cabin and number shall be supplied by the Builder.

The tag shall be marked in English only.

3.3.2.3.3	Door Closer

Door self-closing device shall be provided for the following locations except steel doors :

      - Engine room entrance

      - Stairway

      - Electric cable trunk, A.C. ventilation trunk and pipe tunnel in accommodation block

      - Public toilet

      - Inside passage entrance door on upper deck, A deck and B deck

      - Engine control room

      - Galley

      - Wheel house

      - A-class, B-class fire doors (if required by Class rules)

Door closers shall be of sufficient size to close the doors under all conditions of list and trim.

3.3.3	STAIRWAY AND STORM RAIL IN ACCOMMODATION QUARTER

3.3.3.1	Stairway Dimension and Material

| Clear Breadth | 800mm |
| --- | --- |
| Inclination | Approx.45° ~ 50°to horizontal |
| Side frame | Steel, painted |
| Step | Steel, covered with 2mmthick vinyl sheet |
| Step guard (non-slip) | Copper |
| Hand rail | Stainless steel pipe about Φ42 |
| Stanchion | Stainless steel pipe about Φ20 |

3.3.3.2	Storm Rail

Stainless steel (SUS 304) storm rail shall be provided at one side of the corridor at 1.0 m height above deck in accommodation quarter and front of wheelhouse according to the Builder’s practice.

3.4     FURNITURE AND UPHOLSTERY

3.4.1	FURNITURE

3.4.1.1	General

The furniture and fittings shall be of strong construction for shipboard using and in harmony with decorative treatment of the space and shall be in accordance with the Builder’s practice.

Main furniture and fittings shall be as described in detail and the size and number of furniture may be slightly changed by the Cabin Arrangement Plan.

All beds shall be arranged in direction of longitudinal and sofas in private rooms shall be arranged in direction of transverse as far as possible. If not they must be in opposite direction.

In Cabins and Public areas where entertainment appliances are provided must be supplied from individual power socket. An Extra power socket should also be provided at each area.

The general features and size of furniture shall be as listed up in the following. Color scheme book shall be submitted to Buyer for approval.

The surface material of furniture in general shall be as follows.

| Item | Material |
| --- | --- |
| Steel | Spray paint or chromium plating |
| Wood | Polyurethane finish laminated sheets |
| Chair | Wood (Living cabin, Conference room) |
| Fixed revolving steel chair (mess room) | Steel |

In general the furniture to be constructed of the plywood/chipboard cover with decoration plate and trimmed with high quality hardwood in all cabin classes and formaldehyde level to satisfy Chinese Furniture Standard melamine film in visible part only.

Top of all tables and other horizontal top surfaces to be finished with melamine laminate.

Chairs and movable sofas shall be provided with fastening device. Other furniture shall be fixed to wall, deck and/or ceiling.

3.4.1.2	Dimension of Large Furniture

| Class | Bed | Wardrobe | Desk |
| --- | --- | --- | --- |
| Captain | 2,000 x 1,500 | 1,200 x 600 | 2,000 x 800 |
| Senior Officer | 2,000 x 1,200 | 800 x 600 | 1,800 x 700 |
| Junior Officer | 2,000 x 1,200 | 800 x 600 | 1,400 x 700 |
| Petty officer & Rating | 2,000 x 1,100 | 800 x 600 | 1,000 x 600 |

3.4.1.3	Furniture

The list below is for information. The final amount and arrangement to be according to cabin’s arrangement which approved by buyer.

One (1) full size mirror to be provided in the wardrobe.

All drawers and doors of furniture’s to be fitted with snap locks. Locks shall be provided for wardrobes and the uppermost tier of drawer for desk.

All beds shall be provided with two (2) drawers underneath (except for bed in hospital).

Heavy furniture normally will be standing type. If heavy furniture only mounting on wall reinforce plate will pre install inside the lining.

All edges of furniture must be rounded off with no sharp edges.

Life jacket and immersion suit to be fitted in a dedicated shelf in wardrobe.

All refrigerators should be using environmentally friendly gas.

Cabins

| Furniture | Captain class |  | Senior officer |  | Junior officer | Petty officer | Rating |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dayroom | Bed room | Dayroom | Bed room |  |  |  |
| Bed |  | 1 |  | 1 | 1 | 1 | 1 |
| Wardrobe |  | 1 |  | 1 | 1 | 1 | 1 |
| Night table |  | 1 |  | 1 | 1 |  |  |
| Desk | 1 |  | 1 |  | 1 | 1 | 1 |
| Book rack | 1 |  | 1 |  | 1 | 1 | 1 |
| File cabinet | 1 |  | 1 |  |  |  |  |
| Sofa | 1 |  | 1 |  | 1 | 1 | 1 |
| Tea table | 1 |  | 1 |  | 1 |  |  |
| Rev. arm chair | 1 |  | 1 |  |  |  |  |
| Arm chair | 1 |  | 1 |  | 1 |  |  |
| Chair |  |  |  |  |  | 1 | 1 |
| Coat hook | 2 |  | 2 |  | 2 | 2 | 2 |
| Side board with Refrigerator | 1 (120L) |  | 1 (120L) |  | 1 (50L) | 1 (50L) |  |

Notes:

*1) Rev. arm chair in day room shall be of high back type with wheels and brake lever.

*2) Book racks and open shelves to be provided with guardrails

Mess room

| Item | Number |  |
| --- | --- | --- |
|  | Officer’s | Crew’s |
| Mess table | 2 | 2 |
| Side board | 1 | 1 |
| Mess chair | 12 | 14 |
| Bottle cabinet | 1 | 1 |
| Notice board | 1 | 1 |
| Coat hook | 12 | 12 |
| 42” TV with rack | 1(Buyer’s supply) | 1(Buyer’s supply) |
| Water boiler (abt. 10L) | 1 | 1 |

Recreation room

| Item | Number |
| --- | --- |
| Side table | 1 |
| Cabinet and shelve | 1 |
| Sofa with tea table | 1 |
| Color TV with DVD set | 1 (Buyer’s supply) |
| Game table | 2 |
| Chair | 8 |
| Coat hook | 10 |

Gymnasium

| Item | Number |
| --- | --- |
| Ping-pong table | 1 set |
| Bench | 1 |
| Coat hook | 10 |
| Notice board | 1 |
| Side board | 1 |
| Folding chair | 4 |
| Steel folding bed fore Suez crew | 6 |

Office

| Item | Number |  |  |
| --- | --- | --- | --- |
|  | Ship office | Tally office | Meeting room |
| 2-cabinet desk | 2 | 1 |  |
| Conference table | 1 | - | 1 |
| Office cabinet | 2 | - |  |
| File cabinet | 1 | 1 |  |
| Coat hook | 6 | 2 | 12 |
| White board | 1 | 1 |  |
| Tank sounding board (black letters) | 1 | - |  |
| Book rack | 1 | - |  |
| Chair | 6 | 1 | 12 |
| Revolving chair with armrests | 3 | - |  |
| Side table |  |  | 1 |
| Refrigerator (abt. 100 L) | 1 | 1 | 1 |

Hospitaland Medicine locker

| Item | Number |
| --- | --- |
| Bed (steel with adjustable type) | 1 |
| Curtain around the bed | 1 |
| Wardrobe | 1 |
| Desk | 1 |
| Chair | 2 |
| Wash basin | 1 |
| Mirror | 1 |
| O2 bottle and holder | 1 |
| Stretcher | 1 |
| Garbage bin (medical type) | 1 |
| Coat hook | 2 |
| Medical cabinet (LxWxH) | 1 |
| First-aid cabinet | 1 |
| Refrigerator (abt. 100 L) | 1 |

Control space

| Item | Number |  |
| --- | --- | --- |
|  | Wheelhouse | Engine control room |
| Chart table (with chronometer) | 1 | - |
| Binocular box | 2 | - |
| Log book and ballast note board | 1(each) | - |
| Flag locker | 1 | - |
| Flag pigeon box | 1 |  |
| Book rack/file rack | 1 | 1 |
| File cabinet | - | 2 |
| Pilot chair | 1 | - |
| Rev. arm chair | 2 | 2 |
| Sofa | 1 | - |
| Tea table | 1 | - |
| Sounding board | - | 1 |
| Notice board | - | 1 |
| White board | 1 | 1 |
| Kettle rack | 1 | 1 |
| Folding desk front bridge | 2 (fore blk. P/S each) | - |
| Coat hook | 4 | - |
| Working Desk | 1 | 1 |
| Chair with sliding rail | 2 | - |
| Chair | - | 1 |
| Refrigerator (abt. 100 L) | 1 | 1 |

Working space

|  | Electrical workshop | Engine room workshop | Others |
| --- | --- | --- | --- |
| Fittings & Furniture | 1- Shelf (2-tier) (where necessary) 1- Working table (with 2 drawers and cabinet) | 1- Shelf (3-tier) (where necessary) 1- Working table (with 2 drawers and cabinet) 1- Tool locker | 1- Shelf (2-tier) (where necessary) |

3.4.2	UPHOLSTERY

3.4.2.1	Surface Material of Sofa and Chair

Upholstery for sofa and chair shall be as follows:

Woven fabric - Captain class cabins, Senior officer class cabins, Recreation room

Vinyl leather: Other cabins and other public spaces.

3.4.2.2	Stuffing of Sofa, Chair and Mattress

Upholstery of mattress: Cotton cloth

Stuffing of mattress: Polyurethane sponge with coil spring

Stuffing of sofa and chair: Polyurethane sponge

3.4.2.3	Curtain

| Item | Application |
| --- | --- |
| Black-out type curtain | Window of all living space, public space, ship office |
| Curtain with blind cloth behind | Between wheel house and chart space |
| Vinyl curtain | Shower space |
| Bed curtain | Hospital |

3.5	COMMISSARY AND SANITARY OUTFIT

3.5.1	COMMISSARY EQUIPMENT

3.5.1.1	Material

Material of equipment for catering space shall be generally as follows:

| Equipment | Material |
| --- | --- |
| Sink, Top of dresser, Plate or rack, Top of cooking table | Stainless steel |
| Furniture in galley | Stainless steel |
| Cooking machinery | Maker’s standard |

3.5.1.2	Equipment

Equipment shall be provided as following list.

3.5.1.2.1	Galley equipment

| Item | Number |
| --- | --- |
| Galley hood with grease filter | 1 |
| Cooking range with 4-hot plates | 1 |
| Baking and roasting oven | 1 |
| Refrigerator(abt. 500L,domestic type) | 1 |
| Titling frying pan (abt. 10 kW) | 1 |
| Meat slicer | 1 |
| Universal kitchen machine (abt.20L) | 1 |
| Water boiler (abt. 15L) | 1 |
| Electric waste disposer | 1 |
| Can opener | 1 |
| Toaster (domestic type, 4-slice) | 1 |
| Dresser with two sinks (side) | 1 |
| Working table with one sink (center) | 1 |
| Bowl and dish rack | 1 |
| Bottle rack | 1 |
| Chopping board with rack | 2 |
| Glass rack | 1 |
| Service table with shelf | 1 |
| White board | 1 |
| Clock | 1 |
| Rice cooker | 2 |
| knife rest | 1 |

3.5.1.2.2	Pantries

| Item | Number |
| --- | --- |
| Dishwasher | 1 |
| Refrigerator(abt. 200L,domestic type) | 1 |
| Microwave oven | 2 |
| Table with two sinks | 1 |
| Rack (stainless steel) | 1 |
| Shelf (3 tiers, stainless steel) | 1 |
| Whiteboard | 1 |
| Garbage can | 1 |
| Bread toaster | 1 |
| Electrical water boiler (abt. 15L) | 1 |
| Coffee machine | 1 |

3.5.2	SANITARY OUTFIT

3.5.2.1	Material

Private toilet and hospital toilet shall be of prefabricated unit type.

Wash basin shall be of white vitreous china for toilet.

All wash basins shall be provided with hot and cold mixing valve with one (1) outlet. An S-shaped drainage trap shall be fitted.

Water closet to be of gravity type and fitted with flush valve, and seat and cover to be of plastic material.

Bathtub shall be constructed of plastic or white china and fitted with shower, hot and cold water mixing valve and drain plug shall be provided.

Shower shall have flexible hose, and fitted with hot and cold water mixing valve, and the angle of shower head shall be adjustable.

Stop valves shall be provided for each hot and cold water pipe below wash basin.

Paper holder, storm rail, towel rail and soap dish shall be provided for individual use as necessary.

3.5.2.2	Fittings in Lavatory and Changing Room

Private lavatory	Shower set, water closet, wash basin and toilet cabinet with mirror, paper holder, soap dish, towel bar, hand grip, coat/hat hook

Public toilet /Stevedore’s toilet	Water closet, washbasin and lavatory mirror, paper holder, soap dish, towel bar, hand grip, coat/hat hook

Hospital lavatory	Bath tub, water closet, wash basin and toilet cabinet with mirror, paper holder, soap dish, towel bar, handgrip, coat/hat hook

| Changing room | 1 - Wash basin (with mirror, soap dish, towel bar) 1 - Wooden bench 13 - Steel cloth locker 8 – Coat hook |
| --- | --- |

Bathtub to be provided for captain class.

The main pipe of the soil pipes will arrange the corridor ceiling as far as possible.

3.5.2.3	Equipment in Laundry and Drying Room

3.5.2.3.1	Officer’s Laundry

2-	Washing machine with front loading (abt. 5kg capacity, domestic type)

1 -	Drying machine (abt. 5kg capacity, domestic type)

1-	Electric steam iron

1-	Ironing table

1-	Washing sink (stainless steel) with rack on wall

3.5.2.3.2	Crew’s Laundry

1-	Washing machine with front loading (abt. 10kg capacity, domestic type)

1-	Washing machine (abt.5kg capacity, domestic type)

1-	Drying machine (abt. 5kg capacity, domestic type)

1-	Table with shelf under

1-	Electric steam iron

1-	Ironing table

1-	Washing sink (stainless steel) with rack on wall

3.5.2.3.3	Drying Room

One (1) drying room to be provided for each laundry with electric heater.

1-  Electric heater (abt. 1.5 kW)

4-  Cloth hanging rail of stainless steel pipe

3.5.3	DRINK WATER FOUNTAIN

Drink water fountains shall be fitted as follows:

1-  Wheelhouse

2-  officer mess room and crew mess room

1-  Near Engine control room

- Corridor on B deck

3.6	  STORE

3.6.1	REFRIGERATED PROVISION STORE

3.6.1.1	General

Cold rooms to be equipped with lockable quarantine shelves or lockers.

|  | Temperature (0C) | Volume (m3) |
| --- | --- | --- |
| Lobby | No cooling | abt. 10 |
| Dry provision | Cooled by A/C | abt. 20 |
| Vegetable room | +4 | abt. 20 |
| Meat room | –18 | abt. 15 |
| Fish room | –18 | abt. 7 |

| L. T. cooling F.W. temperature | 36oC |
| --- | --- |
| Ambient temperature | 35oC |

Measurement of volume shall be taken to the inner surface of insulation and lining.

Volume of each compartment may be adjusted in accordance with detail design of the accommodation arrangement.

In general, finished ceiling height of compartments shall be not less than 1.9 meters.

3.6.1.2	Insulation

Insulation for refrigerated store:

Side	:	120 mm thick polyurethane foam(PUF)

Top	:	120 mm thick PUF

Partition	:	120 mm thick PUF

Bottom	:	120 mm thick PUF + 12 mm thick plywood + 1.0 mm thick stainless steel plate

The polyurethane foam panel shall be coated with 0.6mm thick stainless steel sheet for visible parts and 0.6 mm thick galvanized steel sheet for invisible parts.

3.6.1.3	Door

| Door type | : | Air-tight pontoon, hinged type. |
| --- | --- | --- |
| Material | : | Stainless steel filled with PUF insulation. |
| Dimensions | : | Abt. 750mmx 1,700mm |

Necessary door fittings such as locking device, open able and unlocking device from inside and door holder, etc. shall be provided.

3.6.1.4	Furnishing

Meat, fish and vegetable room shall be fitted with 2-tier shelves made of stainless steel.

Thermal expansion valve control panel shall be fitted near the chamber.

A remote reading thermometer shall be fitted for each room and an indicator shall be mounted in the passage in vicinity of the lobby. One (1) ozone generator shall be provided in vegetable room.

Alarm bell shall be fitted in galley with signal light outside the compartment and push button arranged in each chamber and lobby. Other details refer to "- Electric Part".

A meat chopping block and a sink with fresh hot and potable water supply shall be provided in lobby.

Plastic gratings shall be fitted on the floor.

Water seal scupper shall be provided for each compartment.

3.6.1.5	Refrigerating Plant

The refrigerating system shall be of refrigerant R-407Cdirect expansion with automatic control device.

Two(2) sets of condensing unit shall be arranged in Air conditioning unit room(one as stand-by) and each unit shall be consisted of compressor and condenser and necessary accessories as per Maker’s standard.

Cooling water shall be supplied by low temperature cooling F.W. system in engine room.

The refrigerating plant shall be capable to maintain the specified temperatures with one(1) compressor working at full load not more than 16 hours per day. The other compressor shall serve as standby. Cooling down to specified temperatures shall be worked by two compressors. Ambient temperature of atmosphere shall be based on 35C and cooling water temperature of 36C. Compressors shall be automatically started and stopped to maintain the specified temperatures.

Each compartment shall be cooled by one (1) ceiling unit cooler. Diffuser shall consist of an aluminum fin cooling coil and an electric fan contained in an enameled metal case. Drain pan shall be fitted and drain pipe shall be led to the scupper in the chamber.

Unit cooler, drain pan and pipe shall be equipped with electric heating coils for defrosting (auto operation) for meat room and fish room.

Thermostat, pressure gauges, remote reading thermometers, necessary valves and safety valves, sight glass, and gas charging device, etc., shall be fitted completely.

De-frosting of diffusers shall be provided for meat room and fish room with electric heater with time controller.

Materials for compressors and condensers, etc. shall be in accordance with Maker's standard.

3.6.2	DRY PROVISION STORE

| Compartment | Facility |
| --- | --- |
| Dry provision store | 3-tiergalvanized steel shelves with wooden batten |

3.6.3	MISCELLANEOUS STORES

| Compartment | Facility |
| --- | --- |
| Bonded store | 2-tier steel shelves with fiddles and wooden grating |
| Linen store | 2-tier wooden shelves |
| Electrical equipment room | 2-tier steel shelves with wooden batten |
| Navigation equipment room | 2-tier steel shelves with wooden batten |
| Paint store | 2-oil cans, tin hooks, 2-tier steel shelves with wooden batten |
| Bosun store | 2-tier steel shelves with wooden batten, working desk and tool box |
| Rope store | 2-tier steel shelves with wooden batten |
| Battery room | steel shelves with wooden plate and F.R.P. box for storage of batteries and shelves for distilled water bottle etc., wooden grating |
| Oxygen/Acetylene store | steel shelves, wooden grating Removable “U” rings with screw shall be provided in acetylene/oxygen bottle space. |
| Other stores & lockers | steel shelves, wooden batten |

3.7  ELEVATOR

No elevator to be provided.

CHAPTER 4

OUTFITTING

Version:A

Key words: Breadth 27.5m, Gearless, 6S60ME-C10.5 Tier II, 220ReeferPlugs, Open loop scrubber

CONTENT

CHAPTER 4 OUTFITTING	3

4.1     anchoring AND MOORING EQUIPMENT	3

4.1.1   Deck Machinery in General	3

4.1.2   WINDLASS AND MOORING WINCH	4

4.1.3	ANCHOR and  CHAIN CABLE	5

4.1.4	HAWSE PIPE AND CHAIN PIPE	6

4.1.5	mooring fitting AND MOORING ROPE	7

4.2	STEERING SYSTEM	8

4.2.1	STEERING GEAR	8

4.2.2	RUDDER AND RUDDER STOCK AND OTHERS	9

4.2.3	BOWTHRUSTER	10

4.3	CARGO HATCH COVER	11

4.3.1	CARGO HATCH	11

4.3.2	CARGO HATCH cover	12

4.4     CONTAINER lashing and securing	14

4.4.1	CONTAINER arrangement	14

4.4.2	Lashing bridge and reefer maintenance platform	15

4.4.3	container DECK STANCHION and operation platform on weather deck	16

4.4.4	Cellular Construction	16

4.4.5	Others	17

4.5	  CARGO GEAR	18

4.5.1	CARGO gear	18

4.6	LIFE SAVING APPLIANCE	18

4.6.1	LIFEBOAT WITH EQUIPMENT	18

4.6.2	Lifeboat davit and winch	18

4.6.3	LIFE RAFT AND DAVIT	20

4.6.4	PERSONNEL LIFESAVINGS	20

4.7      deck outFITTING	22

4.7.1    ship’s service handling gear	22

4.7.2	ACCOMMODATION LADDER AND PILOT Rope LADDER	22

4.7.3	FORE MAST, RADAR MAST, FORE AND STERN FLAG STAFF	23

4.7.4	CANVAS COVER	24

4.7.5	NAME PLATE AND MARKS	24

4.7.6	WORK RAFT	25

4.7.7	GARBAGE store	25

4.7.8	Pipe tunnel trolley	26

4.8	outfitting for access	26

4.8.1	access HATCH, MANHOLEcover and steel door	26

4.8.2	steel ladder and HAND RAIL	27

4.9     SPARE PART AND DECK INVENTORY	29

4.10	NAVIGATION EQUIPMENT	30

4.10.1	NAVIGATION EQUIPMENT	30

4.10.2	SOUND SIGNAL AND SHAPE EQUIPMENT	30

4.10.3	FLAG	30

4.10.4	VOICE TUBE	30

CHAPTER 4 OUTFITTING

4.1     anchoring AND MOORING EQUIPMENT

4.1.1   Deck Machinery in General

The arrangement of windlass and mooring winch shall be in accordance with the requirement of the Classification Society and Panama Canal regulations of 2020. Final arrangement of winches and mooring fittings shall subject to detailed Mooring Fitting Arrangement Plan during detail design stage.

Electric hydraulic type deck machinery with auto-tension device for mooring drum shall be provided as shown on the G.A. plan.

Electric heater to be provided for electrical motor of deck machinery.

Gear for cable lifter part shall be of open type with steel cover for dust protection and gear for mooring winches shall be of enclosed type.

The mooring drum shall be of split type and large enough to wind the required mooring rope of not less than 220 m long and 64mm diameter(size subject to maker’s recommendation) on storage of the drum.

Warping head shall be of cast iron smooth barrel tube type and shall be directly connected to driving shaft without clutching device.

Protection plate or pipe to be provided on top of the hydraulic motor especially if the motor is axial piston type.

Each cable lifter and mooring drum shall be fitted with two (2) halves type hand operated friction band brake and clutch operated locally. Brake drum surface for cable lifter and mooring drum shall be of mild steel. Break lining wear indication method to be provided as per maker’s standard.

Grease nipple to be provided for greasing the pin of brake bands and all pins. Grease

nipples to be of the same type as far as possible.

Seat of cable lifters shall be of open type with height as low as possible.

Steel chock for cable lifters and resin chock for mooring winches shall be used for the liner of foundation.

The windlass/mooring winch shall be operated by local control. The control handle combined with a control stand shall be provided near the driving motor of each windlass and mooring winch at each machine side for local control. The clutch of mooring winch shall be provided with manual control device at machine side for local control.

Shipside remote control stand for the mooring winch to be provided with following function:

For mooring winch: remote control stand shall be combined with auto-tension control, speed, emergency stop, direction change

Material, construction, spare parts, etc. shall be in accordance with the manufacturer's standards and shall comply with requirements of Classification Society.

The stainless steel name plate shall be provided for each combined windlass/mooring winch and mooring winch.

Snap back zones to be indicated on the mooring arrangements and paint marked on deck.

It is safe practice to provide rope guard around the mooring winch operation platform.

All platforms frame for windlass and winches to be galvanized open grating

4.1.2   WINDLASS AND MOORING WINCH

4.1.2.1	Combined Windlass/Mooring Winch

Two (2) sets of combined windlass/mooring winch of independent type shall be located at forecastle deck.

Each combined windlass/mooring winch shall be composed of, one (1) declutchable cable lifter, two(2) declutchable mooring drum and one (1) warping head.

Gypsy wheel and mooring drum shall not be operated simultaneously.

The mooring drum shall be designed to take one whole rope on six (6) or seven (7) layers, declutched to the shaft and operated through clutch.

Brake holding force of windlass shall be sufficient in accordance with the requirements of Classification Society.

It has to be ensured that all flanges of the hydraulic pipes at the pumping station and on deck near the windlass and mooring winches fall with in the oil coaming.

Particulars of each combined windlass/mooring winch shall be as follows.

| Item | Gypsy | Mooring drum | Warping head |
| --- | --- | --- | --- |
| Quantity | 1 | 2 | 1 |
| Capacity | A rated line pull in compliance with the requirement of the Classification Society at average hoisting speed of about 9m/min | per drum abt.15tx15m/min (on the first layer) | abt.15t |
| Standard for chain, rope and capacity | Φ70, Grade 3 chain of welding. Joining shackles shall be of Kenter type | Φ64x 220m rope (size subject to maker’s recommendation) With min. strength abt.638KN | six(6) turns |

4.1.2.2	Mooring Winch

Two (2) sets of mooring winch shall be provided. Mooring winches to be split type drums.

| Location | Quantity | Combination |  |
| --- | --- | --- | --- |
|  |  | Mooring drum | Warping head |
| deck aft. | 2 | 2 | 1 |

Particulars:

| Item | drum | Warping head |
| --- | --- | --- |
| Rated Capacity | Abt. 15 t x 15 m/min | abt.15 t |
| Rope capacity and standard | Φ64 x 220m(size subject to maker’s recommendation) | 6 turns |

The rated capacity of each mooring winch shall be obtained on the first layer of the drum at operation of single drum.

Slack rope speed shall comply with manufacturer's standards and Rule/Regulations of Classification Society.

4.1.3	ANCHOR and  CHAIN CABLE

The equipment in accordance with Rules requirements shall be provided.

4.1.3.1	Anchor and Chain Cable

Two (2) bower anchors of stockless, SPEK type shall be provided for each vessel. Each weight is 6225 kg.

Anchor chain cable shall be of flush butt welded stud link, grade 3 type and provided in accordance with the requirement of the Classification Society.

The diameter of the anchor chain cable shall be 70mm and total length shall be 632.5m as per Rule requirement. Anchor chain cable shall be jointed with Kenter type shackle at every 27.5 m length. A swivel piece shall be fitted at each extreme outboard end of the anchor chain cable.

Anchor chain cable shall be marked with red paint at every Kenter shackle and white paint at same number of common links as shot numbers and stainless steel (SUS304) band tape at each last common link.

End link of chain cable shall be fastened to cable clench located outside each chain locker. These shall be released manually from bosun store in an emergency.

Accessories for anchor chain shall be supplied according to Maker’s standard in compliance with Rules and as below

6 - Chain hook

2 – Pin punch

1 – Shackle punch

2 – Spare kenter shackle

1 – Spare end shackle

1 – Spare taper pin for anchor shackle

3 – Spare taper pin for kenter shackle

1 – Pin hammer

1 – Shackle hammer

1 – Disengaging tool for kenter shackle

4.1.3.2	Chain Stopper

One (1) set of roller type chain stopper shall be arranged between each hawse pipe and windlass.

The chain stopper shall be of welded steel frame construction with a cast steel roller and a bar type stopper.

Necessary turnbuckle and steel wire rope shall be provided for fastening the anchor chain.

4.1.4	HAWSE PIPE AND CHAIN PIPE

4.1.4.1	Hawse Pipe

Location: one(1) at each side of forecastle deck forward.

| Material: | Pipe | Welded steel plate |
| --- | --- | --- |
|  | Bell mouth | Welded steel plate and cast steel |

Deck flange shall be fitted at the deck openings of the hawse pipes to allow satisfactory housing of the chain cable and easy leading to the gypsy wheel. Attention shall be paid to the anchoring arrangement to obtain good clutch of the chain cables round the cable holders. Fixed guard rails shall be fitted around the deck flanges.

The bell mouth shall extent to outside to ensure the clearance of min. 300 mm between anchor and side shell under ship upright condition on even keel. Anchor when stowed in the bell mouth should have 3 point contact.

Galvanized steel cover plate shall be provided for hawse pipe opening on forecastle deck, and fixed with stainless steel stud-bolt and pin. Three (3) nozzles shall be provided in hawse pipes for cable chain.

Watching platform and protection handrail shall be arranged on the forecastle bulwark in suitable place for anchor inspection.

4.1.4.2	Chain Pipe

Location: one(1) under each gypsy wheel

| Material: | Pipe | Welded steel plate |
| --- | --- | --- |
|  | Cover | Galvanized steel plate |

Chafing ring of fabricated steel bar on deck and bell-mouth at lower edge shall be fitted for protection and smooth running of cables.

4.1.5	mooring fitting AND MOORING ROPE

4.1.5.1	Mooring Rope

Mooring ropes shall be equipped in accordance with the Rules requirements as follows:

| Item | No. | Standard |
| --- | --- | --- |
| Mooring rope | 11 | 64mm dia. x220m floating type (Polypropylene or equivalent material) (size subject to maker’s recommendation)with strength of as per Rules but min. 500kN |
| Towing rope(Buyer’s supply) | 1 | Steel wire, 260m with strength of min.1,471kN |

4.1.5.2	Mooring Fitting

The mooring fittings shall be arranged as symmetric as possible as shown on the G.A. plan.

Mooring fittings, such as bollard, fairleader, close chock, stand fairleader etc, to be arranged as shown on the G.A plan. ISO or GB standard type mooring fitting will be choosen by design stage.

Size, number and type of mooring fitting shall be finally determined according to the “Mooring fitting arrangement plan” during detail design stage based on the requirements of Rules.

Construction, material, dimensions etc. of the mooring fittings not specified in the Specification shall be in accordance with the Builder’s standard.

2-roller or 3-roller type fairleader to be provided. All roller type chocks to be provided with upper roller and/or retaining pipe, in order to avoid rope jumping.

All deck rollers to be removed / under surface cleaned and final coat applied prior delivery to avoid rust streaks running from dust trapped underneath during grinding in vicinity as far as possible.

All moving parts of the fittings shall have bronze bushes and fitted grease nipple. The

grease gun to be provided and the grease nipples to be of same size/type as far as possible.

Non-slip paint shall be provided around bollard and rope handling space for anti-slip.

All surfaces of the fittings in contact with the mooring rope shall be ground smoothly so that the mooring rope can be protected from wearing.

Seat of mooring fitting shall be of steel plate and strengthened under deck as per Rules.

Round bar or half round bar shall be fitted on the top of shell plate in way of mooring fitting to protect rope.

The safety working loads to be welded on the equipment.

4.2	STEERING SYSTEM

4.2.1	STEERING GEAR

One (1) set of electro-hydraulic ram type with 4 cylinders and 2 rams steering gear shall be installed in the steering gear room.

The steering gear shall be capable of turning the rudder from 35 deg. on one side to 30 deg. on the other side within 28 seconds with operation of one of the two (2) pumps (one (1) pump stand by), under the condition that the Vessel runs ahead at the summer load draught with maximum ahead service speed.

The operating angle of the steering gear shall be able to reach maximum 35 degree both sides, and the rudder stoppers shall be fitted on the steering gear to prevent the rudder from being hard over at about 37 degree both sides.

The steering gear shall consist of two (2) electro-hydraulic pump units, each with 100% capacity, and integrated hydraulic piping instruments. If one power unit fails, the remaining unit to be able to operate by change of the gear. One (1) of two power units can be operated at the capacity in accordance with relevant rule by emergency electric power supplied from emergency generator in case of the main electric power failure.

Each pump unit shall be equipped with a hydraulic oil pump, an IP44 type squirrel cage electric motor and a hydraulic oil tank with a low level alarm switch.

Each one pump unit in sea trial for testing to be output 100%.

The power unit shall be started/stopped from the wheelhouse and the steering gear room.

In case of main power failure, the power shall be automatically supplied from emergency generator to steering gear within 45 seconds in compliance with the Rules and Regulations.

A notice plate for emergency operation shall be provided in steering gear room and wheelhouse.

The steering gear shall be controlled by the steering control system (auto pilot) in the wheelhouse. In addition, the steering gear also shall be provided with a manual control device in the steering gear room for emergency operation in case of failure of control system.

The remote control and rudder angle indicator of the steering gear shall be provided in the wheelhouse and bridge wings, and the remote rudder angle indicator shall be provided in the engine control room.

The piping between the pumps and cylinders shall be seamless steel.

The hydraulic system of the steering gear shall be flushed at the manufacturer’s shop according to their standard, and then shall be properly protected during transportation and installation.

A fixed hydraulic oil storage tank with manual pump ready shall be provided in the steering gear room for recharge of the system as required by the Rule.

The mechanical type rudder angle indicator shall be fitted on the steering gear body. The rudder angle indicator shall be independent of the steering gear control system, and be readily visible from all control positions.

A gyro repeater shall be installed in the steering gear room readable from the manual control stand.

The construction, materials, accessories, etc., shall be in compliance with the requirements of the Classification Society and the manufacturer’s standards unless otherwise specified.

Lifting eyes shall be fitted underneath of upper deck for maintenance of the steering gear.

Working platform with handrail shall be provided around the steering gear for easy accessibility.

Non-slip paint (width : 1 m) shall be provided from the entrance of the steering gear room to the steering gear in compliance with the Rules and Regulations.

Steel coaming of about 100 mm high shall be provided around the steering gear.

Steel chock or resin chock shall be used for liner of steering gear foundation according to Maker’s recommendation.

4.2.2	RUDDER AND RUDDER STOCK AND OTHERS

Rudder to be of semi balance spade type. A suitable rudder bulb shall be fitted on the rudder.

The rudder area to be designed as per Class rules.

Propeller shall be capable of removal without dismantling the rudder ,rudder bulb.

The rudder plates and horizontal & vertical rudder frames to be of fabricated steel plate.

The rudder blade is made of high quality, sand blast shipbuilding steel and is pressure tested.

The rudder blade is constructed with a streamline cone coupling for the connection to the rudder stock.

The rudder blade has a protection against corrosion, which is inside solution and outside primer.

Rudder blade to be arranged with provision for hoisting eye.

Earthing device to be provided for rudder stock.

The jumping stopper shall be provided in way of the rudder top.

Rudder stock to be constructed of forged steel and to have keyless ways at the upper and lower ends to take steering gear and rudder respectively. Lower part of rudder stock to be tapered with screw, and connected with rudder by a nut. Rudder stock to be fitted with sleeve in way of lower bearing. Sleeves for rudder stock to be of stainless steel 316L material

Lifting hole, lifting eye plate, air and drain plug of the rudder to be provided. Peep hole or small manhole shall be provided to take rudder pintle and bush clearance in afloat condition.

4.2.3	BOWTHRUSTER

The bow thruster shall be provided as follows:

| Location | Bow |  |
| --- | --- | --- |
| No. of set | One (1) |  |
| Type | Tunnel type, CPP(Controllable Pitch Propeller) |  |
| No. of blade | Four (4) |  |
| Material of blade | Ni-Al-Bronze |  |
| Prime mover | Type | Electric motor, IP23, single speed, squirrel cage ,non-reversible |
|  | Voltage | AC 440V, 60Hz, 3 phase |
|  | Insulation | F class |
|  | Capacity | Approx. 1,000kW input power |
|  | Rating | Thirty (30) minutes at rated capacity |
| Starting method | As per Maker’s recommendation |  |
| Control | Start & stop | From the wheelhouse |
|  | Emergency stop | From bridge & wing console |
|  | Pitch control | From the bridge & wing console |

Bow thruster shall consist of propeller unit, electric motor control panel.

The electric motor shall be connected to the thruster unit through a coupling. The motor can be started/stopped from local starter and wheelhouse and can be stopped in emergency from both bridge wings. The running indication shall be provided in wheelhouse.

A suitable interlocking device shall be provided to prevent starting of driving motor unless vent fan is in operation.

Tunnel of bow thruster shall be fabricated by steel plate. A stainless steel (SUS 316) ring shall be installed inside the tunnel in way of the propeller blades for protection from cavitation.

The other material, accessories, spare parts and tools shall be provided in accordance with the Maker’s standard to comply with the Rule/Regulations.

Galvanized steel grid shall be provided at entrance of thruster tunnel according to Maker’s recommendation. The configuration and orientation of thruster tunnel grid shall be determined as per results of stream line paint test and Builder's standard.

Bow thruster room shall be ventilated by one (1) mechanical supply fan to remove the heat. The ventilation duct shall be arranged so that the air flow passes over the electric motor. Vent shall not be in a position where moist air could be drawn into the bow thruster motor space.

Electric heater for the bow thruster room shall be provided.

Bilge high level alarm is to be fitted in the Bow thruster Room.

Bow thruster starter/control panel to be placed outside the bow thruster motor level.

A easily accessible hatch with lifting arrangements to be provided for lifting the Bow Thruster motor from the location to outside the forecastle space to be provided.

4.3	CARGO HATCH COVER

The hatch cover to be provided as shown on the G.A. plan.

The hatch cover to be designed in accordance with the requirement of the Classification Society and to be equipped with necessary outfit and accessory.

The material, construction, designs, etc. of the hatch cover to be in accordance with the Maker’s standard subject to approval of the Classification Society.

4.3.1	CARGO HATCH

Cargo hatches shall be arranged as shown on the G.A. plan and opening size shall be as follows:

| Hatchway | Length (Approx. mm) | Width (Approx. mm) | No. of panels | Type | Tightness |
| --- | --- | --- | --- | --- | --- |
| No. 1 | 12,600 | 15,282 | 2 | Pontoon, Non-sequential | Weather- tight |
| No. 2 | 12,600 | 20,258 | 2 |  |  |
| No. 3 ~No.8 | 12,600 | 25,030 | 2 each |  |  |

4.3.2	CARGO HATCH cover

4.3.2.1	Strength and Structure

The hatch cover shall be of single plated, welded open type steel construction. The hatch covers shall be fabricated with approved mild steel and/or higher tensile steel according to the manufacturer's practice.

Cargo hatch covers shall be designed according to Class rules and MO32 of AMSA, and designed according as the followings:

- Uniform load according to the requirements of Class and ICLL.

- Design stack load

|  | 20ft | 40ft |
| --- | --- | --- |
| For hatch No.1 | 60 MT | 90 MT |
| For hatch No.2 ~No.8 | 90 MT | 120MT |
| Upper deck aft | 90 MT | 120MT |

The above stack load on hatch covers to be only as design basis of hatch cover strength, and the actual achievable stack load on hatch covers at specified positions shall be decided by lashing force calculations by the Maker.

For weather-tightness, hatch covers shall be fitted with rubber packing along the hatch periphery and double lips rubber seals or equivalent type for longitudinal panel joint in accordance with the manufacturer's standards. Water bar will be fitted on top plates along the longitudinal joints.

Maximum hoist weight of hatch cover panel (including fixed fittings but excluding all other container loose fittings) shall be about 35 MT.

Operation of panels to be non-sequence type operated by shore cranes.

The design condition of hatch cover operation shall be max. 2 degree of trim and 3 degree of heel.

4.3.2.2	Fittings and Others

Flexible pad type (replaceable) with laminated rubber bearing pad or equivalent shall be provided as per Maker's standard.

The hatch cover panel shall be secured to the hatch coaming top plate by manual anti-lifting devices according to Maker’s standard.

Transverse/longitudinal pin stopper at forward of hatch panel and transverse stopper at aft of hatch panel shall be provided which shall be also used to serve the entry guide of covers.

When being opened, the hatch cover to be stacked on the top of other hatch cover up to three (3) panels as far as practicable.

When stacking, the stacking foot underneath the hatch cover to be rested on stacking foundation on top plate of the other hatch cover.

When stacking, sufficient height considering the container loose fitting (bottom twist lock) to be provided between each stacked panels.

Lifting sockets(8 pcs per panel) shall be fitted near the center of gravity of each panel for hatch cover handling by 20ft and 40ft spreader and eye plate(only for 20ft) . A doubling plate shall be provided around the lifting sockets.

ISO raised sockets on the hatch cover may be used as hatch cover lifting sockets, if possible.

Holding down device and jumping stopper shall be provided for the hatch cover.

Necessary marks such as name of hatch, bay number, row number and panel weight shall be marked with intermittent welded line and painted on each hatch cover.

The necessary ventilation for holds around hatch covers to be installed, if required.

Hatch cover handling test shall be carried out with each hatch cover to demonstrate the performance of the hatch cover guiding system and the hatch cover stacking system. This test can be combined with the installation of the hatch cover during construction of the Vessel.

4.4     CONTAINER lashing and securing

The Vessel to be designed for carrying ISO standard container of the types stated in Section 2.2 of “General Part” of the Specification.

Lashing and securing system shall ensure safe carriage of containers of all variations of high and low cube containers as per the requirement of Rules and Regulations.

Cargo Safe Access Plan (CSAP) in accordance with MSC.1/Circ.1352and MSC.1/Circ. 1353 to be prepared by the Builder.

All container loose fittings and Container Securing Manual (CSM) required by IMO shall be provided by the Buyer.

The loose twist stackers supplied by the Buyer for mixed stowage shall be stowed in the

dedicated storage bins and racks supplied by the Buyer on upper deck

4.4.1	CONTAINER arrangement

4.4.1.1	Container Fittings in Hold

The container stowage and securing method in hold shall meet the requirement of OSHA.

Twin 20ft containers shall be loaded in the 40ft cell up to maximum five (5) tiers from tank top.

Fixed stacking cones shall be welded on the tank top with doubling plate at each corner of 40ft container, and guide fitting at 20ft ISO gap for supporting the containers against the shifting.

Regardless of the maximum designed stack weight for tank top strength, the stack weight and the weight distribution will follow the calculation results of Rules.

The transverse clearance between two adjacent rows container in hold to be about 50 mm, in general.

4.4.1.2	Container Fittings on Deck and Hatch Cover

The container stowage on deck and hatch cover shall meet the requirement of CSS Code including MSC.1/Circ.1352.

Arrangement and number of containers on upper deck, hatch cover and container deck stanchion to be as shown on the G.A. plan.

45ft container (45’ x 8’ x 9’6”) could be stowed on hatch covers from 2ndtier above No.2~No.8 hatch without additional lashing as outlined on G.A. plan.

Container on hatch cover to be arranged as symmetrical as possible in the section view and the transverse clearance between containers to be about 38mmin general.

In general, the containers on hatch covers and upper deck shall be secured by the combined use of twist locks, lashing rods, turnbuckles, and lashing eyes.

Regardless of the maximum designed stack weight for the hatch covers and upper deck aft, the maximum achievable stack weight of containers, especially for the outmost row(s) containers, may be adjusted in accordance with the lashing calculation result as per Class Rule based on the following lashing patterns or lashing patterns by Maker’s recommendation.

Lashing pattern:

Internal lashing pattern shall be applied, in general.

|  | For 40ft /45ft containers on hatch cover and upper deck | For 20ft containers |  |
| --- | --- | --- | --- |
| Inboard container stacks | Double cross lashing (short) | Gap | single short cross |
|  |  | End | single short cross |
| Outboard container stacks | Double cross lashing (short) + Single diagonal lashing(long) | Gap | Single short cross+long wind lashing+long vertical lashing |
|  |  | End | double lashing |

Container lashing method shall be in accordance with the Maker’s standard to comply with the Rules requirement and shall be submitted to the Buyer for reference.

The permissible weight distribution of containers in each tier to be decided in accordance with the Class Rules.

The container fixed fitting to be arranged on hatch cover, deck stanchion and upper deck based on the above lashing patterns.

ISO raised type twist lock foundation (about 110 mm high) and fixed type lashing eye plate shall be fitted on hatch cover, deck stanchion and upper deck for container stowage. Eye plate shall be sufficiently spaced to allow for easy inserting /removal of lashing bar ends.

Elongated-hole type twist lock sockets shall be fitted on outmost side of hatch cover or on deck stanchion considering hatch cover movement as per Maker’s recommendation.

All loose lashing and securing fittings such as semi-automatic twist locks, lashing rods, turnbuckles etc., and Container Securing Manual (CSM) required by IMO shall be supplied by the Buyer.

4.4.2	Lashing bridge and reefer maintenance platform

No lashing bridge and reefer maintenance platforms to be provided on deck.

Two (2) sets of aluminum portable ladder to be provided for reefer container.

4.4.3	container DECK STANCHION and operation platform on weather deck

Containers stowed on the outermost sides shall be supported by hatch covers and container deck stanchions as shown on the G.A. plan.

Container deck stanchion shall be of fabricated box type steel construction, and shall be designed based on the same stack load as the hatch covers.

Vertical ladder or hand grips and steps to be provided for access from weather deck to top of the deck stanchion.

Fixed handrail to be provided where necessary.

The deck stanchion shall be designed to ensure clear height of passage way from upper deck to bottom of first tier container on hatch cover to be min. 2,000 mm. Non-skid paint to be applied along the walkway with skirting of contrast color.

Transverse working platform shall be provided between cargo hold: Width of clear passageway on working platform: 600mmin general. Height to beat the same level as the top of hatch covers.

Removable handrails and galvanized steel rope with vinyl coating on both side of working platform to be provided,

Access to working platform shall be provided through a vertical ladder from upper deck.

Reefer container socket to be fitted on the hatch coaming and weather protected (IP56)

4.4.4	Cellular Construction

Cellular construction shall be of fixed, continuous type and designed to guide and support the six(6) tiers of 8’6”standard containers.

The cell guide shall be reinforced to permit the stowage of 9’6” high cube container randomly in cargo hold in flexi loading condition.

The cell guides shall be of about 160 mm x 160 mm x 14 mm equal angle construction. The cell guides shall extend continuously from the entry guide down to around 2.0 meters above the inner bottom or container platforms in the cargo hold.

The top of each cell guide shall be connected to a fixed entry guide fabricated with 22 mm thickness steel plate, and the enter guides shall be of wave type in accordance with the Builder’s practice.

The cell guides and cell guide support structure shall be designed based on the container unit weight of 30.5MT per 20ft/40ft container. From entry guide within 2 meters the support bracket shall be double pieces to be installed (to be confirm at approved drawing).

Side guide plates shall be provided in way of location between each 20ft container as follows:

For guide plate in way of side longitudinal bulkhead of all cargo hold:

(1) Approx. from hatch coaming top level to upper deck level

(2) Approx. 1.5m length above the tank top 1000m.

The side guide rails would be not fitted on other step bulkhead.

The test for container loading and unloading with real or dummy container shall be carried out in accordance with the Builder’s practice.

The bay numbers based on 40ft containers and row numbers shall be marked with Intermittent welding line and painted on the side/end wall of each cargo hold.

4.4.5	Others

Painted steel boxes or hangers for loose container fittings storage to be provided by the Buyer.

Turnbuckle shall always be secured by lashing eyes on hatch covers and deck stanchions if necessary and in accordance with the Maker’s standard.

Lashing bar storage provided by builder, other storage boxed are provided by Buyer.

4.5	  CARGO GEAR

4.5.1	CARGO gear

No deck crane to be fitted.

4.6	LIFE SAVING APPLIANCE

The Vessel shall be provided with lifesaving and protection equipment for a complement of 24 persons and to comply with the requirement of the Rule/Regulations.

Effective term of ten (10) months shall be available for all LSA and FFA certificates at the time of the Vessel’s delivery.

4.6.1	LIFEBOAT WITH EQUIPMENT

The lifeboat shall be provided as follows:

| No. of set | Two (2) |
| --- | --- |
| Type | Totally enclosed Starboard side lifeboat shall be used as rescue boat. |
| Material | Glass reinforced plastics(GRP) |
| Capacity | 24persons |
| Propulsion | Diesel engine (sea water cooling) |
| Location | Beside deck house as shown on G.A. plan |

Dimension, capacity and certified date shall be marked at position approved by the Regulation.

Name of ship, Call sign, complement and port of registry shall be painted.

The boat embarkation area to be protected and illuminated sufficiently.

Emergency lighting and instruction sheet (in English) shall be fitted on wall near the lifeboat.

Construction, material, and accessories shall be in accordance with the approved manufacturer’s standard to comply with the Rule/Regulations.

4.6.2	Lifeboat davit and winch

One (1) set of hinged gravity type davit to be provided at port and starboard side as shown on G.A. plan.

The davit to be made of fabricated steel and to be complete with necessary blocks, tackles and galvanized steel wire rope, etc.

One (1) set of electric motor driven winch shall be fitted for each lifeboat davit. Davit shall be capable of manual operation by handle in case of emergency. The winch to have sufficient capacity for lowering fully equipped life boat with persons on board to the ballast draft. One independent starter/control panels to be used for each life boat davit and winch.

Each winch shall have two (2) limit switches to avoid over-hoisting.

Guide fittings (skate) for easily lowering the lifeboat shall be fitted. Lighting and instruction sheet (in English) shall be fitted on the wall near the lifeboat.

Other material, fittings, and construction not specified above shall be in accordance with the manufacturer’s standard in compliance with the Rule/Regulations.

The speed of starboard side winch of hoisting speed to be in accordance with the requirements for rescue boat. The designated rescue boat davit shall be capable for the lifeboat recovery within 5 minutes as per SOLAS requirements.

4.6.3	LIFE RAFT AND DAVIT

4.6.3.1	Life Raft

Life rafts shall be provided as follow:

| No. | Capacity (persons) | Type | Position |
| --- | --- | --- | --- |
| 1 | 24 | Inflatable, throw overboard type with a hydrostatic releasing device | Accommodation port side |
| 1 | 24 | Inflatable, throw overboard type with a hydrostatic releasing device | Accommodation starboard side |
| 2 | 6 | Inflatable, throw overboard type | Ship’s forward |

Each one (1) embarkation ladder shall be supplied for liferaft at accommodation port side and starboard side.

Two (2) embarkation ladders shall be arranged at the protected location on forward ship.

The canvas covers for these ladders to be provided.

The inflatable life rafts shall be stowed in fiber glass reinforced plastic (FRP) container. The container shall be stowed in cradle and float free,

Effective term of nine (9) months shall be available for the life rafts at the time of the Vessel’s delivery.

Indication lamp and instruction sheet (in English) shall be provided for life raft.

4.6.3.2	Life Raft Davit

Life raft davit shall not be provided.

4.6.4	PERSONNEL LIFESAVINGS

Personnel lifesaving appliances shall be provided by the Builder in accordance with the Rule/Regulations of Classification Society and Flag Authority.

Owner supply the SCBA and EEBD recharging compressor. Ship yard supply socket for the compressor,

4.6.4.1	Life Jackets and Immersion Suits

Life jackets and immersion suits shall be provided for 24persons and stowed in each cabin in accordance with the Regulations.

Each life jacket shall be colored orange and marked with the name of the vessel and port of registry.

For other spaces, additional life jackets and immersion suits shall be provided and stowed, according to requirement of Classification Society and Flag Authority.

4.6.4.2	Lifebuoy

Lifebuoy shall be provided as per rules, general will be as follow:

| Location | No. | Accessory |
| --- | --- | --- |
| Vessel’s side (port and starboard) and stern rail | as per rule | Buoyant lifeline of about 30 m |
| Pilot boarding area | 2 | Self-igniting light |
| Near lifeboat and rescue boat | 2 | Self-igniting light |
| Near accommodation ladder | 2 | Self-igniting light and buoyant life line of about 30m |
| Navigation bridge deck (port and starboard) | 2 | Self-igniting light and self-activating smoke signal with quick releasing device |

The self-igniting light shall be kept near the lifebuoy to which it belongs.

The self-activating smoke signal shall be capable of producing smoke of highly visible color for at least 15 minutes.

Each of the lifebuoy shall be marked with the name of the Vessel and port of registry in black letters.

Lifebuoy shall be made of rigid polyurethane with plastic cover and fitted with grab line.

4.6.4.3	Lifeline Throwing Appliance

One (1) lifeline throwing appliance consisting of four (4) projectiles with four (4) lifelines shall be provided.

Each projectile shall be capable of carrying the line at least 230 m in calm weather.

4.6.4.4	Distress Flares

Twelve (12) sets of rocket parachute flares shall be provided.

4.7      deck outFITTING

4.7.1    ship’s service handling gear

4.7.1.1	Provision Crane

| No | One (1) set |
| --- | --- |
| Type | Electric driven lifting and manual slewing |
| Location | B deck PS side as per GA plan |
| Destination | Handling spare parts of engine room, Suez boat etc. |
| Lifting capacity | 4.0 t |
| Max outreach | abt. 4 m |
| No | One (1) set |
| Type | Electric driven lifting, manual slewing |
| Location | B deck SB side as per GA plan |
| Lifting capacity | 4.0 t |
| Max outreach | abt. 4 m |

4.7.1.2	Suez Canal searchlight davit

One (1) manual steel jib type weighted 0.25t to be place on forecastle for hoisting Suez Canal searchlight and articles in bosun store.

4.7.1.3	Hose Davit

Two (2) sets of SWL 0.5 t, electric motor driven hoisting, manual slewing type bunker hose handling davit shall be provided on port and starboard as per G.A. plan.

4.7.1.4	CO2 bottles handling Davit

One (1) set of SWL 0.2t, manual hoisting and slewing type davit for handling CO2 bottles.

4.7.2	ACCOMMODATION LADDER AND PILOT Rope LADDER

4.7.2.1	Accommodation ladder

| No. of set | Two (2) |
| --- | --- |
| Location | Accommodation sides on upper deck |
| Type | One (1) man operated self-stowing, vertical stowing, one (1) piece type |
| Step | Fixed non-slip curved step |
| Material | Al-alloy for frame, step, rail and stanchion Galvanized steel for upper platform and turn table |
| Clear breadth | Approx. 600mm |
| Davit | One (1) davit and one (1) winch with galvanized steel wire rope rigging for each accommodation ladder |
| Motor | One (1) set of fixed electric motor for each accommodation ladder |
| Accessories | Rail and stanchions of hinged type, safety net, lower/upper platform and protection rollers |
| Control | From the control stand on the upper deck near each accommodation ladder |

Ladder shall have length to reach not more than 600 mm above the light ballast water line at an inclination of 55 deg. to horizontal.

Ladder shall be stowed vertically within the Vessel’s side line on upper deck.

The SWL shall be marked with paint. Must be marked with the maximum load the weights and number of persons on the ladder at the entrance and exit.

Secondary support for accommodation ladder shall be provided.

4.7.2.2	Pilot Rope Ladder

Two (2) sets of pilot rope ladder with wooden steps shall be provided. Pilot rope ladder to be confirmed that the ISO or Class certificate is attached

4.7.3	FORE MAST, RADAR MAST, FORE AND STERN FLAG STAFF

Fore mast to be of fabricated steel construction, and to be arranged on the forecastle deck at the center line of the Vessel.

The fore mast to be furnished with vertical ladders, back hoop, mast light. Necessary fittings and eye plates for steel wire rope stays to be fitted to minimize vibration.

Radar mast to be of fabricated steel and to be arranged on the compass deck at the center line of the Vessel.

The radar mast shall be connected with under deck structure effectively and fastened by rigging and tackles to minimize vibration.

The radar mast to be furnished with vertical ladder, back hook, radar scanner, antenna fittings, mast light, signal yard, signal lamp. Necessary fittings and eye plates for steel wire rope stays to be fitted to minimize vibration.

Antenna post to be of steel pipe construction of self -standing type.

Necessary rigging and tackles for davits, flag yards, signal yard, etc. to be furnished completely.

All riggings to be greased before delivery and covered with vinyl cover.

One (1) ensign staff to be fitted at stern and one (1) jack staff at bow. And anchor lamp to be provided. Rigging to be fitted for flag staff completely.

One (1) flag box to be place around the fore and stern flag staff.

Cables installed on mast or post on upper deck to be wired in galvanized steel pipe up to about 300mm height from the deck.

4.7.4	CANVAS COVER

Canvas cover shall be provided for each of the following equipment on weather deck:

- Chain pipe

- Winch drum of life boat davit

- Winch drum of accommodation ladder and pilot assistant ladder

- Pilot rope ladder

- Embarkation rope ladder for liferaft

- Magnetic compass and gyro repeater

- Search light

- Exposed flood light projector

4.7.5	NAME PLATE AND MARKS

Name plate and Marks shall be provided as follows and shall be distinguished by color paint.

| Item | Location | Remarks |
| --- | --- | --- |
| Name of Vessel | Bow (P&S) | Intermittent welding line and painted |
|  | Transom stern |  |
|  | Lifeboat, liferafts, lifebuoys | Painted |
| Port of registry | Transom stern | Intermittent welding line and painted |
| Vessel's name board | Wheelhouse top (P&S) | steel plate and painting |
| Draught mark | Bow, midship and stern (P&S) | 6.0 mm steel plate welded and painted, Arabic numerals of 100 mm projected height and 200 mm pitch |
| Freeboard mark | Midship (P&S) | 6.0 mm steel plate welded and painted |
| Notice boards, caution plates | Suitable location | Brass, stainless steel or plastic plate as per Builder’s practice |
| Name plates to identify quarters and spaces | Cabins, public rooms, stores, lockers |  |
| Vessel's call letter | Wheelhouse |  |
| Funnel mark Funnel in Owner's colors and Owner's flag at both side of the accommodation. | Around of funnel | Intermittent welding line and painted as per Owner’s drawing |
| Bow thruster mark | Bow (P&S) | Intermittent welding line and painted |
| Bulkhead mark (tug pushing mark shall be used as bulkhead mark) | Side shell (P&S) | Intermittent welding line and painted |
| Cargo hold No. | On hatch cover |  |
| Hatch and manhole cover | Suitable location | 6.0 mm steel plate welded and painted |
| Bay no. and row no. | In hold/deck/deck stanchion | 6.0 mm steel plate welded and painted |
| Indication of bottom plug | Adjacent to bottom plug | 6.0 mm steel plate welded and painted |
| Pilot boarding | Suitable location | Intermittent Welding line and painted |
| Official number | Suitable location | Intermittent Welding line and painted |
| “CC” mark | Suitable location | Welding line and painted |
| “NO SMOKING” mark | Longitudinal bulkhead in dangerous cargo loading hold | Painted |
| Tank boundary with initial letter if rule require | Side shell | 6.0 mm steel plate welded and painted |
| “In water” survey mark | Suitable location | welding line and painted |
| The Builder’s name plate | Deckhouse’s entrance | Bronze or artificial marble |
| Company logo | Side shell | Intermittent Welding line and painted |
| IMO number | Stern and E/R forward bulkhead | Intermittent welding line and painted |
| Frame number | In under deck passageway | 6.0 mm steel plate welded and painted |

Note:

If bottom plug mark very close to tank boundary name mark,plug name mark will be omitted.

Pipe outlet mark shall be satisfied with the Rule’s requirement for under water inspection.

Tank name mark welded near the Manhole cover by 6.0 mm steel plate welded and painted. All shell openings under water including inside sea chests shall be clearly and suitably marked with welding bead and painting

Design and Type of small name plates for identification of valves, pipes etc shall be

provided during plan approval drawing stage.

4.7.6	WORK RAFT

Nil

4.7.7	GARBAGE store

Seven (7) metallic garbage bins with cover shall be provided in garbage store and faucet for washing shall be fitted around with louver ventilation and mechanical exhaust fan.

4.7.8	Pipe tunnel trolley

A manual operated small trolley for carrying maintenance parts with rail shall be fitted in pipe tunnel.

4.8	outfitting for access

4.8.1	access HATCH, MANHOLEcover and steel door

4.8.1.1	Access Hatch Cover, Rope Hatch Cover, etc.

Hatches for access, etc. shall be provided to the space listed in the table hereinafter. Nuts, bolts for hatches on weather deck shall be of stainless steel SUS304.

The coaming height shall be complied with ICLL requirement and all weather-tight doors and hatches shall be tested according to class approved method..

All hatches except those specially indicated shall be of hinged type.

Number and size of hatches for each compartment to be as follows:

| Served Space | Number | Size (mm) |
| --- | --- | --- |
| Rope store | 1 | Ф 600 |
| Bosun store | 1 | 1,200 x 1,200 (sq.) |
| Cargo hold | 2 | 630 x 830 |
| Engine room | 1 1 | 1,600 x 2,000 (bolted flush type) |
| Escape access from engine room | 1 | 800 x 800 (sq.) |
| Access to bow thruster room | 1 | 630 x 830 |
| Bow thruster room (for handling motor etc.) | 1 | As per detail design (bolted flush type) |
| CO2 room | 1 | 1,000 x 1,000 (sq.) |
| Pipe duct access | 2 | 600 x 600 (sq.) |
| Steering gear room | 1 | 1000 x 1000 (sq.) |

Note:

- Hatch for rope handling shall have protector of steel round bar for running of rope where sharp edge exists at coaming or deck beam etc.

- All hatches shall have eye plates for pad lock (except escape hole).

- The manually operated hinged type hatch of 1.2 m x1.2 m size and above shall be fitted with counter balance or equivalent for easy handling.

4.8.1.2	Manhole Cover and Other Bolted Cover

Oil tight or water tight oval manhole cover to be provided for double bottom tank, side tanks, cofferdams.

The manhole cover to be of steel and watertight or oil tight construction with synthetic rubber sheet gasket, bolts and nuts. Studs and nuts to be stainless steelSUS304, but those on exposed deck to be SUS316.The manhole cover to be of flush type on container stowage position in cargo hold and on under deck passageway floor. However on the tank top in engine room and oil tank, steel coaming of about 100 mm in height to be fitted.

In general, at least two (2) manhole covers shall be provided for each big tank with capacity over 20m3.

The manhole cover on wall to be fitted with handle.

The marks to be of welding line on the manhole cover, no additional mark on tank bulkhead. Steps and handles to be provided on vertical fitted manhole covers if the distance from lower edge of manhole to deck is more than 600 mm.

Number and size of manhole cover and bolted cover for each space to be as follows:

| Served Space | Size (mm) |
| --- | --- |
| Oil tank | 400 x 600 (oval) |
| Fresh water tank | 400 x 600 (oval) |
| Fore peak tank | 400 x 600 (oval) |
| Aft peak void | 400 x 600 (oval) |
| Chain locker (on chain locker wall in bosun store) | 600 x 800 (hinged) |
| Ballast water tank | 400 x 600 (oval) |
| Other hull structure tanks in E/R | 400 x 600 (oval) |
| Cofferdam and void space | 400 x 600 (oval) |
| Pipe duct | 400 x 600 (oval) |
| Stern tube cooling water tank | 400 x 600 (oval) |

4.8.2	steel ladder and HAND RAIL

4.8.2.1	Access Ladder on Deck, in Cargo Hold / Tank etc.

Access ladders shall be provided as follows:

| Served space | Type | Breadth (mm) | Side stringer | Tread | Handrail |
| --- | --- | --- | --- | --- | --- |
| Exposed upper deck | Inclined | 700 | Section | Checkered steel plate pitch:abt.220mm | Top rail 42.25mm diameter galvanized water gas pipe. |
| Other exposed deck |  |  |  |  |  |
| Steering gear room, CO2 room | Inclined | 600 | Section | Checkered steel plate pitch:abt.220mm | 42.25mm diameter galvanized water gas pipe |
| Cargo hold | Vertical | 350 | Steel flat bar | Steel square bar, 22x22mm | -- |
| Fuel oil bunker tank, Ballast water tank, F.W. tank, E/R escape, funnel, bow thruster room, access trunk, deck stanchion | Vert. | 350 | Steel flat bar | Steel square bar 22mm, pitch: abt.300mm | Round bar, 25mm dia. where necessary |
| Mast, post, miscellaneous part | Vert. | 300 | Steel flat bar | Steel square bar 22mm pitch: abt.300mm | Round bar, 25mm dia. where necessary |

In general, ladder breadth shall be 350 mm and in no case not less than 300mm, middle support to be used for ladder length more than 2.5m.Ladder safety hoop will be install  according to AMSA rules.

Non-slip paint to be taken at landing area from exposed deck to ladder.

Hand grips and steps to be provided, where necessary.

Ladder and handrail in ballast water tank and exposed to weather to be galvanized and to be painted as surrounding area.

Elevation angle of inclined ladder to be 55 deg. (max. 60 deg. In E/R and max. 65 deg. In cargo hold) to horizontal. But the elevation angle of inclined ladder used as means of escape to be not greater than 50 deg. To horizontal. Hand rail of inclined ladder to be fitted on each side.

Mid Rail should be provided for incline ladders and the rail if the distance of the top railing from the step base is equal to or more than 500 mm in height.

All ladders and railing stanchions when welded to decks should be done with a doubler plate.

4.8.2.2	Hand Rail on Exposed Deck

The maximum distance of the rail stanchion, hand rail shall comply with Rules.

The height from hand rail top to deck shall be abt. 1,050mm, in general.

Handrails of Ø 42.25x 3.25 mm outer diameter top pipe (galvanized water gas pipe) and  0ne(1) Ø 20 mm steel round bars lower rails shall be fitted for exposed decks. Stanchion shall be 60 mm x16 mm flat bar with stay. Stanchion for masts, posts and platforms etc. on exposed deck shall be 60 mm x 16 mm flat bar without stay.

Stanchions shall be be welded with pad and welded to the curtain plate or deck and shall be well withstood to ensure rigidity of rail.

The stanchion shall be provided at about 1,500 mm intervals in general and the stay at every third stanchion, if fitted.

Non-magnetic hand rail shall be used in way of the compass on wheelhouse top shall be as per Maker's requirement.

Gap between handrail and bulkhead/fittings shall be minimum 60 mm.

4.8.2.3	Hand Rail in Cargo Hold

The maximum distance of the rail stanchion, hand rail shall comply with Rules.

The height from hand rail top to deck shall be abt. 1,050 mm, in general, mid rails will be provided.

Handrails of Ø 42.25x 3.25mm outer diameter top pipe (galvanized water gas pipe) and 0ne(1) Ø 20mm steel round bars lower rails shall be fitted for stringer platform in cargo hold. Stanchion shall be 60 mm x16 mm flat bar without stay.

4.8.2.4	Storm Rail

Galvanized storm rail of 42.25mm outside diameter steel pipe (galvanized water gas pipe) to be fitted on outside wall of deckhouse where persons frequently walk around and to be positioned at about 1,000 mm above deck with adequate distance from the wall.

Removable life lines and stanchions should be provided between accommodation front and forecastle deck as Rule's requirement where the width of clear passageway is more than 1.2m.

4.9     SPARE PART AND DECK INVENTORY

Spare parts, tools and inventory shall be furnished as per the Maker's and/or Builder's practice in accordance with the Rules recommendation of the Classification Society unless otherwise specifically described in this Specification.

4.10	NAVIGATION EQUIPMENT

4.10.1	NAVIGATION EQUIPMENT

Navigation equipment shall be fitted as follow:

2 -   Binocular, prism (marine type, 50mm x 7 times)

1 -   Aneroid barometer millibar scale

1 -   Thermometer for sea water

2 -   Thermometer for atmosphere (dry and wet)

1 -   Hygrometer

1 -   Hand lead, 3.2kg weight with 55 meters line

1 -   Deep sea lead, 12.8kg weight 220 meters line with reel

1 -   Chronometer, portable type (battery use)

1 -   Sextant

2 -   Clinometer (small clock type in wheelhouse and E/R)

1 -   Clinometer (in ship office)

Hygrometer and Barometer should have calibration certificates.

Electric navigation instrument such as gyro compass, radar, echo sounder, velocity-meter, clock, etc. shall refer to "Part IX –Electric Part".

4.10.2	SOUND SIGNAL AND SHAPE EQUIPMENT

| Item | Number |
| --- | --- |
| Electric whistle on fore mast | 1 |
| Air horn on radar mast | 1 |
| Big signal bell, | As per Rules |
| Signal gong (460 mm) | As per Rules |
| 610mm black ball | 3 |
| Ship's bell (300 mm dia. on forecastle deck) | 1 |
| Black diamond shape (610 mm dia. x 1,200 mm folding type) | 1 |
| Black cylindrical shape (610 mm dia. x 1,200 mm folding type) | 2 |

4.10.3	FLAG

Code flags shall be provided as follow:

1 - International signal flag

2 - National flag (Buyer’s supply)

1 - Ship's name code flags (Buyer’s supply)

1 - International signal code book (Buyer’s supply)

4.10.4	VOICE TUBE

Nil

CHAPTER 5

HULL PIPING

Version:A

Key words: Breadth 27.5m, Gearless, 6S60ME-C10.5 Tier II, 220ReeferPlugs, Open loop scrubber
CONTENT

CHAPTER 5	HULL PIPING	3

5.1	HULL PIPING	3

5.1.1	GENERAL	3

5.1.2	PIPING MATERIAL AND WORK	3

5.1.3	BILGE PIPING	7

5.1.4	BALLAST PIPING	8

5.1.5	ANTI-HEELING PIPing	9

5.1.6	VALVE REMOTE CONTROL SYSTEM	10

5.1.7	AIR AND SOUNDING PIPES	10

5.1.8	FRESH WATER SERVICE SYSTEM	12

5.1.9	SCUPPER AND DRAINAGE SYSTEM	14

5.1.10	COMPRESSED AIR PIPE FOR DECK USE	15

5.1.11	FUEL OIL PIPING	15

5.1.12	FUEL OIL TANK HEATING SYSTEM	16

5.1.13	DRAFT GAUGE	16

5.1.14	SCHEDULE OF HULL PIPE	17

5.1.15	SCHEDULE OF VALVES	19

5.1.16	SCHEDULE OF PIPE INSULATION	20

5.2	FIRE FIGHTING SYSTEM	21

5.2.1	GENERAL	21

5.2.2	SEA WATER FIRE FIGHTING SYSTEM	21

5.2.3	CO2 FIRE EXTINGUISHING SYSTEM	23

5.2.4	PORTABLE FIRE EXTINGUISHER	24

5.2.5	FIREMAN OUTFIT	24

5.2.6	FIRE DETECTION AND ALARM SYSTEM	24

5.2.7	FIRE control STATION	25

5.2.8	ESCAPE TRUCK FOR ENGINE ROOM	25

5.2.9	MISCELLANEOUS	25

5.3	AIR CONDITIONING AND VENTILATION SYSTEM	26

5.3.1	AIR CONDITIONING SYSTEM	26

5.3.2	VENTILATION FOR CARGO HOLDS	29

5.3.3	VENTILATION FOR ACCOMMODATION AND OTHER SPACES	30

CHAPTER 5	HULL PIPING

5.1	HULL PIPING

5.1.1	GENERAL

Description of this section shall generally cover piping system located outside the engine room, unless otherwise specified.

Materials and dimensions of pipes, joints, valves including bolts, nuts, etc. shall be manufactured in accordance with the Chinese Industrial Standard, and/or the Builder’s Standard except otherwise specified hereinafter, the diameter of connecting flange and distance between flanges of valves shall be in accordance with ISO standard.

The other recognized industrial standards than mentioned above may be applied to meet the requirements of the Classification Society.

Valves, pipes and fittings mounted on its equipment shall be in accordance with the manufacturer’s standard.

All overboard discharges shall be kept clear of pilot ladder, accommodation ladder, draft marking, load line marking and lifeboat launching area as far as practical.

Piping shall be designed to allow for stress due to thermal expansion and deflection of the Vessel’s structure.

No ballast pipes or vents for ballast tanks shall pass through oil tanks or vice versa, in general.

5.1.2	PIPING MATERIAL AND WORK

5.1.2.1	Pipe

The materials and wall thickness of pipes shall be in accordance with “Table of pipes” of this section and to meet the requirements of the Classification Society.

The pipes as a part of machinery or equipment supplied by the manufacturer in package shall be of the manufacturer’s standard.

Pipe bore shall be decided so as to accommodate given flow quantity and corresponding pump pressure.

Pipe lines shall be led with a minimum number of bends and the arrangements shall avoid well part where drains likely to stay as far as practical.

Suitable draining shall be provided for the well parts of water pipes in exposed locations where those may be frozen.

Liquid pipes shall be kept away from switchboard and the electrical appliances. Where the piping is unavoidably led above the electrical appliances (other than MSB, G/Es, transformers etc.), the welded sleeve joint shall be provided.

Where the pipe lines pass through girders or any other structural elements, the compensation shall be provided in accordance with the requirements of the Classification Society and Builder’s standard if necessary

The bend radius of pipe shall be not less than 2.5 times outside diameter, except elbow.

For flushing of the pipe, refer to the “Chapter 6  Machinery” of the Specification.

5.1.2.2	Valves

In general, the valve side shall be of the same nominal bore as that of the pipes connected except automatic control valves.

In general, for DN80mm and above. butterfly valves will be used as far as possible. Such as high pressure and high temperature system to be excepted. Butterfly valves to be wafer type with NBR/ Viton liner and bronze/stainless steel disc except shipside valves.Shipside butterfly valves to be double flange type. Some of wafer type may be changed into lug type, it will discuss in detail design stage between the Owner and yard.

Use of gate valves shall be limited where require by Classification Society for the purpose of fire protecting closure.

Cast steel and ductile cast iron valves shall be fitted to such locations as required by the Classification Society.

Installation of the ship side valve shall be in accordance with the requirements of the Classification Society and shall be fitted rigidly to the shell with short stub piece of heavy wall (as per Rule requirement) as far as practicable.

Generally the actuators of remote butterfly valves shall be of enclosed double acting type, single acting type to be used according to the requirement of class and shall be directly coupled to the valves.

Remote control valves shall be provided with local manual operation device required by Classification Society.

Valve name plates of brass or stainless steel shall be fitted to the valves on handle or other suitable place.

5.1.2.3	Pipe Jointing

In general, the welded sleeve joints, butt welding and flange connection shall be applied to steel pipe, joints, fittings, machinery and where necessary according to the Builder’s practice.

Pipe joint of ballast system and bilge system shall be of flange type as far as possible. The material of pipe connection shall be of the same material or similar to that of the pipes connected, unless otherwise specified.

In general, flange joint shall be of steel fillet weld-on type for steel piping

Preformed welding socket shall be used for high pressure steel pipe such as hydraulic oil line, etc.

Butt welding joint shall be applied for reducer, elbow, tee, and others except otherwise specified.

Galvanized steel hexagonal head bolts and nuts shall be used for piping in ballast water tank, weather deck, etc.

Metric threaded and/or welded type unions may be used at dismantling point of small bore steel pipe or non-ferrous tubes.

For copper or aluminum brass piping, braze union or bite joint or sleeve shall be applied for small bore pipes and screw union for large bore pipes.

U bends, where not practical the joint must be inspected prior fitting the sliding coupling by owner’s site team, such as ballast water pipe and bilge pipes in pipe duct, shall be generally used for ballast, bilge, fire main, cable protection lines, etc., where necessary, to compensate for the pipes expansion and contraction.

Offset expansion bends (Loop) shall be applied to compressed air line, steam line, high pressure hydraulic oil lines and wherever practicable.

5.1.2.4	Bulkhead and Deck Penetrations

Where the pipe lines pass through the watertight or oil tight bulkheads and decks, the bulkhead or deck shall be sufficiently compensated, where necessary, in accordance with the requirements of the Classification Society.

The standard penetration pieces shall be complied with class requirement.

The connections through the insulated bulkheads shall be of suitable length to permit access to flange connections without disturbing the insulations.

Full penetration at the side shell welding and inspection shall be performed according to rule and standards.

5.1.2.5	Tees, Bends, and Other Fittings Etc.

Branch pipes for low pressure piping up to 5MPa shall be welded directly to the main pipe.

For high pressure pipes of DN40 mm and below, socket welding type tee shall be used. Standard elbow pieces and reducers shall be of fabricated steel made by electric arc welding.

Fittings forming a part of pipe shall be of the equivalent material as jointing pipes in general.

As a standard for steel and non-ferrous pipes, pipe bending shall be carried out by means of cold bending machine, to maximum practicable according to the Builder’s facility.

If cold bending is not applicable or small bending radius is necessary to facilitate the piping arrangement, high frequency induction bending or butt-welding elbow shall be used.

The loose fittings (Filters, Strainers, Traps, Rose Box, Mud Box, Rose Plates, Plugs, Nozzles, Orifice, expansion couplings, separators, air pipe heads, etc.) shall be selected by pipe size, line pressure and pipe line material according to Builder’s practice.

5.1.2.6	Suction Bell Mouth

Inverted hopper shape suction bell mouth fabricated of mild steel shall be used in general,.

Double plate shall be installed below bell mouths (10mm).

The clearance between suction bell mouth and tank bottom shall be Builder's standard.

5.1.2.7	Gasket and Packing

Universal heat and oil resistant asbestos free gaskets shall be used for all piping system.

The type of gasket shall be ring type, and the thickness of gasket used shall be according to Builder’s practice.

Material of packing for valve stem shall be of valve maker’s standard.

5.1.2.8	Pipe Supports

The piping shall be securely supported by steel angle with clamping methods to avoid damage, vibrations and movement in accordance with the Builder’s practice.

PE/Copper Pipe Makers standard to be followed in systems where the material is utilized.

Pipe supports for non-ferrous pipe shall be lined with sliding pad of copper, brass, plastic or synthetic rubber so as to not to directly contacted with pipes.

All pipe supports and on deck as well as inside water tanks to be of hot-dip galvanized material.

Fixing supports shall be provided for preventing floated pipe from excessive free moving, where required.

For clamping, “U” shape steel round bar bolt clips shall be used for steel pipes (DN≥15mm) and Maker’s standard to be applied for the others. All U clamps on Deck and Ballast tanks to be of stainless steel. Steam pipe, condensate pipe and copper  line  to be installed with Teflon or equivalent pads at bottom and top to avoid metal to metal contact with the pipes.

In general, double nuts U-bolts shall be used for all heating coils  (each one on both sides of the support).

However, all size of vent pipes, and the other pipes not mentioned above shall be supported according to shipyard standard.

Heating coils pipes and others subject of temperature expansion to be provided with sliding pads or other means to compensate expansion.

Sliding pads shall be inserted between pipes and supports for the following pipes:

- Longitudinal pipes of ND125 mm and above in pipe duct

5.1.2.9	Pipe Protection

The piping works shall be protected from mechanical damage, where necessary and the protection shall be constructed with steel angels, plates or channels as per Builder’s practice.

5.1.2.10	Surface Treatment and Cleaning

The surface treatment and coating of the machinery, loose fittings and valves shall be carried out in accordance with builder's standard for the intended purpose, and no special surface treatment shall be made exceeding Maker’s standard unless otherwise specified.

The pipe fittings whose function is to join branches of the system (such as tees, Y-pieces, elbows, flanges, unions, bushing, etc.) shall be painted same as connected pieces.

The loose fittings other than above pipe fittings supplied by makers (such as expansion coupling, filters, strainers, traps, separator, rose boxes, nozzles, plug, orifice etc.) shall be painted in accordance with maker’s standard unless otherwise specified.

The galvanizing shall be carried out after fabrication of pipes in principle, but if welding has to be carried out after galvanizing for the cases mentioned below, the external surface of the damaged parts shall be touched up with two (2) coats of zinc solution paint of high purity.

a)	Welded sleeve joints fabricated on-board

b)	Flanges of pipes adjusted on-board

c)	Middle flanges of penetrating pieces adjusted on -board

d)	Butt wild joint adjusted on-board

The fuel oil service pipes, lubricating oil pipes and hydraulic oil pipes shall be pickled before installation, refer to Machinery Chapter paragraph 6.10.1of the Specification.

5.1.2.11	Inspection and Test

All the tests and inspections of pipe lines shall be carried out in accordance with the requirements specified in the specification and the Rules and Regulations and the Builder’s standard.

Visual Test

Dimensions, scantlings, locations, surfaces finishing, welding, fastenings, joints, etc. shall be visually inspected where necessary.

Leakage Test (Flow Test)

The pipes which to be occasionally filled up by gravity or pipes through which liquid flow without pressure during the actual service shall be tested by free flow of water to confirm tightness of pipe lines.

Pressure Test

The statically hydraulic pressure test shall be carried out in accordance with the requirement of the Classification Society.

During the pressure test, a blank flange shall be provided at the open ends with the valves and/or any other means of closing on the piping fully open. For pipe insulation refer to Machinery Chapter, item 6.9 of the Specification.

5.1.3	BILGE PIPING

Cargo hold

One (1) main bilge line for cargo holds shall be arranged in pipe duct and branched to each bilge well, fitted with rose box, non-return valve and remote control butterfly valve(arranged in pipe duct) at each suction mouth.

Each cargo hold shall have two (2) bilge wells at the aft end of the hold (one on each side of the Vessel).

The main bilge line shall be connected to fire, bilge & G.S pumps in engine room.

Dangerous goods bilge water in cargo holds shall be discharged overboard through the main bilge line by a dangerous goods bilge eductor located outside of the engine room when carry dangerous goods.

The bilge system shall be so designed that the dangerous goods can be carried in No.1~3 cargos holds. Shut off valve should be fitted directly at the bulkhead on machinery space of end of bilge main line to ensure the separation of bilge water from engine room when dangerous goods are loaded in cargo hold.

Bow part

One (1) bilge eductor shall be provided to discharge bilge in the chain locker, void space above fore peak tank and bosun store. And driving water for eductor shall be supplied from the fire main line.

One (1) bilge eductor shall be provided to discharge bilge in the bow thruster & emergency fire pump room. And driving water for eductor shall be supplied from the fire main line.

Stern part

Bilge water in the steering gear room shall be led to the bilge well in the engine room through self-closing valve.

Other spaces

The daily bilge piping system in E/R shall be independent of main bilge piping system.

Oily bilge discharge pipe with stop valve and MARPOL shore connection flange shall be provided at each side on upper deck.

The valve for bilge lines in engine room shall be manually operated.

For bilge pump and bilge system in the engine room, refer to the Machinery Specifications.

High level alarm shall be provided for bilge wells.

5.1.4	BALLAST PIPING

One (1) ballast main ring line shall be arranged in pipe duct for ballasting, de-ballasting and transferring the ballast water in double bottom ballast tanks, side ballast tanks.

Ballast system shall consist of two (2) ballast pumps and one (1) ballast water treatment plant in engine room.

Trim adjustment shall be remotely done by ballast system from ship office.

A suction bell mouth shall be provided in each ballast tank. The suction mouth shall be connected with flange in tanks and easily dismounted.

Drain hole shall be provided enough at structure member surrounding mouth pieces.

The remote control pneumatic butterfly valve shall be fitted on each branch ballast line in pipe duct and shall be remote operated from ship office.

All remote operated ballast water valves can be open/shut manually in case failure of remote operation. And local open/shut indication shall be provided for the remote operated ballast water valves.

The valve before collision bulkhead shall be fitted directly at the collision bulkhead inside the bow thruster & emergency fire pump room located after the collision bulkhead, the valve operation position as per Class Rules.

Remote level able to display tank volumes and draft reading for ballast tanks shall be indicated on monitor in ship office and loading computer.

Particulars of ballast pumps and remote control butterfly valve system refer to Machinery Chapter of the specification.

5.1.5	ANTI-HEELING PIPing

Two (2) anti-heeling tanks (P&S) shall be cross connected by steel pipes and one (1) dedicated anti-heeling pump shall be provided.

During loading and unloading of containers, heel adjustments shall be carried out by means of transferring the sea water between one pair of side ballast tank (port and starboard).

The anti-heeling operation shall be effected by start and stop sequence of heeling pump and respective control of valve and shall be controlled the Vessel's heel within +/-2 degrees.

Heeling control system shall have the start/stop function for heel adjusting, the heel indication and the over heeling alarm for heel adjusting tanks on the heeling control mode on TFT in ship office.

Before loading and unloading of containers, both anti-heeling tanks shall be about half filled with sea water.

Anti-heeling Pump

No. of set	:	One (1)

Capacity	:	About 350 m3/h x 15 mTH (according to final calculation)

Type	:	Reversible propeller type

Prime mover	:	Electric motor

Materials	:	Pump casing of bronze, impeller of Ni-Al-bronze and shaft of stainless steel

The following alarm shall be provided visible on the display of the operator panel and audible at the remote control panel:

Excessive Heeling (>5 degree)/ Emergency stop

Failure heeling pump and relative valves

Heeling tanks level alarm

The effect at any alarm occurring shall result in closing of valves and stopping of pump.

System common abnormal alarm to AMS shall be provided

5.1.6	VALVE REMOTE CONTROL SYSTEM

1)	Valve remote operation

The valve control system of pneumatic type shall be provided for the remote control of following valves:

-	Ballast valves in cargo hold area and major valves in engine room for ballast system

-	Bilge suction valves in cargo hold area

-	Fuel oil filling / suction main valves in cargo hold area

Discharge main valves of the ballast pumps shall have open, closed and intermediated position indications. Other valves shall have open and closed indication only.

The remote operated suction valves for HFO and LSHFO/ULSFO bunker tanks in cargo hold area shall be single acting type.

The remotely controlled valves shall have an emergency operation facility by means of portable hand pump or spanner.

Remote valves should not be placed in areas they serve and should be easily accessible in any operational condition of vessel.  where is possible.

2)	Control

A control station with color graphic monitor shall be provided in the ship office, engine control room for ballast, bilge, FO.

Control functions and indications mentioned below shall be mounted on the monitor.

-	Mimic diagram of ballast, bilge and FO system

-	Open / shut function with indication for remote operated valves (Remote control valves in fuel oil system shall be controlled from engine control room.)

- Start/ stop function with running indication for ballast pump, fire, bilge & G.S. pumps.

-	Suction and discharge pressure indication on monitor for ballast pump

-	Level indication able to display tank volumes on monitor for water ballast, fresh water tank, HFO and LSHFO/ULSFO storage tanks and MGO storage tank.

-	Draft indication on monitor

-	High level alarm for cargo hold bilge wells, bosun store, fore & aft end of pipe duct, bow thruster & emergency fire pump room.

-	Other necessary equipment and fittings not mentioned above shall be provided as per maker's standard

- 	HFO and LSHFO/ULSFO and MGO storage tanks shall be provided with independent level switches for high level alarm.

5.1.7	AIR AND SOUNDING PIPES

5.1.7.1	Air escape/Overflow Pipe

The air pipes shall be arranged in compliance with the load line regulation.

In general, one (1) or two (2) air pipes shall be provided for each tank and void space and shall be led to above exposed deck.

Air escape/overflow pipes shall be provided to all cofferdams and tanks, and shall be terminated in the place that shall be in accordance to the requirements of the classification society with suitable heads.

The air pipe head for fuel oil tank shall be fitted with float type closer and fire proof screen of stainless steel, and the head for fresh water tank shall be fitted with closer and stainless steel wire gauge insect guard. Air escape/overflow pipes for oil tanks shall be of steel and for other tanks shall be of steel pipe galvanized.

Oil spill coaming with plug shall be provided around air escape pipe head of oil tanks in accordance with the U.S.C.G. requirements.

Size of air escape/overflow pipes shall be not less than 125% of sectional area of suction pipe in accordance with the requirements of the Classification Society.

Air vent pipes for HFO and LSHFO/ULSFO bunker tanks shall not be arranged passing through the water ballast tanks or vice versa as far as practicable.

The location of air vent head for fuel oil tanks shall be as per Rule’s requirement.

Welded sleeve joint shall be used for air escape pipe passing through deck.

Steel vent pipes to be required for water tanks outside of engine room.

5.1.7.2	Sounding Pipe

Sounding pipe with bronze screw type cap (deck stand type) or deck flush type shall be fitted to water ballast tanks, cofferdams and chain lockers, and terminated above upper deck or hatch coaming platform in accessible location.

Double bottom tanks in engine room shall be provided with short sounding pipe terminated above floor plating about 800 mm with self-closing devices, and self-closing test valves are only used for double bottom oil or oily tanks in engine room according to class requirement..

The sounding pipes for side ballast water tanks shall be terminated on weather deck level. Those for double bottom ballast tanks and cargo hold bilge wells shall be arranged between transverse bulkhead and cell guide in cargo holds, and terminated on weather deck level.

The sounding pipes for HFO and ULSFO/LSHFO tanks shall be terminated on weather deck level in accessible location.

The sounding pipe shall be of steel pipe of 50 mm for fuel oil tanks and cargo hold bilge wells, 40 mm for water tank and chain locker unless otherwise required by the Class. Application of galvanization for sounding pipes shall be the same as air escape/overflow pipes.

Generally the sounding pipe shall be as straight as practicable and fitted with brass cap.

The lower end shall be extended to a height of about 20 mm above the tank bottom and striking pad of 10 mm thick shall be fitted to pipe end.

Lower part of sounding pipes about 350 mm length shall be detachable for removing broken sounding tapes.

One (1) test cock with self-closing valve shall be provided for stern tube cooling water tank.

Two (2) sounding tapes with plumb shall be furnished, one (1) for clean water tanks and the other for dirty oil tanks.

Sounding pipes for HFO and LSHFO/ULSFO bunker tanks shall not be arranged passing through the water ballast tanks or vice versa as far as practicable.

100mm coaming to be provided for all the sounding pipes used for oil tanks.

Ship side pipes/ Scuppers shall be welded to side shell with deep penetration weld.

5.1.7.3	Remote Reading Level Gauge and Bilge Alarm

One (1) remote reading level and volume gauge of electronic pressure sensor type able to display tank volumes shall be provided for each water ballast tank and fresh water tank.

One (1) remote reading level and volume gauge of electronic pressure sensor type able to display tank volumes shall be provided for each HFO , LSHFO/ULSFO and MGO tank as specified at Machinery Chapter 6.2.18.

High level alarm shall also be provided for HFO , LSHFO/ULSFO and MGO tank in engine room as specified at Machinery Chapter 6.2.18..

One (1) bilge high level alarm by a float switch shall be provided for each bilge well in all cargo holds, bosun store, , bow thruster & emergency fire pump room and fore end of pipe duct and one (1) common audible alarm for bilge wells shall be provided in engine control room and wheelhouse.

Suitable protection pipe for oil tank for electrical sensors cable submerged in tank shall be provided accordance with Maker’s recommendation. PVC protection pipe to be used for water tanks and bolt(if used) to be stainless steel.

5.1.7.4	Name plate

Name plates for air and sound pipes shall be provided in English.

5.1.8	FRESH WATER SERVICE SYSTEM

5.1.8.1	Cold fresh water system and technical water system

One (1) filling pipe shall be provided for fresh water tanks. And pipe inside of F.W. tank shall be seamless steel pipe.

Diameter of fresh water filling pipe shall be not less than 65mm.

Hydrophore tank system shall be adopted to supply cold fresh water.

Cold fresh water system shall have a general fresh water service line, and one (1) fresh water hydrophone tank and two (2) fresh water pumps driven by electric motor.

One (1) of two (2) pumps shall serve as stand-by for the respective service line mentioned above.

The pumps shall suck from the fresh water tanks, and discharge through the stop check valves to hydrophone tank.

Generally, fresh water supplied from the fresh water hydrophore tank in the engine room to be led to, wash basins, showers, bath tub, laundries, hospital, sanitary water system by means of branches with shut-off valves at the main for each deck and service for engine room, front windows of wheelhouse and deck washing.

The fresh water hydrophore tank shall be constructed of stainless steel and finished completely with gauge glass, pressure gauge, relief valve, drain connection and connection from the compressed air system for charging purpose.

The automatic pressure control shall be fitted to start and stop the pumps to maintain the necessary pressure in the tank.

5.1.8.2	Hot fresh water system

The closed loop system shall be adopted to supply hot fresh water and to consist of one (1) calorifier for heating water, one (1) hot water circulating pump, main circulating pipe and branch pipes.

The calorifier shall be shell& tube type which using steam &electric heating.

The system shall be supplied through non-return valve from fresh water hydrophore tank, and hot water shall be led to galley, pantries, wash basins, showers, bath tub, laundries, and lobby of refrigerated provision chamber, etc.

Hot water and cold water shall be also supplied to wheelhouse for front windows washing.

5.1.8.3	Distilled water system

The distilled water from the fresh water generator shall be passing through water meter and salinity indicator to the fresh water tanks.

5.1.8.4	Drinking water system

Drinking water line to be leaded from the fresh water hydrophore.

One (1) set of F.W. sterilizer and one (1) set of F.W. mineralizing unit to be provided for drinking water system.

The drinking water only led to galley, pantry and drinking water fountains.

5.1.8.5	Sanitary Water System

Grey water discharge pipe shall be led overboard independently of sewage pipe through one (1) storm valve above the light ballast water line and to the sewage holding tank.

Grey water shall include the discharged water from wash basin, small wash basin, sink, shower and bath tube.

Black water main pipe from toilets and urinals in each board can be led to the sewage treatment plant installed in engine room during in harbor or directly be discharged overboard by-passing the sewage treatment plant through one (1) gate valve and one (1) non-return valve near the light ballast water line during at sea.

After the treatment plant, the sewage also can be discharged overboard at sea or to sewage holding tank in harbor duration, then using the sewage pump to discharge overboard at sea.

Black water and grey water pipe line from hospital shall be led independently to the sewage treatment plant.

The sewage holding tank must be provided with a pipe all round on the top with perforation to clean and flush the tank with sea water.

Sufficient scuppers shall be provided for galley, pantry, lavatories, refrigerated provision chambers, passages in living quarters and other enclosed spaces, wherever necessary.

5.1.9	SCUPPER AND DRAINAGE SYSTEM

Sufficient scuppers shall be provided for all decks, top of deck houses and other enclosed spaces, wherever necessary.

Drainage shall be arranged away from accommodation ladder, pilot ladder and life boat position.

Upper deck scuppers shall have \grids, a rubber plug shall be provided on the top of each scupper.

Separate scuppers for refrigerated space shall be with plugs.

Scupper pipes shall be led to the weather decks or overboard directly according to the situation.

Storm valves shall be fitted for necessary pipes as required by the Classification Society.

Water sealed traps shall be fitted to the refrigerated provision chambers.

Scuppers shall be arranged accessibly, and hard bending for galley scuppers and soil pipes shall be avoided as far as possible.

Gutter way in stainless steel shall be arranged on the deck of galley with scuppers on each corner. Fat trap shall be provided for galley scuppers.

Adequate cleaning plugs shall be fitted, wherever necessary.

Necessary drain plugs shall be fitted as required.

Drain pan shall be fitted frames.

Gutter bar shall be fitted along the inside deck house on each deck, finally according to the detail design.

Gutter bars shall be fitted around deck machinery, steering gear etc.

Drain pipe from funnel top to engine casing top shall be provided.

5.1.10	COMPRESSED AIR PIPE FOR DECK USE

Suitable number connection piece shall be provided on weather deck including deckhouse, mooring deck, etc for general service.

Connection piece with DN15mm hose valve and hose coupling (quick coupling type) shall be provided.

Air shall be supplied from deck service air reservoir in the engine room.

Compressed air shall be supplied for following space or equipment:

Bosun store

Steering gear room

Workshop

Bow thruster & Emergency fire pump room

CO2 room

Emergency generator room

Air conditioning room

Paint store

Forecastle deck

Upper deck at an interval of 30m between cargo holds

Each deck outside of living quarters P/S

The separate airline from main air reservoirs via two (2) reducing valves shall be supplied to air horn with locally open valve.

5.1.11	FUEL OIL PIPING

Fuel oil filling main lines of one (1) DN200 for HFO and LSHFO/ULSFO and one(1) DN100 for MGO respectively with butterfly valve and blank flange shall be provided.

The remote control valve shall be fitted on branch fuel oil filling line of each HFO and LSHFO/ULSFO bunker tanks.

The main lines shall be connected to the fuel oil fill piping and fuel oil transfer pump in engine room.

Each one(1) bunker station for HFO and LSHFO/ULSFO and MGO shall be provided at each side of the vessel on upper deck, and with sampling device, thermometer, butterfly valves blank flanges, pressure gauges, spill trays etc.

Oil spill spaces with drain plug shall be provided at F.O. filling stations.

Caution plate required by U.S.C.G. shall be fitted on bunker station and ship office.

Remote level reading for oil tanks shall be indicated in engine room.

High level alarms shall be provided for HFO/ULSFO bunker tanks and MDO/MGO bunker tank.

5.1.12	FUEL OIL TANK HEATING SYSTEM

A steam line for HFO and LSHFO/ULSFO bunker tank heating and a drain line in side void space (P or S) shall be led from engine room to fuel oil bunker tanks and FO overflow tank.

Heating coils shall be of seamless steel and heating area ratio shall be 0.06m2/m3 for heavy fuel oil bunker tanks.

The steam shall be led from steam supply main line in void space through under deck to each HFO/ULSFO tank via a manually operated shut-off valve, and led back to drain main line in the void space through a steam trap/strainer and a shut-off valve .

Steam tracing shall be adopted for suction pipe of HFO and LSHFO/ULSFO line and steam tracing shall be insulated together with HFO and LSHFO/ULSFO suction main line.

The remote temperature reading shall be provided for the HFO and LSHFO/ULSFO bunker tanks.

5.1.13	DRAFT GAUGE

The sea valves for draft gauges shall be manually operated at local side.

Draft gauges shall be provided to read fore, midship (P/S) and aft draft with remote reading by analogue and digital display on TFT in ship office.

5.1.14	SCHEDULE OF HULL PIPE

| System |  |  |  | Pipe |  |  | Pipe Joint | Pressure for flange (MPa) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Size | Material | Thickness |  |  |
| Tank heating coil in fuel oil tank |  |  |  | All | Seamless steel | B | sleeve | 1.0 |
| Compressed air pipe for deck service |  |  |  | All | Seamless steel galv.* | A | Sleeve or flange | 1.0 |
| Fuel oil | In pipe duct |  |  | All | Seamless steel | A | Sleeve or flange | 0.6 |
|  | On deck |  |  |  |  | B |  |  |
| Ballast | In pipe duct water ballast tanks and void spaces |  |  | All | Seamless steel galv. | B | Sleeve or flange | 0.6 |
| Bilge | In water ballast tanks |  |  | All | Seamless steel galv. | B | Sleeve or flange | 0.6 |
|  | In other spaces |  |  | All | Seamless steel galv. | A | Sleeve or flange | 0.6 |
| Domestic steam & drain |  |  |  | 15 & above | Seamless steel galv. | A | Sleeve or flange | 0.6 |
|  |  |  |  | 10 | Seamless copper |  | Socket | 0.6 |
| Fresh water in accommodation |  |  |  | All | PERT** |  | Sleeve or Flange | 1.0 |
| Hot fresh water in accommodation |  |  |  | All | PERT** |  | Sleeve or Flange | 1.0 |
| Soil pipe |  |  |  | All | Seamless steel galv. or PVC** | B | Sleeve or Flange | 0.6 |
| Sanitary Water |  |  |  | All | Seamless steel galv. or PVC** | B | Sleeve or Flange | 0.6 |
| Scupper |  | Above upper deck |  | All | Seamless steel galv. | B | Sleeve or flange | 0.6 |
|  |  | Under upper deck |  | All | Seamless steel galv. | B as per Rule | Sleeve or flange | 0.6 |
| Air |  | For oil tanks |  | All | Seamless steel. | B as per Rule | Sleeve or flange | 0.6 |
|  |  | For other tanks |  | All | Seamless steel galv. | B as per Rule | Sleeve or flange |  |
| Sounding |  | In oil tanks |  | All | Seamless steel | B as per Rule | Sleeve or flange | 0.6 |
|  |  | In other tanks |  | All | Seamless steel galv. | B as per Rule | Sleeve or flange | 0.6 |
| Fire and wash deck |  |  |  | All | Seamless steel galv. | B | Sleeve or flange | 1.0 |
| Refrigerant |  |  | Connecting pipe | All | As per Maker’s standard (to be seamless) |  |  |  |
| CO2 |  |  | Connecting pipe | All | As per Maker’s standard (to be seamless) |  |  |  |

Notes:

Thickness of pipe in the table refer to Paragraph 7.11.4 of Machinery Part. A……ordinary,   B……Medium,	C……Heavy

* Compressed air horn pipe exposed to weather shall be of seamless copper.

** If the PERT or PVC used ,the pipe penetration to be steel filling with PERT or PVC inside.

All galv. pipes to be of hot dip type.

The F.O. pipe, steam pipe to be not through water tank. The water pipe to be not through oil tank.

5.1.15	SCHEDULE OF VALVES

| Piping system | Size (nominal) | Material |  |  |
| --- | --- | --- | --- | --- |
|  |  | Body | Seat | Stem |
| Steam and drain line for tank heating | 50 mm & above | Cast steel | As per Maker’s standard |  |
|  | 40 mm & under | cast steel |  |  |
| Fuel oil line | All | Cast steel or cast iron | ditto |  |
| Scupper line (storm valve) | All | Cast steel | ditto |  |
| Ballast and bilge line | All | Cast iron | ditto |  |
| Cold water line | All | As per Maker’s standard ditto |  |  |
| Hot water line | All | As per Maker’s standard ditto |  |  |
| Domestic steam and drain | 40 mm & under | Cast steel | ditto |  |
| Fire and wash deck line | 50 mm & above | Bronze* | ditto |  |
|  | 40 mm & under | Bronze | ditto |  |
| Refrigerant | All | As per Maker’s standard |  |  |
| CO2 | All | As per Maker’s standard |  |  |

“*” Except butterfly valve.

For the valve seat and stem material, finally to be decided by the usage and the Maker, and satisfied with the rule’ requirement.

5.1.16	SCHEDULE OF PIPE INSULATION

| Piping system | Range of Insulation | Material |  | Cover |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Pipe | Flange & Valve | Pipe | Flange & Valve |
| Exhaust gas pipe for emergency generator engine | in accommodation | Ceramic fibre moulding or equivalent | Ceramic fibre moulding or equivalent | Wire net + Glass cloth + galv. steel sheet 0.5 thick |  |
| Domestic steam and drain | in accommodation | Ceramic fibre moulding or equivalent | Ceramic fibre moulding or equivalent | Glass cloth | Glass cloth |
| Hot water | in accommodation | Ceramic fibre moulding or equivalent | Ceramic fibre moulding or equivalent | Glass cloth | Glass cloth |
| Refrigerant | Suction line from ref. chamber to compressor | As per maker’s standard |  | As per maker’s standard |  |
|  | Line from expansion valve to ref. chamber | As per maker’s standard |  | As per maker’s standard |  |

5.2	FIRE FIGHTING SYSTEM

5.2.1	GENERAL

The following fixed fire-fighting systems shall be provided for the spaces as listed below.

Sea water system	Exposed decks, Engine room, Steering gear room, Cargo hold, Paint store

Living quarters, and other working spaces

Fixed fresh water mist system	Localized fire suppression in E/R

CO2 fixed system	Engine room, Cargo hold, Galley hood

Portable fire extinguisher	Living quarters and other working spaces, engine room, emergency generator room and emergency fire pump room

All fire-fighting system and equipment shall comply with the requirements of SOLAS.

Fire Control Plan shall be provided in accordance with Rule and Regulation and shall be stored in galvanized steel case on both sides of living quarters on upper deck.

Fire control station is shall be provided with controls for quick closing valves, E/R fans, CO2 releasing box for engine room, oil pumps etc.

One fire detector for every laundry.

5.2.2	SEA WATER FIRE FIGHTING SYSTEM

5.2.2.1	Fire Main Line

Fire main line water would supplied from two (2) sets of the fire, bilge & G.S. pumps in E/R, and an emergency fire pump. The fire main line and brand line shall be led along the void and connected to fire hydrants on open deck.

Start/stop of the fire, bilge & general service pump shall be done remotely from engine control room, ship office, fire station, wheelhouse (only for fire & general service pump) and locally at pump side in accordance with rule requirement.

One (1) isolation valve with a pressure gauge shall be provided on the fire main. The pressure of fire main shall be indicated in the ECR in addition to the local one.

Sufficient number of drain cock shall be provided for fire main line to release the remaining water for anti-freezing use.

The system also shall be used for washing decks and for other purpose as follows:

1)	For washing chain, nozzles shall be provided in each hawse pipe with one stop valve.

2)	Power water for bilge eductor of chain locker & bosun store.

3）	Power water for dangerous goods bilge eductor.

4）	Power water for ballast stripping eductor.

5)  Cooling for Windlass and mooring winch hydraulic unit

Two (2) sets of mobile water monitor with a dual-purpose spray/jet type nozzle shall be provided, and the minimum capacity of the monitor shall be 60 m3/h, at the pressure required by SOLAS regulation II-2/10.7.3, and in compliance with IMO Res.MSC.365(93) requirements.

For open deck containers, water mist lance as required by MSC.365 (93) to be provided.

5.2.2.2	Fire Hydrant and Hose Rack/Box

Fire hydrant shall be so arranged that the jets of water may reach any part of the ship and any part of an empty cargo space, furthermore, such hydrants shall be positioned near the accesses to the protected spaces.

The nominal diameter of fire hydrant shall be DN50 mm for all areas.

Two (2) international shore connections shall be provided adjacent to the living quarters.

The fire hydrant shall be manually operated valve with hose coupling of brass.

Each fire hose outside machinery space shall be 18 m length complete with a coupling and a combined jet/water fog nozzle, which shall be stowed in steel cabinet for weather & outside of living quarters and fiberglass cabinet in the other enclosed spaces. In machinery space, the fire hose shall be 15m length.

Fire hydrants line and the hose racks/boxes shall be painted in red.

5.2.2.3	Emergency Fire Pump

An electric motor driven emergency fire pump shall be installed in bow thruster & emergency fire pump room.

The electric power for emergency fire pump shall be served from the emergency switchboard.

The pump should be able to draw water and build up design pressure at ship ballast condition.

The pump shall be self-priming centrifugal type. For material of pump, refer to Machinery Chapter of the specification.

The emergency fire pump shall have independent sea chest and discharge to fire main line through non-return stop valve.

The emergency sea chest shall be below the waterline of lightest ballast condition.

5.2.2.4	Sprinkling Water System for Paint Store and Garbage Store

The spraying sea water for the paint store and garbage store shall be supplied from fire main line and capable of being operating from outside as per Rule/Regulations.

Smoke detector shall be provided in the paint store as per Rule/Regulations.

5.2.2.5	Water mist fire fighting system in E/R

Refer to “Chapter 6-Machinery, 6.11.8.2.2”.

5.2.3	CO2 FIRE EXTINGUISHING SYSTEM

A total flooding CO2 Fire Extinguishing system shall be provided for the engine room, cargo holds etc. in accordance with the requirements of the Rule and SOLAS.

CO2 bottle each containing 45 kg of CO2 shall be arranged in CO2 room.

CO2 fire extinguishing system should be designed that the releasing of CO2 bottles can be carried out by remote control method in fire control station for engine room, and in CO2 room for cargo hold and in accordance with MSC.339 (91) requirements. Manual or group release in CO2 room for cargo hold shall be in accordance with Class requirement.

The remote control system shall be interlock with audible and visual alarm.

CO2 line shall be led from CO2 bottles to each cargo hold and via 2-way valve on discharge line and solenoid valve on smoke suction line in CO2 room, which can be operated remotely from fire control station.

The engine room fans shall be stopped automatically by means of a limit switch in the release cabinet.

Adequate number of CO2 discharge nozzles shall be so arranged on the branch lines that CO2 gas may be efficiently discharged into cargo holds and engine room.

CO2 gas smoke suction line and discharge manifold shall be led in under deck void to each protected container hold, and smoke tubes shall be led to the detecting cabinet in the CO2 room.

Necessary accessories shall be provided in accordance with the requirements of Regulations and manufacturer’s standards.

An electric horn (Explosion-proof type when carry dangerous goods) shall be provided in each cargo hold for alarming CO2 discharge.

One (1) CO2 bottle with manual releasing valve shall be fitted in the gallery and releasing nozzles shall be connected to the exhaust duct of hood if necessary in accordance with the requirement of the SOLAS.

One (1) CO2 weighting device shall be provided and safety devices shall be provided in accordance with the requirements of Regulations and manufacturer’s standards.

Maker standard plate operation and distribution instruction (in English) for CO2 system shall be provided in CO2 room, wheelhouse and fire control station.

CO2 fire extinguishing system shall provide with compressed air connections.

CO2 gas leakage alarm shall be provided.

A smoke extraction fan unit shall be installed in the CO2 room, but the outlet of the extraction pipe shall be arranged far away from the normal crew working area.

5.2.4	PORTABLE FIRE EXTINGUISHER

Portable form, CO2 and/or dry power fire extinguishers shall be furnished and properly secured in accordance with the requirement of the SOLAS.

| Type | Space |
| --- | --- |
| 5 kg CO2 type | Wheel house & chart space, electric equipment room, electrical workshop, galley, ER workshop, emergency generator room, engine room, air conditioning room, ship office and fire control station |
| 45L foam type | Engine room |
| 135L foam type | Engine room |
| 9L foam type or 6 kg dry powder type | Space where required by rules |

The number of portable fire extinguishers shall be decided in accordance with the rule requirement. Spare charges shall be stowed in the safety store in living quarters.

Survey card shall be fitted for each portable fire extinguisher.

5.2.5	FIREMAN OUTFIT

Fire-fighting apparatus shall be provided in accordance with the requirements of the Rule/Regulations as follows:

- Two (2) sets of firefighter’s outfits including self-contained breathing apparatus in fire control station;

- Two (2) two-way portable radiotelephone apparatus (as a headset in the mask);

- Four (4) full protective clothing resistant to chemical attack for dangerous goods carriage;

- Two (2) self-contained breathing apparatuses in addition to the above for dangerous goods carriage;

Heat protection suit shall be of heavy duty version with heat protection for gloves and boots and hood.

5.2.6	FIRE DETECTION AND ALARM SYSTEM

Indication of fire shall be provided on fire alarm panel in wheelhouse and extension alarm in fire control station in accordance with SOLAS.

Comply with latest FSS Code. E.g Detector and Control panel are to be tested according to EN54(2001)

Suitable numbers of fire/smoke detectors shall be provided in the machinery spaces, living quarters, alleyways, and staircase according to the requirement of the Rule/Regulations.

Smoke tube type fire detecting system shall be provided for cargo holds.

CO2 smoke suction line shall be led in each cargo hold, smoke detection with valve and suction fan shall be arranged in the CO2 bottle room, a repeater panel shall be in the wheel house.

Audible / visual alarm shall be provided according to the Rule.

Suitable numbers of push buttons shall be provided in access of every deck and passage ways and suitable spaces.

The details of the system refer to “Chapter 9 - Electric Part” of the Specification.

5.2.7	FIRE control STATION

A fire control station for fire-fighting system shall be arranged in living quarters.

Followings shall be controlled or stowed in:

- Emergency stop of the ventilation fans for E/R

- Emergency stop of fuel oil pumps and lubricating pumps in E/R

- Emergency quick-closing of valves on oil tanks in E/R

- Start/stop push button of the bilge, fire & G.S. pumps and emergency fire pump

- Fire alarm push button

- Fire alarm repeater panel

- Portable international shore connector

- Portable fire extinguisher as required by the Rule

- CO2 releasing box for engine room

- Auto telephone

- Others as per the Rule and Regulation

5.2.8	ESCAPE TRUCK FOR ENGINE ROOM

One (1) escape truck shall be provided on each floor of engine room and engine control room to a safe position outside engine room. Escape route of E/R workshop to be considered (additional one (1) if necessary)

5.2.9	MISCELLANEOUS

Fire-fighting equipment, such as axes, emergency escape breathing devices (EEBD), etc. shall be provided and stowed in the space in accordance with the Rules and Regulations in the Specification.

5.3	AIR CONDITIONING AND VENTILATION SYSTEM

5.3.1	AIR CONDITIONING SYSTEM

Air Conditioning System shall be provided to serve as ventilation, heating and/or cooling for all cabins, offices spaces, public spaces and hospital in this Specification.

The air condition system shall be of medium or high pressure, medium velocity, single duct system having a central heating and cooling plant.

The system shall consist of refrigerating plant and air handling unit with distribution duct.

One (1) set of water-cooled package air conditioner shall be provided for engine control room, and galley. The package air conditioners to be cooled by LT FW in the engine room.

Spot cooling shall be arranged in wheelhouse, chart room, catering space, and not be counted as air conditioned spaces with regard to temperature and relative humidity.

The fresh air shall be conditioned by air handling unit, and the conditioned air shall be supplied to the air conditioning spaces and spot air cooling/heating spaces.

The surplus air in conditioned space, except hospital, shall be exhausted into the corridor or main line pipe through the door louver and/or into the appropriate private sanitary space, if applicable, through the door gap.

The surplus air in corridor shall be exhausted naturally to weather.

No recirculation from hospital, catering space, dry provision store and sanitary space.

Diffuser shall not be located directly above bed or desk, as far as practicable.

One (1) set of portable type, refrigerant recovery unit shall be provided in accordance with Classification Society requirements.

Air intakes for accommodation ventilation shall not be situated near fuel/lube vents or be able to take in tainted air (air that smells of fuel fumes)

The doors to each cabin or wall where necessary, shall be fitted with ventilation shutter to allow exhausting air into the corridor.

- Design condition:

|  | Cooling | Heating |
| --- | --- | --- |
| Outside | 35℃ 70%RH | -20℃ |
| Inside | 27℃ 50%RH | 20℃ 50%RH |
| Cooling F.W. temp | 36℃ |  |

The plant shall be designed to cool and/or heat the supply air and to maintain the above condition when the doors and windows are closed.

The fresh air ratio shall be 50% fresh air for summer and winter, and 100% fresh air for other seasons.

- Air changing rate for air condition system

Air conditioning system shall be designed to have the following minimum air change rate based on the volume of each compartment.

| Compartment | Air changes per hour | Remarks |
| --- | --- | --- |
| Private space | 6 |  |
| Public space | 10 |  |
| Office space | 10 |  |
| Hospital | 10 |  |
| Catering space except galley | 15 | Spot A.C |
| Changing room | 6 | Spot A.C. |
| Wheelhouse/radio space/chart space | 15 | Spot A.C. |
| Dry provision store | 6 | Spot A.C. |
| Laundry | 5 | Spot A.C. |
| Bonded store | 5 | Spot A.C. |
| Elect. Equipment store | 5 | Spot A.C. |

- Air conditioning plant

Two (2) sets of refrigerating plant, which to be R-407C direct expansion and electric motor driven, shall be installed, and each capacity shall be 60% of total.

Each plant shall consist of:

One (1) – Compressor

Electric motor driven, reciprocating or scroll type with manual start/stop, automatic unloading

One (1) – Condenser

Horizontal straight shell and tube type,

One (1) - Gas dryer

Leakage detector also shall be provided.
Air handling unit

One (1) set of air handling unit (100% capacity) shall be installed in air conditioning unit room.

The unit shall consist of:

One (1) - Ventilation fan

Single speed, electric motor driven, centrifugal type,

One (1) – Heating coil chamber.

One (1) - Cooling coil chamber,

One (1) - Humidifying fresh water spay nozzle for humidifying,

One (1) - Fresh air and air recirculation air mixing chamber with two (2) manual dampers,

One (1) - Removable air filter.

The accessories such as pressure gauge and thermometer shall be provided in accordance with the Maker’s standard.

Piping system

Cooling fresh water for condenser shall be supplied from the L.T. cooling fresh water system in engine room.

Temperature control system

Automatic temperature controller shall be fitted.

In summer, the supply air temperature from air handling unit shall be automatically controlled by expansion valve in AHU and compressor unloading mechanism in compressor.

In winter, steam regulating valve shall be controlled by a temperature sensor located in the main supply duct. Fresh water will be used for humidity.

In mild climates when cooling or heating is not required, the system to serve as mechanical ventilation for the accommodations, supplying fresh air only.

The temperature in each room shall be manually controlled by means of adjustment of air flow rate at each diffuser.

Ducting system

Fresh air for the air handling unit shall be drawn through the air supply head and shall be let to the central unit.

Recirculation air for the air handling unit shall be drawn through the grill provided on the wall or ceiling of the corridor space and shall be led to the air handling unit.

Air conditioning supply ducts shall be pre-insulated spiral or rectangle duct made of galvanized steel sheet.

The air conditioning supply ducts shall be carried into each room and terminated with ceiling type controllable diffuser and galvanized louvers in spot cooling spaces.

Necessary damper shall be fitted to main branches at easy accessible locations.

Non-return flaps and fire dampers shall be installed in accordance with the requirement of the rules.

Drainage of water shall be arranged from the lowest points of the ducts.

Easy access for inspection and service for fire closing and balancing dampers is to be provided.

Air supply line to hospital shall be provided with non-return damper and an individual exhaust fan.

Test and trial

The air handling unit shall be tested at the Maker's shop in accordance with the Maker's usual practice.

After the completion of air conditioning system, following tests shall be performed in accordance with Builder's practice.

1.	Air volume test

2.	Cooling or heating test

The air conditioning test shall be carried out as applicable in connection with testing season. When the heating or cooling is not applicable, only the running test shall be performed.

5.3.2	VENTILATION FOR CARGO HOLDS

For No.1~No.4 cargo holds, when dangerous cargo to be loaded, two (2) sets of exhaust ventilation fan to be supplied. This exhaust ventilation with air change of two (2) times per hour based on empty hold for carrying dangerous cargo. The fans shall be explosion proof type.

Fans shall be vertical axial flow and single speed type. And fans for cargo hold shall be split type.

The amount of fans to be according to the final design.

Remote start and stop for each fan shall be done in ship office.

One (1) hold fan monitor panel with running lamps and common buzzer for abnormal stop for cargo holds shall be provided in wheel house.

Ventilation ducts to be made of steel plate or galvanized spiral steel sheet for cargo hold.

Each ventilator shall be fitted with stainless steel wire net. Vent head shall be of mushroom ventilator type. Heights of ventilator openings above upper deck to be sufficient according to load line requirements.

The duct shall be designed to have the max. air flow of 15 m/s. The duct and vent head shall be painted same as surroundings.

5.3.3	VENTILATION FOR ACCOMMODATION AND OTHER SPACES

Mechanical supply and/or exhaust ventilation system shall be designed to have the following air change rate based on the volume of each compartment.

| Compartment | Supply | Exhaust |
| --- | --- | --- |
| Office space | *1) | 10 |
| Hospital/hospital bath room | *1) | 12 |
| Private lavatory | - | 10 |
| Public/deck toilet | - | 15 |
| Laundry | *1) | 10 |
| Drying room | *1) | 10 |
| Galley | AC30*2) | 40 |
| Pantry | *1) | 15 |
| Dry provision store | *1) | 5 |
| CO2 room | - | 6 |
| Battery room | - | 5 |
| Paint store |  | 6 |
| Air conditioning unit room | - | 15 |
| Emergency generator room | 20(finally according the calculation) | - |
| Steering gear room combined with store space | 6 | - |
| Bow thruster and emergency fire pump room | 10(finally according the calculation) | - |
| Pipe duct in double bottom | 6 | - |

Remarks:

*1)	Conditioned air shall be supplied in accordance with the air change rate specified on 5.3.1.

*2)	Conditioned air for galley shall be supplied from the separate unit.

3)	Hospital and galley shall have independently exhaust system.

4) AC: Spot A.C.

5.3.3.1	Mechanical Ventilation

Ventilation Fan

Mechanical ventilating fan shall be A.C. electric motor driven, single speed, duct/axial or centrifugal type.

The electric motor for the paint store shall be of explosion proof type.

Start and stop of the fan shall be done remotely.

Materials of the fan shall be as Maker’s standard.

Mechanical ventilation duct in living quarters

Mechanical ventilation duct shall be single spiral or rectangular duct made of galvanized steel sheet.

Stainless steel canopy shall be provided over electric range in galley for exhaust. Canopy shall have stainless steel mesh and fire damper.

Ventilator shall be having stainless steel wire net grid.

The duct opening on weather side shall have a rat proof wire net, and stainless steel insert screens with steel cover.

Mechanical ventilation duct in other spaces

Gooseneck, mushroom or wall louver type ventilations shall be provided as a ventilation head on weather deck. The size and type of ventilator shall be decided in accordance with the Builder’s standard to comply with the Rules/Regulations.

Ventilators shall be of fabricated mild steel plate and painted same as surrounding. Coaming heights shall be in accordance with requirement of Rules/Regulations.

In general, trunk and duct shall be of fabricated mild steel and rectangular cross-section type and painted same as surrounding. Small duct shall be galvanized steel plate and rectangular or circle cross-section type as per practice.

Engine room ventilation shall be described in “Part VII- Machinery Part”.

5.3.3.2	Natural Ventilation

The space which has no mechanical ventilation system shall be naturally ventilated through suitable mushroom ventilators, wall ventilators, and gooseneck ventilators or door louvers.

Natural ventilator heads leading to the stores and accommodation spaces shall be fitted with stainless steel rat-proof nets and those leading to living quarters shall be fitted with stainless steel wire net insect screen.

Natural ventilators shall be fitted with hinged type weather-tight closing device if required by the Rule.

Coaming height of ventilators shall be in compliance with Rule requirements and Builder’s practice.

CHAPTER	 6

MACHINERY

Version:A

Key words: Breadth 27.5m, Gearless, 6S60ME-C10.5 Tier II, 220ReeferPlugs, Open loop scrubber

CONTENT

CHAPTER 6   MACHINERY	4

6.1	GENERAL	4

6.1.1	GENERAL	4

6.1.2	INSPECTION AND TEST	5

6.1.3	FUEL OIL	7

6.1.4	DESIGN CONDITION	7

6.2	PRINCIPAL PARTICULARS OF MACHINERY	10

6.2.1	NOx EMISSION SOLUTION	10

6.2.2	MAIN ENGINE	10

6.2.3	SHAFTING AND PROPELLER	10

6.2.4	ELECTRIC GENERATING PLANT	11

6.2.5	STEAM GENERATING PLANT	11

6.2.6	COMPRESSED AIR SYSTEM	12

6.2.7	COOLING WATER SYSTEM	13

6.2.8	FEED AND DRAIN SYSTEM	14

6.2.9	FUEL SYSTEM	14

6.2.10	LUBRICATING OIL SYSTEM	15

6.2.11	FIRE, BILGE, BALLAST AND GENERAL SERVICE SYSTEM	16

6.2.12	FRESH WATER GENERATOR	17

6.2.13	FRESH WATER SERVICE SYSTEM	17

6.2.14	scrubber system	18

6.2.15	FANS	19

6.2.16	WORKSHOP EQUIPMENT	19

6.2.17	MISCELLANEOUS	20

6.2.18	TANKS IN ENGINE ROOM	21

6.3	MAIN ENGINE	23

6.3.1	GENERAL	23

6.3.2	FITTINGS AND ACCESSORIES	23

6.3.3	INSTALLATION	24

6.4	SHAFTING AND PROPELLER	25

6.4.1	GENERAL	25

6.4.2	INTERMEDIATE SHAFT	25

6.4.3	PROPELLER SHAFT	25

6.4.4	COUPLING	26

6.4.5	INTERMEDIATE SHAFT BEARING	26

6.4.6	STERN TUBE AND STERN TUBE BEARING	26

6.4.7	PROPELLER, FAIR WATER CAP AND PROPELLER NUT	27

6.4.8	EARTHING DEVICE	28

6.4.9	MATERIAL	28

6.5	ELECTRIC GENERATOR ENGINE	29

6.5.1	GENERAL	29

6.5.2	CONSTRUCTION FOR GENERATOR ENGINE	29

6.5.3	ACCESSORIES	29

6.6	STEAM GENERATING PLANT	31

6.6.1	GENERAL	31

6.6.2	ACCESSORIES	31

6.6.3	Materials	32

6.7	AUXILIARY MACHINERY	32

6.7.1	PUMPS	32

6.7.2	HEAT EXCHANGERS	34

6.7.3	AIR COMPRESSOR AND AIR RESERVOIR	35

6.7.4	OIL PURIFIER	36

6.7.5	FRESH WATER GENERATOR	37

6.7.6	ENGINE ROOM VENTILATING FANS	38

6.7.7	OILY BILGE SEPARATOR	38

6.7.8	SEWAGE TREATMENT PLANT	38

6.7.9	INCINERATOR	39

6.8	MISCELLANEOUS EQUIPMENT AND OUTFITTING	40

6.8.1	OVERHAULING AND REMOVAL OF MACHINERY	40

6.8.2	TANKS IN ENGINE ROOM	41

6.8.3	VENTILATING OF MACHINERY SPACE	42

6.8.4	SILENCERS	43

6.8.5	ENGINE CONTROL ROOM, ENGINE ROOM WORKSHOP & STORES	43

6.8.6	LADDERS, HANDRAILS, GRATINGS AND FLOOR PLATES	44

6.8.7	DISTINGUISHING AND UNIT	45

6.9	INSULATION AND PAINTING	46

6.9.1	INSULATING	46

6.9.2	PAINTING	48

6.10	 PIPING SCHEDULE IN ENGINE ROOM	49

6.10.1	GENERAL	49

6.10.2	VALVE AND FITTING	51

6.10.3	VELOCITIES IN PIPE LINES	52

6.10.4	MATERIALS & SIZE OF PIPING	52

6.10.5	STRAINERS	57

6.11	 PIPING SYSTEMS IN ENGINE ROOM	59

6.11.1	COOLING SEA WATER SYSTEM	59

6.11.2	COOLING FRESH WATER SYSTEM	59

6.11.3	LUBRICATION OIL SYSTEM	60

6.11.4	FUEL SYSTEM	61

6.11.5	COMPRESSED AIR SYSTEM	63

6.11.6	STEAM AND DRAIN SYSTEM	64

6.11.7	FEED WATER SYSTEM	64

6.11.8	SHIP'S SERVICE SYSTEM OF ENGINE ROOM	65

6.11.9	EXHAUST GAS SYSTEM	67

6.11.10	INSTRUMENTATION	68

6.12       scrubber system	69

6.13	SPARE PARTS AND TOOLS	70

CHAPTER 6   MACHINERY

6.1	GENERAL

6.1.1	GENERAL

The vessel shall be designed and constructed as a single screw driven by a diesel engine located aft and in compliance with the requirements of Classification notation.

The propelling machinery shall consist of a low speed, direct reversible, high efficiency turbocharger engine, driving a fixed pitch propeller through a forged steel shafting.

The propeller shaft shall be turned clockwise viewed from aft of M.E. going ahead.

The central cooling F.W. system shall be provided.

The electric generating plant shall consist of three (3) sets of main diesel generators and one (1) set of emergency diesel generator.

The main engine shall be remotely controlled from the engine control room and wheelhouse and local control console and navigation bridge deck wings.

The steam generating plant shall be of one (1) set of composite boiler with exhaust gas section and oil fired section.

Open loop scrubber to be provided to satisfy the requirements of Global Sulfur Cap 2020

An air-conditioned sound proof engine control room shall be provided in engine room and equipped with engine control device, main switchboard and other necessary monitoring equipment.

The ECR should have 2 Glass windows of the size 2200x600mm .

The machinery and equipment in engine room shall be designed, constructed, installed or supplied according to the manufacturers and/or the Builder’s standard in compliance with the requirement of the Class and the Regulatory body as far as they are concerned.

During design and construction of engine room, particular attention to be paid to ensure
that adequate space is provided for operation and maintenance of the mechanisms or
other arrangements.

Ensure that adequate space is provided for maintenance of the piping systems as far as
possible.

Eye plates to be provided for machinery equipment and pumps more than 50kg in weight.

SI unit and bar unit to be adopted for design, construction of machinery and measuring units, such as power, pressure, temperature and flow etc.

Valves and cocks shall be according to GB, CBM, CB, JIS standard. Pipe dimension and material are of Chinese standard except those pipes/valves attached on imported equipment, which are to be Maker's standard. The connecting diameter of flange is according to ISO standard except those flanges attached on imported equipment, which are to be Maker's standard.

If there are minus differences between this Specification and actual machinery in respect of the description of machinery construction and piping systems, it to be in accordance with machinery Maker’s standard.

The materials and accessories of the machine and the equipment shall be in accordance with the requirements of Classification Society and Maker’s standard.

6.1.2	INSPECTION AND TEST

All inspection and tests for machinery shall be carried out in accordance with the Classification Society and Maker's inspection standard. The builder should notice the buyer,  if the Buyer's representative cannot attend due to his own cause, these inspections and tests shall be entrusted to surveyor of the Classification Society and/or Builder's inspector, results of which shall be regarded as acceptance by the Buyer.

Data shall be recorded during these tests and the results shall be submitted to the Buyer. However, if the Buyer's representative cannot attend due to his own cause, these inspections and tests shall be entrusted to surveyor of the Classification Society and/or Builder's inspector, results of which shall be regarded as acceptance by the Buyer.

There will be a "items for inspection and test" document to be confirmed with Buyer in detail design stage.

The Buyers representative will have the right to postpone any shop test if test procedures are not submitted in advance.

6.1.2.1	Main Engine Shop Test

The main engine shop test shall be performed according to the requirement of the classification society and Maker’s standard.

The effective output of main engine shall be determined at various loads by means of hydraulic dynamometer.

Shop test shall be carried out with MDO in the presence of builder, Buyer Representation(s) and Surveyor(s) of the class.

The relevant pressure, temperature, and revolution, etc., for all test condition shall be measurement and record.

(1)	Load Test

| Load | 25% | 50% | 75% | CSR | 100% | 110% |
| --- | --- | --- | --- | --- | --- | --- |
| Hours | 1/2 | 1/2 | 1/2 | 1 | 1 | 1/2 |

(2)	Governor function test at full load

(3)	Fuel oil consumption measurement at all loads for reference (only CSR to be of guarantee point)

(4)	Starting test and reversing test

(5)	Minimum revolution test at no load (approx. 5 minutes)

(6)	Astern running test at no load (min. 5 minutes)

(7)	One cylinder cut off test (test load according to Maker's standard only for proto engine)

(8)	Safety device confirmation before M/E shop test

(9) Overhaul inspections for necessary portions after shop test shall be carried out in accordance with Maker's usual practice and class requirement. One cylinder shall be dismantled completely for inspection.

6.1.2.2	Generator Engine Shop Trial

The Generator engine shop trial will be done according to the requirement of the classification society and Maker’s standard.

The following tests shall be performed by using equipment, lubricating oil supplied by the manufacturer coupled together with the alternator.

(1)	Load Test

| Load | 25% | 50% | 75% | 100% | 110% |
| --- | --- | --- | --- | --- | --- |
| Hours | 1/2 | 1/2 | 1/2 | 1 | 1/2 |

(2)	Starting test at no load

(3)	Emergency trip test and safety device test

(4)	Governor test

(5)	Fuel oil consumption measurement at 100% load

(6)	Torsion vibration test recorded to be submitted for the first vessel

(7)	Voltage regulation test

(8)	Overhaul inspection shall be carried out in accordance with Maker's usual practice and class requirement. Overhaul of one cylinder unit per engine to be done as well as inspection of all cams and gear wheel drive.

Note: During test on board of Generator, PF (power factor) = 1.0 applied as test load.

6.1.2.3	Other Aux. Machinery

The air compressors, pumps, ventilation fans, and other auxiliary machineries to be carefully tested in respect of performance and function at Maker's shop (if any) or on board in compliance with the requirement of Classification Society and Maker’s standard (if any).

For the machinery having the same capacity, performance test shall be carried out for only one set.

Above mentioned test shall be not applied to spare parts (if any), which the dimension test only shall be applied to.

6.1.3	FUEL OIL

HFO and LSHFO/ULSFO and MGO shall be on board as fuel.

The main engine shall be capable of burning HFO of up to viscosity of 700cSt/50˚C, max. density of 1,010kg/m3 at 15˚C, and  MGO shall be used prior stopping for long time only.

The fuel oil piping system and equipment shall be designed with fuel oil viscosity of 380 cSt/50˚C.

The main generator engines shall burn HFO and LSHFO/ULSFO same as for main engine including the running condition of start, stop, MGO will be used when the engine is expected to be inactive for a prolonged period with cold engine.

The emergency diesel generator shall be designed to burn MGO.

Same heavy fuel oil used in the main engine shall be used for oil fired section of the composite boiler, and cold starting of the boiler shall burn MGO.

However, when the ship is navigating to enter into a SOx emission control area, the burning fuel oil for engines and boiler shall be changed over to MGO or ULSFO (sulphur content 0.1 % m/m).

The change over from HFO to LSHFO/ULSFO or  MGO or vice versa will be automatic type.

MGO cooler will be cooled by LT cooling fresh water.

MGO will not be purified, and MDO will be used instead of MGO for MGO system function test.

Fuel standard as follow:

HFO: ISO 8217-2017(E), Class RMK380, Viscosity is 380 cSt/50oC (Sulfur content < 3.5% m/m), density 1,010 kg/m3 at 15 oC.

MGO: ISO 8217-2017(E), Class DMZ, Viscosity is 3~6 cSt/40oC (Sulfur content <0.1% m/m)

LSHFO (Sulfur content < 0.5% m/m, viscosity 10~70 cSt/50oC) is new hybrid fuel type that don’t fit into the ISO 8217 standard, and will be based on the available specification data from the fuel supplier.

ULSFO (Sulfur content < 0.1% m/m, viscosity 10~60 cSt/50oC) is new hybrid fuel type that don’t fit into the ISO 8217 standard, and will be based on the available specification data from the fuel supplier.

6.1.4	DESIGN CONDITION

The main engine, generator engines and the aux. equipment including the shafting shall be so designed that the main engine is satisfactorily operated at the maximum continuous rating at sea water temperature of 32C, L.T.F.W. temperature of 36C, engine room ambient air temperature of 45C, barometric pressure of 0.1MPa and relative humidity of 60%.

The basic ideas or design condition in planning the machinery plant are to be as numerated below.

6.1.4.1	Shafting and Propeller

The diameter of the shafts shall be determined by the Rule's requirements and to have margin refer to item6.2.3.

The propeller design refers to “Chapter 1 General paragraph 1.2.7.2”.

6.1.4.2	Machinery Installation Aspects

The supporting chocks of main engine shall be made of epoxy chocks.

The generating engine shall be direct coupled to and mounted on a common bed with the generator. The common bed shall be resiliently mounted to ship’s structure as manufacture’s recommendation.

The steel liner or steel sheet shall be used for pumps and other aux. equipment.

6.1.4.3	Electric Power

Refer to “Electric Chapter”.

6.1.4.4	Steam

Necessary steam for ship's service shall be as follows.

Normal sea service	Exh. gas section of the composite boiler

and oil fired section of the composite boiler if necessory.

Harbor in/out operation	Oil fired section of the composite boiler

Port	Oil fired section of the composite boiler

6.1.4.5	Compressed Air

Two (2) main air reservoirs shall be able to start main engine consecutively at least twelve(12) times without replenishment, and total capacity of two (2) main air compressors shall be capable of recharging two (2) main air reservoirs within one (1) hour. The main air reservoir shall be also able to start generator engine normally.

One (1) aux. air reservoir for generator engines shall be provided and charged through the main air line from main air reservoir normally, while this reservoir shall be charged by the emergency air compressor which is electrically driven type in case of dead ship condition. The emergency air compressor motor shall be fed from the emergency switchboard.

6.1.4.6	Heat Exchanger

(1)	The coolers for main engine shall be designed on the basis of SMCR.

(2)	The fouling margin for plate type cooler shall be of 10%.

- The fouling margin for shell-tube type heat exchangers to be of 15%.

6.1.4.7	Purifier

(1)	For fuel oil

Two (2) self-cleaning type purifiers shall be provided for HFO/LSHFO cleaning. One (1) for working and the other as stand-by. The purifiers can handle fuel density of 1,010 kg/m3.

(2)	For lube oil

One (1) self-cleaning type lube oil purifier shall be provided for main engine lube oil system.

One (1) self-cleaning type lube oil purifier shall be provided for generator engines.

Lub. purifier must be possible to switch purifiers to Main Engine and Aux Engine sump.

6.1.4.8    Pump

Where two (2) pumps for M/E, G/E and boiler etc., service provided in one (1) system, one (1) shall be sufficient to handle the system and the other service as stand-by.

The specified capacity and motor output of rotary positive displacement pumps shall be designed on the basis of following viscosity.

|  | Capacity cSt | Motor output cSt |
| --- | --- | --- |
| Fuel oil circulating and supply pump | 25.8 | 1000 |
| HFO,LSHFO/ULSFO transfer pump | 25.8 | 1,000 |
| MGO transfer pump | 3.0 | 100 |
| Main lubricating oil pump | 25.8 | 260 |
| Lube oil transfer pump | 25.8 | 1,000 |

6.2	PRINCIPAL PARTICULARS OF MACHINERY

6.2.1	NOx EMISSION SOLUTION

Only IMO Tier II NOx emission limitations.

6.2.2	MAIN ENGINE

| No. of set | : | One (1) |
| --- | --- | --- |
| Type |  | MAN B&W6S60ME-C10.5,Tier II, two (2) stroke, single acting， direct reversible, high efficiency turbocharger type |
| NMCR | : | 14,940 kW x 105 r/min |
| SMCR | : | 10,380 kW x 96 r/min |
| CSR (90%SMCR) | : | 9,342 kW x 92.7 r/min |
| No. of cylinder | : | 6 |
| Cylinder bore | : | 600 mm |
| Stroke | : | 2,400 mm |
| Tuning method | : | Low-Load |
| NOx emission compliance | : | IMO Tier II compliant |
| BSFC | : | Abt. 159.0 g/kW.h +5% at CSR based on the use of a fuel oil with lower calorific value of 42,700 kJ/kg at shop test under ISO ambient condition at Tier II mode(ME back pressure max. 60mbar) at shop test. |
| Cooling type | : | Central fresh water cooling |
| Fuel | : | HFO,LSHFO/ULSFO,MGO |

6.2.3	SHAFTING AND PROPELLER

| One (1) -Intermediate shaft Material: Diameter: |  | Forged steel Abt. 3mm above the Classification Society Rules, subject to torsion vibration calculation, and bearing part to be of 5mm over size with at least 15 mm beyond the ends of bearing. |
| --- | --- | --- |
| One (1) -Propeller shaft Material: Diameter: |  | Forged steel with integral flange at fore end. Abt. 5 mm above the Classification Society Rules, subject to torsion vibration calculation |
| One (1) - Intermediate shaft bearing |  | Cast iron or cast steel construction with removable steel half shells (upper and lower part) lined with white metal. (Number may be adjusted as the result of shafting line alignment calculation.) |
| One (1) -Stern tube assembly |  | The stern tube shall be of forge steel combined with steel plate tube, to be installed on hull structure by epoxy resin. |
| One (1) -Stern tube seal |  | Oil seal at forward (two rings seal) and air seal (four rings seal) for aft end with rope guard and net cutter . A split spacer ring of aft seal to be fitted for purpose of removal so that seal nest can be moved on shaft liner. |
| One (1) -Stern tube bearing (Forward and aft.) |  | Cast iron bearing bush lined with white metal. |

One (1)	-Propeller

Type:	Solid, aerodynamic section, keyless, right handed looking from aft.

Material:	Nickel aluminum bronze of suitable diameter and pitch.

One (1)	-Fair water cap

Material:	Ni Al bronze

One (1)	-Propeller Nut	Conventional type made of forged steel with hydraulic mounting.

Neither spare propeller nor spare propeller shaft to be provided.

6.2.4	ELECTRIC GENERATING PLANT

(1)	Generator engine

No. of set	:	Three (3)

Type	:	4-stroke, single acting trunk piston medium speed engine

Output	:	As per Maker’s standard

Revolution	:	Max. 900 r/min

Generator	:	abt.1,250kWe, AC,450V, 3¢, 60 Hz (drip-proof Brushless type)

Fuel	:	HFO, LSHFO/ULSFO,MGO

(2)	Emergency generator engine

No. of set	:	One (1)

Type	:	Four cycle, vertical, single acting, radiator cooled type		diesel engine.

Output	:	As per Maker’s recommendation

Revolution	:	1,800 r/min

Alternator	:	Abt.150 kWe, AC, 450V, 3¢, 60 Hz

Starting system	:	By battery and hydraulic or equivalent

Fuel	: 	MGO

6.2.5	STEAM GENERATING PLANT

Composite boiler

No. of set	:	One (1)

Type	:	Cylindrical vertical type with pressure atomized burner and feed water regulator (on/off control).

Evaporation

Oil fired section	:	Abt.1,800kg/h, final capacity to be decided acc. to steam balance.

Exhaust gas section	:		abt. 1,300 kg/h at CSR of M/E under ISO condition.

Steam state	:	0.7 MPa (working) saturated

Feed water temperature	:	80oC

Fuel	:	HFO, LSHFO/ULSFO,MGO

The exhaust gas section of the composite boiler also serves as M/E exhaust gas silencer and the noise attenuation shall be as per Maker’s standard.

Final capacity to be decided acc. to final selected maker’s calculation.

6.2.6	COMPRESSED AIR SYSTEM

Main air compressor

No. of set	: Three (3)

Type	: Motor driven, two or three stages, fresh water cooled

Capacity	: 180m3/h (F.A.) x 3.0 MPa

Revolution	: As per Maker’s standard.

Emergency air compressor

No. of set	: One (1)

Type	: Motor driven, two or three stages, air cooled

Capacity	: 30 m3/h (F.A.) x 3.0 MPa

Revolution	: As per Maker’s standard.

Service air compressor

No. of set	: One (1)

Type	: Motor driven, screw type, air cooled

Capacity	: 100m3/h (F.A.) x 0.7 MPa

Revolution	: As per Maker’s standard.

Main air reservoir

No. of set	: Two (2)

Type	: Cylindrical all welded

Capacity	: 5.0 m3 x 3.0 MPa

Service air reservoir

No. of set	: One (1)

Type	: Cylindrical all welded

Capacity	: 1.0 m3 x 0.7 MPa

Aux. air reservoir

No. of set	: One (1)

Type	: Cylindrical all welded

Capacity	: 0.25m3x 3.0 MPa

Control air reservoir

No. of set	: One (1)

Type	: Cylindrical all welded

Capacity	: 0.25m3x 0.7 MPa

Control air dehydrator

No. of set	: One (1) setType	: Refrigerant type (R134a or R407C will be used and comply with the class notation.)

Capacity	: abt. 50m3 /h (F.A.)

6.2.7	COOLING WATER SYSTEM

Main cooling S.W pump

No. of set	: Three (3) each 50% (two working, one for stand-by)

Type	: M.D. vert. centrifugal,

Capacity	: abt.420 m3/h (depending on final heat balance)

Total head	: 0.25 MPa

M/E jacket cooling F.W. pump

No. of set	: Two (2)

Type	: M.D. vert. centrifugal

Capacity	: abt.75 m3/h

Total head	: 0.30 MPa,  as per ME’s recommendation

L.T. cooling F.W. pump

No. of set	: Three (3) each 50% (two working, one for stand-by)

Type	: M.D. vert. centrifugal

Capacity	: abt. 320 m3/h (depending on final heat balance)

Total head	: 0.35 MPa

M/E Jacket cooling F. W. cooler

No. of set	: One (1)

Type	: plate type, stainless steel

Heat dissipation	:. abt.1,370 kW, as per ME’s recommendation

Central F.W. cooler

No. of set	: Two (2) (Each 60% of total load)

Type	: plate type, to be made of titanium and equipped with in line filter at SW side

Heat dissipation	: depending on final heat balance

Note: the back washing piping for central coolers sea water side shall be arranged.

Main engine jacket F.W. preheating unit

No. of set	: One (1)

Type	: As per Maker’s recommendation, to be provided with removable tube bundle

Capacity	: As per Maker’s recommendation

Main engine jacket F.W. preheating pump

No. of set	: One (1)

Type	: M.D. horiz. centrifugal

Capacity	: 10 m3/h

Total head	: 0.3 MPa

G/E H.T.F.W. preheating unit

No. of set	: One (1)

Type		: As per Maker’s recommendation

Capacity	: As per Maker’s recommendation

6.2.8	FEED AND DRAIN SYSTEM

Feed water pump

No. of set	: Two (2)

Type	: M.D. centrifugal

Capacity	: As per Maker’s recommendation

Total head	: As per Maker’s recommendation

Atmospheric condenser

No. of set	: One (1)

Type	: Shell tube type, Tube Bundle needs to be remove-able and

LT Cooling Fresh Water cooled

Cooling area	: to be capable of condensing the steam from the exhaust gas boiler at main engine CSR under ISO reference condition

Cleanliness factor	: 85%

6.2.9	FUEL SYSTEM

M/E & G/E F.O. supply module unit

No. of set	: One (1)

Capacity	: According to the requirement of M/E at SMCR and two(2) sets G/E at max output

Including fuel oil supply pumps, automatic filter and bypass filter (35 microns nominal), de-aeration tank, fuel oil booster pumps, steam heaters, viscometer, MGO cooler (cooled by L.T.F.W.), automatic  change-over device between HFO and LSHFO/ULSFO and MGO, and other accessories, the supply/booster pumps and viscosity control to be suitable to MGO. One (1) MGO pump shall be fitted for emergency.

HFO, LSHFO/ULSFO transfer pump

No. of set	: One (1)

Type	: M.D. screw

Capacity	: 30 m3/h

Total press.	: 0.4MPa

MGO transfer pump

No. of set	: One (1)

Type	: M.D. screw

Capacity	: 30 m3/h

Total press.	: 0.4MPa

HFO /LSHFO/ULSFO purifier with steam heater and electric motor driven oil feed pump

No. of set	: Two (2) (one working, one for stand-by)

Type	: Automatic, self-cleaning, no gravity ring

Effective capacity	: abt. 3,000 l/h (based on 380 cSt/50C, max. density of 1,010kg/m3)

The fuel oil supply unit for composite boiler to be provided and recommended by composite boiler Maker.

6.2.10	LUBRICATING OIL SYSTEM

Main lubricating oil pump

No. of set	: Two (2)

Type	: M.D. vert. centrifugal tank mounting, deep well type

Capacity	: abt. 320 m3/h (if lub. oil to be used for auto. filter back washing, back washing capacity shall be added)

Total head	: 0.62 MPa

Lub. oil transfer pump

No. of set	: One (1)

Type	: M.D. vert. screw

Capacity	: 5m3/h

Total press	: 0.35MPa

M/E L.O. purifier with steam heater and electric motor driven oil feed pump

No. of set	:	One (1)

Type	: Automatic, self-cleaning

Effective capacity	: abt.2,030 l/h (SAE30)

.

G/E L.O. purifier with steam heater and electric motor driven oil feed pump

No. of set	:	One (1)

Type	: Automatic, self-cleaning

Effective capacity	: abt. 2,030 l/h (SAE30, and the capacity of SEA40 accordign to the maker standards)

M/E Lubricating oil cooler

No. of set	: One (1)

Type	: Plate type, stainless steel

Heat dissipation	: 1,020 kW

Cylinder L.O. transfer pump

No. of set	: One (1)

Type	: M.D. horiz. screw

Capacity	: 1m3/h

Total press.	: 0.3MPa

Lub. oil fine filter with manual by-pass filter

No. of set	: One (1)

Type	: Auto. back wash

Absolute fineness	: 50 micron

Capacity	: 320m3/h

Working press.	: 0.40 MPa

Gen. engine L.O. cooler attached to G/E

No. of set	: Three (3)

Capacity	: As per Maker’s recommendation

Gen. engine L.O. filter attached to G/E

No. of set	: Three (3)

Capacity	: As per Maker’s recommendation

6.2.11	FIRE, BILGE, BALLAST AND GENERAL SERVICE SYSTEM

Fire, bilge & G.S. pump

No. of set	: Two (2)

Type	: M.D. vert. centrifugal with self-priming

Capacity	: 240/120 m3/h

Total head	: 0.4/0.9MPa

Emergency fire pump (To be located in bow thruster room)

No. of set	: One (1)

Type	: M.D. vert. centrifugal with self-priming

Capacity	: 72 m3/h

Total head	: 0.8 MPa

Ballast pump

No. of set	: Two (2)

Type	: M.D. vert. centrifugal with self-priming

Capacity	: abt. 350m3/h

Total head	: 0.32MPa

Ballast treatment plant

No. of set	: One (1)

Type	: Filtration + UV or electrolysis(USCG + IMO approved type)

Capacity	: 350 m3/h

Anti-heeling pump

No. of set	: One (1)

Type	: M.D. reversible propeller type

Capacity	: abt. 350 m3/h

Total head	: 0.15 MPa

Ballast stripping eductor

No of set	: One (1)

Capacity	: 60 m3/h

Suction lift	: 4 m WG

Discharge pressure	: 0.10 MPa

Daily bilge pump

No. of set	: One (1)

Type	: M.D. Single screw

Capacity	: 5 m3/h

Total press.	: 0.35 MPa

Sludge pump

No. of set	: One (1)

Type	: M.D. Single screw

Working temperature	: 80 deg.C

Capacity	: 10 m3/h

Total pressure	: 0.4 MPa

Oily bilge separator

No. of set	: One (1)

Type	: IMO approved type, to meet the requirement of Res. MEPC 107 (49), auto. discharge with pump, oil discharge content monitor, high ppm alarm and steam coil inside (or elec. heating according to Maker’s standard).

Capacity	: 5 m3/h, 15 ppm

Note: Oil Bilge separator must be provided with ‘No Flow’ alarm and feed from GPS for recording position during discharge.

Chain locker bilge eductor

No. of set	: One (1)

Capacity	: 20 m3/h

Suction lift	: 5 m WG

Discharge pressure	: 0.1 MPa

Bow thruster & emergency fire pump room bilge eductor

No. of set	: One (1)

Capacity	: 10 m3/h

Suction lift	: 5 m WG

Discharge pressure	: 0.1 Mpa

Fixed water mist system

No. of set	: One (1)

Capacity	: As per Maker’s recommendation

Dangerous goods bilge eductor

No. of set	: One (1)

Capacity	: 25 m3/h

Suction lift	: 5 m WG

Discharge pressure	: 0.17 MPa

Clean Drain Tank pump

No. of set	: One (1)

Type	: M.D. centrifugal with self-priming

Capacity	: abt. 4 m3/h

Total head	: 0.25 MPa

6.2.12	FRESH WATER GENERATOR

F.W generator

No. of set	: One (1)

Type	: Plate types, vacuum distilling

Capacity	: abt. 20 t/d (at CSR of M/E under ISO reference condition) Heating medium	: M/E jacket cooling fresh water

6.2.13	FRESH WATER SERVICE SYSTEM

Fresh water pump

No. of set	: Two (2)

Type	: M.D. centrifugal

Capacity	: 10 m3/h

Total head	: 0.6 MPa

Hot water circulating pump

No. of set	: One (1)

Type	: M.D centrifugal

Capacity	: 2m3/h

Total head	: 0.15 MPa

Fresh water hydrophore tank

No. of set	: One (1)

Capacity	: 1000 l

Pressure	: 0.6 MPa

Material	: Mild steel

Calorifier

No. of set	: One (1)

Type	:	shell & tube. using steam & electric heating (electric heating capacity abt. 18 kW)

Temperature in/out	: 10-70C

Capacity	: 500 l

Material	: Mild steel

Sterilizer

No. of set	: One (1)

Type	: Ultra violet ray

Capacity	: 1,500 l/h

F.W. mineralizing unit

No. of set	: One (1)

Type	: Dolomite filter

Capacity	: 1.5 m3/h

6.2.14	scrubber system

Scrubber

No. of set              : One (1)

Type                  : U-Type, Open loop

Capacity               : 1x ME with 100%SMCR + 2 x AE with each 85%MCR

Maximum exhaust gas back

pressure along scrubber(mmWC)   :150

Material               : According to the maker’s standard

Minimum alkalinity of sea water : 2200μmol/l

Sea water pumps

No. of set              : Three (3) (two working ,one standby)

Capacity               : According to the maker’s standard.

Seal air fan

No. of set              : Two(2) (One for standby)

Capacity               :According to the maker’s standard

6.2.15	FANS

Engine room ventilating fan

No. of set	: Four (4)

Two (2) of them to be reversible

Type	: Axial flow, split type,

Capacity	: abt.1,300 m3/min

Static pressure	: 600 Pa

Capacity shall be according to calculation which shall be based on ISO 8861 where a temperature rises of 12.5deg. C shall be considered.

Oil purifier area exhaust fan

No. of set	: One (1)

Type	: Centrifugal

Capacity	: 200m3/min.

Static pressure	: 500 Pa

ER workshop exhaust fan

No of set	: One (1)

Type	: Centrifugal

Capacity	: 50 m3/min

Static pressure	: 400 Pa

6.2.16	WORKSHOP EQUIPMENT

Lathe

No. of set	: One (1)

Type	: Motor driven.

Center span	: 1,000 mm

Max. drilling dia.	: 38 mm

Table driller for electrical workshop

No. of set	: One (1)

Type	: Motor driven, table type

Max. drilling dia	: 20 mm

Grinder

No. of set	: One (1)

Type	: Double head, dry type, electric motor driven.

Stone dia.	: abt. 200 mm

Electric welder with voltage reduce device

No. of set	: One (1)

Type	: A.C. arc welding shock proof type

Capacity	: 200 A

Note: The socket for electric welder to be installed in bosun store.

Gas welder compatible unit

No. of set	: One (1)

Type	: 2 x 40 l Oxygen bottles.

1 x 40 l Acetylene bottle

Hose length	: 50m x 2

Fuel oil injector testing bed

No. of set	: One (1)

6.2.17	MISCELLANEOUS

Incinerator

No. of set	: One (1)

Type	: Solid waste and waste oil burning, IMO approved type

Capacity	: abt. 400,000kcal

Fuel	: MGO

Note： Unit must be able to load Solid waste while burning sludge and oil waste.

Mechanical or compressor air mixing unit to be provided for the attached waste oil tank.

Sewage treatment plant

No. of set	: One (1)

Type	:	Biological and gravity type, according to the MEPC regulation

Capacity	: 24 P

Sewage pump

No. of set	: One (1)

Type	: M.D. close coupled with non-clogging system

Capacity	: 10m3/h

Discharge pressure	: 0.15MPa

Main engine overhauling crane

No. of set	: One (1)

Type	: hoisting, traveling and traversing by motor driven.

Capacity	: total 4.0 t

E.C.R. unit cooler

No. of set	: One (1)

Type	:	Packaged type

Capacity	: Abt. 22 kW

Cooling media	: R407C

Galley unit cooler

No. of set	: One (1)

Type	: Packaged type

Capacity	: abt.17 kW

Cooling media	: R407C

Marine growth preventing system fitted on main SW strainers

No. of set	: One (1)

Type	: Pole ionizing anode.

Capacity	: bases on total capacity of two (2) main cooling SW pumps, one (1) FW generator SW pump and scrubber sea water pump.

Anodal life  	: Five (5) years continuous running (for each sea chest in E/R)

Shut off valves control device for oil tanks

No. of set	: One (1)

Type	: Air operated

Work pressure	: As per Maker’s recommendation

Valves remote control system

No. of set	: One (1)

Type	: pneumatic

Work pressure	: As per Maker’s recommendation

Cold water fountain in engine room

: Refer to Chapter 6  Hull Piping

6.2.18	TANKS IN ENGINE ROOM

| Tank | Set | Capacity (m3) | Remarks |
| --- | --- | --- | --- |
| HFO, ULSFO/LSHFO tank* |  | refer to G Chapt. | HL, RL,SD,HC,HT |
| HFO, ULSFO/LSHFO settling tank ** | 2 | Abt. 40 | I, HC,LL, AT, ML, HT, HL, RL,HSP,LST |
| HFO, ULSFO/LSHFO service tank ** | 2 | Abt. 40 | I, HC, LL, AT, ML, HT,RL |
| MGO tank |  | refer to G Chapt. | HL, RL, SD |
| MGO service tank | 1 | Abt. 30 | RL, LL, PL, |
| Emergency generator M.G.O. service tank | 1 | Abt. 1.0 | PL, LL |
| MGO tank for incinerator | 1 | Abt. 0.6 | PL, LL |
| Waste oil tank for incinerator | 1 | Acc.to Maker’s std. | HC, I, FL,HL, AT |
| FO overflow tank | 1 | Abt. 40 | HC, HL, SD, RL |
| FO drain tank | 1 | Abt. 1 | HC, HL, SD, |
| M/E lub. oil storage tank | 1 | Abt. 40 | ML |
| M/E lub. oil settling tank | 1 | Abt. 21 | HC, ML |
| M/E lub. oil sump tank | 1 | Abt. 21 | HC, SP, LL, RL |
| G/E lub. oil storage tank | 1 | Abt. 19 | ML |
| G/E lub. oil settling tank | 1 | Abt. 13 | ML |
| L.TBN Cyl. Oil storage tank | 1 | Abt. 35 | PL |
| H.TBN Cyl. Oil service tank | 1 | Abt. 35 | PL |
| Waste oil tank | 1 | Abt. 10 | HL,SD,HC |
| FO purifier’s sludge tank | 1 | Abt. 4 | S, HC, HL, SD |
| LO purifier’s sludge tank | 1 | Abt. 4 | HC, HL, SD. |
| Sludge tank | 1 | Abt. 30 | S, HC, HL, SD, |
| H.T. F.W. expansion tank | 1 | Abt. 1.25 | GL, LL |
| L.T. F.W. expansion tank | 1 | Abt. 1.25 | GL, LL |
| Bilge water tank | 1 | Abt. 32 | HC, HL, SD |
| Sewage holding tank | 1 | Abt. 32 | HL,SD |

Abbreviations:

HC	: steam heating coil

S	: steam blow

I	: insulated

ML	: magnetic float level gauge

SP	: self-powered level gauge

GL	: glass level gauge

FL	: float level gauge

PL	: plain glass level gauge

SD	: sounding pipe

HL	: high level alarm

LL	: low level alarm

AT	: auto. temperature regulating

HT	 : high temperature alarm

RL	 : level remote measuring

HSP	 : High level stop

LST	: Low level start

Notes:

(1)	The final capacity for some pumps and aux. equipment serving M/E, G/E and boiler and so on such as cooling water pump, and plate cooler etc. may be adjusted after detail heat balance.

(2)	The final capacity for the tanks may be adjusted after determination of hull line and position of the tank.

(3)	The total pressure for screw pump and gear pump to be contained 0.05MPa suction head.

(4)	AT with mark “*” to be arranged for HFO and LSHFO/ULSFO bunker tanks within cargo hold area.

(5)	The HFO tanks with mark “**” to be meant the tanks to have inclined bottom.

(6)	All tanks to be made of welded steel plate and well secured to the hull. Each tank shall be provided with necessary fittings such as inlet and outlet connections, drain, air vent, manhole or hand-hole, overflow, thermometer socket, steam blow-out connection, etc. all depending on the purposes for which they are intended.

(7)	The M/E L.O. sump tank shall be fitted with heating coil only around the suction bell mouth.

(8)	All of the oil tanks shall be provided with drip tray, as practicable.

6.3	MAIN ENGINE

6.3.1	GENERAL

The main engine shall be designed and manufactured under the license from MAN B&M. The particulars of main engine shall be as specified in Paragraph 6.2.1.

The direction of engine revolution shall be clockwise, looking from aft.

The main engine shall be remotely controlled from wheelhouse, navigation bridge deck wings and engine control room in the engine room.

The main engine shall be temporarily controlled from local operating console. The main engine output shall be valid at following conditions:

Atmospheric pressure:	0.1 MPa

Ambient temperature in E/R:	45oC

Sea water temperature:	32oC

L.T. fresh water temperature:	36oC

Relative humidity:	60%

The construction and materials of M/E shall be in accordance with Maker’s standard and to be complied with the requirement of Classification Society.

The main engine shall be completed with necessary fittings and accessories to develop the maximum continuous rating for propelling as described in Paragraph 6.3.2.

6.3.2	FITTINGS AND ACCESSORIES

The engine shall be completed with necessary fittings and accessories according to main engine Maker’s standard and the main body shall be included as follows:

1- Set of high efficiency exhaust turbo-charger.

2- Sets of electronically controlled auxiliary blowers complete with starters for continuous running according to maker’s standard. Provide the lub. oil pump running to interlocked with the ME auxiliary blower.

1- Set of air cooler with cleaning and drain pipes.

1- Set of turning gear with electric motor, starter and portable switch box with capture cable of 20m and pneumatic interlocking apparatus.

1- Air starting system.

1- Set of steam tracer piping for F.O. pipes on the engine.

1- Set of cleaning device for turbo charger. (According to turbo charger maker's standard)

1- Set of brackets for the engine platforms with gratings, handling and steps.

1- Set of steam connection for fire extinguish system of scavenging air space.

1- Set of pressure regulating valves for fuel oil return.

1-Set of oil mist detecting device.

1-Set of drain box for fuel oil leakage alarm

1- Set of integrated axial damper mounted at the forward end of the crankshaft

1- 2nd order moment compensator (if necessary), and the type of the compensator shall be according to the M/E maker 's standard.

1- Torsional vibration damper (if necessary subject to TV calculation and Rules)

1- Set of friction type engine stays

1- Set of bearing wear monitoring

1- Set of Water in oil detection

1- Spare CCUs to be provided

1- The M/E hydraulic power lubricating oil system for FIVA and bearing lubricating system must be independent circuits and not connected to same source

Maker to provide data analysis support during guarantee period.

Turbo charger selection will be confirmed with Owner.

ME components (rings, piston skirts, liners, exhaust valves, air cooler etc.) to be according to licensor's latest design recommendation.

The spare of the main engine shall be according to maker standard and Class requirement.

Revolution indicator and revolution counter (refer to Chapter 7, Automation 7.1.4.3.4)

Remark:  If above fitting and accessories of main engine is not consistent with main engine standard scope range, maker standard shall prevail.

6.3.3	INSTALLATION

Epoxy resin chock shall be used for installation of the main engine.

6.4	SHAFTING AND PROPELLER

6.4.1	GENERAL

The shafting shall consist of one (1) intermediate shaft and one (1) propeller shaft, one (1) intermediate bearing, one (1) stern tube and sealing. The intermediate shaft shall be fitted with oil-lubricated plain bearing and propeller shaft shall be of oil-lubricated white metal bearing. Thrust shaft and thrust bearing shall be incorporated in main engine.

The shaft diameter, propeller diameter and the number of blades and other details shall be determined and designed in consideration of the main engine torsion vibration as well as hull vibration.

If the barred speed range exists in the operating speed range, the barred speed range shall be remarked on the revolution indicator. Barred speed range with automatic quick by-pass arrangement shall be allowed at nonessential range of shaft revolution.

The calculation sheets regarding the torsional vibration shall be submitted to Classification Society for approval.

The torsional vibration shall be measured during sea trial for the first vessel.

The length of propeller shaft shall be so designed that the shaft can be withdrawn and taken in from engine room side without removal of the main engine.

Spare propeller, spare propeller shaft and intermediate shaft not to be supplied.

6.4.2	INTERMEDIATE SHAFT

The intermediate shaft shall be made of forged high quality carbon steel with integral flanges at both ends, and coupled together with each shaft with parallel forged steel reamer bolts.

The shaft shall be machined all over and finished brightly in way of bearing where the diameter is increased abt.5mm. The shafting shall be electrically earthen to the hull; a shaft earthing device shall be fitted to the intermediate shaft.

6.4.3	PROPELLER SHAFT

The propeller shaft shall be made of forged high quality carbon steel with a solid flange coupling on fore end. The propeller shaft shall be smoothly machined all over and finished brightly in way of bearings where the diameter is increased abt. 5mm.

The after portion of the shaft shall be machined to 1/20 tap to fit the propeller hub and the end of the shaft shall be left-hand threaded to suit a propeller shaft nut.

Removal of the propeller shaft from the Vessel shall be carried out via a temporary opening cut in the shell plating when overhauling and the location shall be clearly marked. The location of temporary opening shall be given on approval drawing.

Eye plates for lifting propeller and rudder to be fitted at stern.

6.4.4	COUPLING

The diameter of the coupling bolts and the thickness of the coupling flanges of intermediate shaft and propeller shaft are to comply with the requirements of the Classification Society.

The fillet radius at the base of flanges is to comply with the requirements of the Classification Society.

6.4.5	INTERMEDIATE SHAFT BEARING

The intermediate shaft bearing shall be of the single oil disc type and of cast iron or cast steel pedestal, upper and lower half shells lined with white metal.

The bearing shall be provided with ample oil reservoir and cooling water jacket inside the pedestal, with end baffle to prevent loss of oil at both ends of bearing.

The bearing shall be central LT fresh-water cooled. And a bearing temperature sensor shall be provided and connected to ECR.

For checking the oil levels in the bearing housing, the bearing shall be fitted with a sounding bar.

The number of bearing may be adjusted at detail design stage as per final shaft alignment.

6.4.6	STERN TUBE AND STERN TUBE BEARING

6.4.6.1	Stern tube assembly

The stern tube shall be of forge steel combined with steel plate tube and be secured by epoxy resin to the stern frame boss and bulkhead of the stern tube cooling water tank.

A steel plate rope guard shall be provided at the after end of the stern frame hub.

6.4.6.2	Stern Tube Bearing

There are two (2) bearings in the stern tube, one at the stern frame hub and another bearing at the forward end of the stern tube.

The bearing shells shall be of cast iron lined with white metal and of oil retaining type.

The length of after stern tube bush is to comply with the requirements of Classification Society.

Two (2) temperature sensors shall be fitted on the aft. bearing to indicate the bearing temperature, one of them as stand-by.

The wear-down measuring gauge shall be provided.

The temperature sensors to be drawn out and replaced from engine room side.                               Two remote reading temp. sensors for aft stern tube bearing to be installed. One of them to be as standby.

6.4.6.3	Stern Tube Seal

The oil seal assembly shall be fitted at forward end of the stern tube and the air seal assembly shall be fitted at aft end of the stern tube to prevent ingress of seawater or loss of lube oil.

Stern tube sealing shall be of non-split simplex type. The forward seal shall have two sealing rings and the aft seal shall have four sealing rings.

Net cutter shall be provided.

A split spacer ring for aft seal to be fitted for purpose of removal so that seal nest can be moved on shaft liner.

The renewing of stern tube sealing rings to be of possible without disconnecting the shafting or Propeller. The aft seal to be protected with welded type steel rope guard.

6.4.7	PROPELLER, FAIR WATER CAP AND PROPELLER NUT

The propeller is to be of fixed pitch, aerofoil section, right handed, nickel aluminum bronze, keyless type.

The propeller design to refer to the “Chapter 1 General paragraph 1.2.7.2”

The surface roughness measurement and the static balancing test of the propeller to be carried out in accordance with the manufacturer's standards in compliance with the requirements of the Classification Society.

The hub shall be bored to taper so that it may be accurately fitted to the taper end of the propeller shaft.

The propeller shall be force fitted to the propeller shaft and secured by hydraulic mounting tool and hydraulic nut with locking device.

A fair water cap of Ni-Al bronze is to be fitted over the propeller nut.

The fair water cap shall be drilled and to have connections with brass plugs for filling the void spaces with rust preventive grease.

After the propeller has been finished and inspected according to ISO 484 class 1 standard, the static balance of the propeller shall be checked.

The shafting shall be so designed that the propeller can be dismounted without removal of the propeller shaft ,and the steering gear shall be able to turn suitable  degree (with dismantle limit stopper/ram pin to release the rudder angle restriction) to enable propeller removal.

Propeller light running margin to 6 %.

6.4.8	EARTHING DEVICE

An earthling device with shaft potential monitoring at intermediate shaft shall be provided which is capable of keeping the electrostatic voltage between crankshaft and hull below 50mV finally according to the M/E requirement.

A digital shaft / hull mili-voltmeter with alarm connected to AMS to be provided.

6.4.9	MATERIAL

6.4.9.1	Shafting

Intermediate shaft	Forged steel

Propeller shaft	Forged steel

6.4.9.2	Stern tube assembly

Stern tube assembly	forge steel or/and steel plate

6.4.9.3    Bush

Bush                            Cast iron with white metal

The material for oil sealing device shall be viton type and accordance with the manufacturer's standard.

6.4.9.4	Intermediate Shaft Bearing

Upper and lower shells	Cast steel lined with white metal

Pedestal	Cast iron

6.5	ELECTRIC GENERATOR ENGINE

6.5.1	GENERAL

The electric generating plant shall consist of three (3) sets of diesel generator and one (1) set of emergency diesel generator. The relevant data refer to Paragraph 7.2.3. Concerning electric alternator, refer to “Electric Part”.

Safety protecting and auto start device for three (3) main generator sets shall be installed according to the requirements of unattended machinery space. Emergency generator set shall be auto started, switched on and supply electric power in case of the power failure of main generator sets.

6.5.2	CONSTRUCTION FOR GENERATOR ENGINE

Generator engine

The design, construction, materials, and accessories shall be of Maker's standard in accordance with the requirements of the Classification Society.

Cooling medium shall be L.T. fresh water for cylinder jacket, cylinder cover, lubricating oil cooler and air cooler.

A governor shall be provided on the engine to regulate automatically the quantity of fuel oil supply at different loads. The governor shall be capable of holding the engine speed in accordance with requirement of Classification Society.

Emergency diesel generator engine

The design, construction, materials, and accessories are to be of Maker's standard in accordance with the requirements of the Classification Society.

The engine shall be a vertical, four cycle, single acting trunk piston, radiator-cooled marine diesel engine.

The engine shall be completed with self-contained cooling system and forced feed lubricating system.

The engine coupled with an alternator shall be installed on a common bed and resiliently mounted on the foundation as manufacture’s recommendation.

6.5.3	ACCESSORIES

For each main generator engine

1)	Exhaust gas turbocharger as per Maker’s design	1 set

2)	Air cooler (Charge air temp. to be auto-regulated if Maker can supply)	1 set

3)	Governor	1

4)	Fuel injection pump	each 1/cyl.

5)	H.T. Cool. F.W. pump (centrifugal type engine driven)	1

6)	L.O. pump (gear type, engine driven)	1

7)	F.O. filter	1

8)	L.O. cooler	1

9)	L.O. filter	1

10)	L.O. pre-lubricating pump	1

11)	Preheating device for the HT-circuit	1

12)	Turning device according to maker's standard  	1

13) 	Gauge board 	1 set

14)	Emergency stopping device	1 set

(over speed, L.O. low press., F.W. excessively high temp.)

15)	Silencer	1

16)	Exhaust expansion connection.	1

17))	Fuel oil leakage alarm	1

Each cylinder to be with local thermometers for exhaust gas and cooling water outlet temp as Maker’s standard.

Turbocharger tachometer for Aux engine shall be provided in ECR.

The other’s accessories of generator engine shall be in accordance with Maker's standard.

For emergency generator engine

All accessories and fitting shall be in accordance with Maker's standard.

6.6	STEAM GENERATING PLANT

6.6.1	GENERAL

The steam generating plant shall consist of one (1) composite boiler with oil fired section and exhaust gas section utilizing the heat of main engine exhaust gas. The exhaust gas section of the composite boiler shall act as main engine exhaust silencer and the noise attenuation shall be as per Maker’s standard.

The composite boiler shall be vertical cylinder welded construction smoke/water tube boiler.

The normal necessary steam at sea shall be generated by the exhaust gas section of the composite boiler, utilizing the exhaust gas heat of the main engine at CSR .

The construction of the composite boiler shall be in accordance with Maker’s standard and having adequate access for cleaning and inspection.

The fuel oil burning equipment and feed water regulating (on/off control) of the composite boiler shall be provided and operated automatically.

The exhaust gas section of the composite boiler shall be capable of dry-running in emergency condition in accordance with Maker’s recommendation.

The simultaneous operation of the oil fired section and exhaust gas section of the composite boiler shall be made when the vessel is operated in the lower temperature area or reduced power operation of the main engine.

Excess steam shall be led to the atmospheric condenser via a dump (F.W. cooled).

Steam production by exhaust gas side of the composite boiler to be measured during sea trials.

Exhaust gas side of the composite boiler to be fitted with in and out thermometers and a gauge to measure the pressure drop across.

Boiler Hot well must be equipped with Oil monitor and salinity monitors as standard.

Boiler must also be able to operate on MGO EU directive 2005/53/EC.

6.6.2	ACCESSORIES

The composite boiler shall be accordingly supplied with burner, boiler control panel, feed water regulator, water level gauge, safety valves, other valves and pressure gauges.

All accessories and fittings will be in accordance with Maker’s standard and requirement of Classification Society.

Chemical dosing unit, boiler water testing kit for measuring PH-value, oil detecting device and salinity alarm device shall be provided.

The feed water pumps and fuel oil supply pumps shall be provided in accordance with the Maker’s recommendation. Fresh water hose connection shall be provided adjacent to the composite boiler for water washing.

6.6.3	Materials

In accordance with manufacturer’s standard.

6.7	AUXILIARY MACHINERY

6.7.1	PUMPS

The relevant data of the pumps refer to 6.2.

Pump construction, material, accessories, spares and tools shall be in accordance with Maker's standard and in compliance with requirements of Classification Society.

The construction, materials and accessories for pumps being attached to other machinery such as generator engine, fresh water generator, etc. shall be in accordance with Maker's standard practice.

Positive displacement pumps are to have relief valves.

In generally, pump's motor shall be located above the lower floor plate; pumps shall be accessible for easy maintenance. Lifting eyes to be provided for above major pumps.

Horizontal pumps shall be mounted on common bed plates of fabricated steel or cast iron with the driving motor.

Main L.O. pumps to be fitted with submerged bearings.

6.7.1.1	Centrifugal Pumps

6.7.1.1.1	General

Electric motor of centrifugal pump shall not be overloaded at the service condition on board.

The pump casing shall be designed that the rotating member may be exposed for inspection or replacement without disturbing the pipe connections except the small pump with capacity less than 100 m3/h.

The shaft seal for main L.O. pump shall be in accordance with Maker’s standard.

All vertical pumps shall be driven through flexible coupling between impeller and motor shaft, or may have the thrust bearing in the driving motor and shall be driven through rigid coupling.

Bottom bearing of vertical pumps, if provided, to be of internal water sleeve type

Shaft gland packings as per maker’s standard.

Each pump to be provided with suction and discharge valves except deep well pumps.

Centrifugal pumps to be fitted with mechanical seal except for deep well pumps which are fitted with sealing according to manufacturer’s standard. Centrifugal pumps (Fire, bilge &G.S. pump, Ballast pump, clean drain tank pump, Emergency Fire Pump) to be fitted with vacuum pump for self- priming.

All seals to be Mechanical Seals unless specifically agreed by owners. Spare Mechanical seals to be provided for each type pump.

Motors of all pumps to be more requirement of the maximum rating of the pumps.

6.7.1.1.2	Materials

| Material | S.W. & bilge | F.W. | Boiler water feed |
| --- | --- | --- | --- |
| Casing | Bronze or Bronze alloy | Cast iron | Maker’s standard |
| Casing ring | Bronze or Bronze alloy | Bronze or Bronze alloy | Maker’s standard |
| Impeller | bronze alloy for SW, bronze or bronze alloy for bilge | Bronze or Bronze alloy | Maker’s standard |
| Shaft | Stainless steel | Stainless steel | Maker’s standard |

6.7.1.2	Gear and Screw Pump

6.7.1.2.1	General

Gear and screws shall be accurately machined and thoroughly cleaned.

Each pump shall be directly coupled to a power end of driver via flexible or rigid coupling according to the Maker's standard.

Gear and screw pumps to develop rated capacity against rated discharge pressure without over loading of driving motors when handling with viscosity for which the pump is intended.

The safety valve shall be fitted on the pump casing.

In general, pumps shall be internally lubricated by the pumping fluid.

6.7.1.2.2	Materials

| Material | Gear pump | Screw pump |
| --- | --- | --- |
| Casing | Cast iron | Cast iron or steel plate |
| Gear | Carbon steel | - |
| Shaft | Carbon steel | - |
| Power rotor | - | Carbon steel |
| Idle rotor | - | Carbon steel or ductile cast iron |
| Gland seal | Mechanical seal | Mechanical seal |

6.7.1.3	Piston Pump, Snake Pump and Membrane Pump

Construction and material shall be accordance with Maker's standard.

6.7.2	HEAT EXCHANGERS

The relevant data refer to 6.2.

The construction, materials and accessories for heat exchangers being attached to other machinery such as generator engine, fresh water generator, etc. shall be in accordance with Maker's standard practice.

Sacrificial flange or pipe to be provided for central coolers and sea water pumps.

Pipes shall be so arranged that the heat exchangers can be overhauled with the min. removal of the connected pipes.

6.7.2.1	Plate Type Coolers

6.7.2.1.1	General

The heat-exchanging surface shall consist of some of corrugated plates clamped together in a frame and sealed at edges by rubber gaskets.

The plates form a closed system of parallel two media between which heat is exchanged flow through alternate interpolate spaces.

Heat exchangers shall generally be provided with glass tube thermometers and manometers at inlet and at outlet of both fluids, air vent cocks, drain cocks.

The accessories shall be in accordance with Maker’s standard.

6.7.2.1.2	Materials

Frame	Mild steel,

Plate	Titanium (For S.W.)

Stainless steel SUS316 or equivalent (For F.W. & oil)

Gasket	Maker’s standard

6.7.2.2	Shell Tube Type Coolers

6.7.2.2.1	General

The tubes shall be expanded into the plates at both ends of the tubes. Tube bundle to be removable.

The cooling fluid shall pass through the tubes and the cooled fluid to flow outside of the tubes.

Each heat exchanger shall be provided with a relief valve (oil heater and fresh water heater) vent valve, drain valve, pressure gauge, thermometer, and protective anode according to its service.

One (1) soda cock and one (1) boiling steam valve to be provided as common use for shell and tube type coolers and connecting seats with blind flanges for soda cock and boiling steam valve to be fitted on each tubular cooler.

Tubular type coolers shall be of such design which will relieve stresses due to restriction of thermal expansion on the shell side. Tubes are to be expanded into the tube at both ends of the tubes.

6.7.2.2.2	Materials

Tube	Aluminum brass

Tube plate	Naval brass

Shell	Steel plate or steel pipe

Water box and cover	Steel plate or cast iron with tar epoxy coating.

6.7.2.3	Heaters

Each heater shall be designed for the following conditions to heat oil or water required for each service intended:

| Item | Heating steam Pressure (MPa) | Fluid temp (oC) |  |
| --- | --- | --- | --- |
|  |  | Inlet | Outlet |
| Heavy Fuel oil heater for M/E & G/E | 0.7 | 90 | 150 |
| Heavy Fuel oil heater for purifier | 0.7 | 50 | 98 |
| Lub. oil heater for purifier | 0.7 | 40 | 90 |
| Calorifier | Steam & electric | 10 | 70 |
| M/E jacket F.W. pre-heater | 0.7 | 15 | 50 |

Heavy Fuel oil heater for boiler shall be according to maker’s standard.

The oil heaters should attach temperature gauge and safety valve.

Pre-heater pump shall automatically stop when ME is started,the steam of the pre-heater to be manual control

6.7.3	AIR COMPRESSOR AND AIR RESERVOIR

The air compressor and air reservoir are to be of sufficient capacity to meet the requirement of the Classification Society.

The construction, materials, accessories, spares and tools shall be in accordance with Maker's standard.

6.7.3.1	Main air Compressor

The main air compressors shall be of single acting, two or three stages, fresh water cooled and piston type and shall be driven by electric motors.

Each main air compressor shall be fitted with unloaded, safety valve, air strainer, pressure gauge, fresh water cooler, drain cock and other necessary fittings. Each main compressor shall be provided with automatic and remote starting and stopping device and local manual controller.

Low lub oil Pressure and Compressed air high temperature safety shall be provided as per maker’s standard.

6.7.3.2	Service Air Compressor

The air compressor to be of screw type, electric motor driven, air cooled, self-contained L.O. system, oil filter and oil separator.

Material, spares and tools to be in accordance with the manufacturer’s standard.

6.7.3.3	Emergency Air Compressor

The emergency air compressor shall be of electric motor driven, air-cooled type and used for charging to the aux. air reservoir and starting the “dead ship”.

6.7.3.4	Air Reservoir

The air reservoirs shall be of cylindrical type of welded steel plate, and complete with pressure gauge, safety valve, and drain connection. The main air reservoirs shall be fitted with manual drain valve and manhole.

The air reservoir drain connection shall be with double shut drain valves. Outlet of all safety valves for main air reservoir should be connected to open deck.

The air reservoirs shall be painted inside with anti-rusting.

The air reservoirs have ample construction and hydraulic test shall be in compliance with the requirement of Classification Society.

6.7.4	OIL PURIFIER

All purifier heaters will be as per maker’s standard.

A working bench with two (2) wash basins (sinks) shall be provided in the purifier room if space available. Drains from each wash basin shall be led to the sludge tank.

6.7.4.1	Fuel Oil Purifier

Two (2) sets of HFO/LSHFO/ULSFO purifier shall be used for HFO/LSHFO/ULSFO, and arranged in the purifier area in engine room.

FO purifiers to be provided with flexible feed and discharge lines. Flexible mounting(rubber cushions) to be provided for the FO purifiers. Heater unit for the purifiers to besteam supplied, equipped with steam trap, insulation, temperature gauges etc.

Each FO purifier shall be equipped with water monitoring system & sludge space monitoring system.

One (1) purifier shall be running and the other as stand-by.

The independent supply pump with driving motor shall be provided for each purifier. The heaters of two FO purifiers to be fully interchangeable.

Materials and accessories shall be provided according to Maker's standard.

Each purifier shall be provided with necessary fittings such as a pressure gauge, a thermometer, abnormal flow detector, oil feed shut off valve, water transducer and etc.

Each fuel oil purifier shall run continuously and sludge discharge shall be carried out automatically by means of a timer and water transducer. Start and stop of the purifiers shall be manually and locally effected.

Automatic control panel to be provided for each FO purifier.

Each one (1) automatic control board with abnormal alarm shall be arranged near the purifiers.

One (1) “I” beam shall be fitted above all the purifiers for maintenance.

One (1)-cleaning table shall be provided with cold water, hot water, MGO pipes and valves and drain valve.

6.7.4.2	Lubricating Oil Purifier

One (1) set of M/E L.O. purifier and one (1) set of G/E L.O. purifier shall be arranged in the purifier area in the engine room. G/E LO purifier shall be also as standby for ME

Each L.O. purifier shall be with independent supply pump.

Materials and accessories shall be provided according to Maker's standard.

Each purifier shall be provided with necessary fittings such as a flow indicator, pressure gauge, thermometer, abnormal flow detector, oil feed shut off valve and etc.

The purifier shall be run continuously and sludge discharge shall be carried out automatically by means of a timer. Start and stop of the purifier shall be manually and locally effected.

Each one (1) automatic control board with timer and an abnormal alarm shall be arranged near the purifier.

“I” beam reference with 6.7.4.1

6.7.5	FRESH WATER GENERATOR

The vacuum type fresh water generator, utilizing the waste heat of M/E jacket cooling F.W. shall be installed in engine room and shall be cooled by sea water.

The generator to be of plate type with titanium plates.

The fresh water generator shall produce rated capacity of fresh water not exceeding 10p.p.m. of salinity at rated vacuum and main engine jacket cooling F.W. temperature of 85C and cooling sea water temperature of 32C.

The fresh water generator shall consist of an evaporator, a condenser and complete with water ejector, ejector pump, distillate pump, salinity indicator, automatic alarm, water meter and other necessaries for operation.

Fresh water with high salinity shall be automatically discharged back to the fresh water generator or bilge.

A chemical injection system included dosage tank shall be provided.

Main Engine to be tested during sea trial for one(1) hour without operation of fresh water generator in order to confirm that the jacket cooler capacity is sufficient for dissipating of main engine heat produced.

The materials shall comply with the requirements of the Classification Society and Maker's standard.

6.7.6	ENGINE ROOM VENTILATING FANS

The relevant data refer to Paragraph 6.2.15.

Four (4) ventilating fans shall be provided the airflow for combustion of main engine, generator engines, boiler and for evacuation of heat emission from aux. equipment in engine room. The ventilating fan shall be vertical split type totally enclosed motor driven axial flow type.

Two (2) sets of them shall be reversible and the capacity and head is reduced when the fan shall be run at reverse rotation. Safety timer to be provided to prevent immediate reversal of fan.

The materials shall be as follows:

Casing	: hot galvanized steel

Impeller	: sea water resistant aluminum alloy casting or Maker’s standard

Shaft	: Carbon steel

In addition to above, each one (1) exhaust fan for purifier area, and welding space in ER workshop shall be installed.

6.7.7	OILY BILGE SEPARATOR

Oily bilge separator shall be provided for handling the oily bilge water in engine room.

The performance is to meet the requirements of MARPOL & IMO MEPC 107 (49). The oil content of treated water shall be less than 15 ppm and oil content bilge alarm shall be provided. And T-valve with automatic function shall be used for return water to bilge water holding tank.

The construction, materials, accessories, spares and tools shall be in accordance with Maker's standard.

An oil content discharge monitoring shall be installed at outlet of clean bilge discharge pump.

Give the capacity of treating the oil in water in the Oil water separator technical agreement.

6.7.8	SEWAGE TREATMENT PLANT

One (1) sewage treatment plant of biological type shall be provided in engine room.

The construction, materials, accessories, spares and tools shall be in accordance with Maker's standard.

Soil pipe from hospital block shall be separately led to the sewage treatment plant.

One  (1) set of unit type toilet shall be provided in engine room  and sewage shall lead to the sewage  collecting tank by gravity.

The performance shall meet the requirement of IMO.

The grey and black water pipes should be sufficient inclined downwards for easy draining of water. Furthermore, horizontal led drain pipes should be avoided. Suitable access openings with blind flanges for cleaning, to be arranged as far as possible.

The plant to be provided two air blower(or air compressor) and an alternative air at 7kg/cm2 .

6.7.9	INCINERATOR

The incinerator shall be in compliance with the requirements of MARPOL 73/78 Annex VI and MEPC 76 (40) and alarms for furnace high temperature, flue high temperature, initiation failure, blanking failure and low fuel oil pressure etc.

The incinerator shall be capable of burning solid waste and waste oil.

Exhaust pipe for incinerator shall be provided with suitable expansion joints.

The following accessories to be included:

Burner unit	1 set

Sludge oil circulating pump	1 set

Diesel oil pump	1 set

Sludge oil tank	1 set

Flue gas fan	1set

Flue gas damper	1 set

Other necessary accessories, fittings, spares and tools shall be according to Maker's standard.

6.8	MISCELLANEOUS EQUIPMENT AND OUTFITTING

Miscellaneous equipment and outfitting such as tanks, ventilating duct, ladders, floor plates, etc. shall be constructed and furnished in accordance with Builder's standard practice and shall be comply with the requirements of the Classification Society.

6.8.1	OVERHAULING AND REMOVAL OF MACHINERY

Overhauling equipment for main engine and auxiliary machinery shall be provided as mentioned below.

During designing and construction of the engine room, particular attention shall be paid to ensure sufficient space for operation and dismantling of machinery and equipment.

6.8.1.1	Overhauling Crane of Main Engine

For overhauling cylinder piston and liner etc. of main engine, there is a crane on the overhead-lifting beam.

The crane shall be of marine trolley type with electric motor driven two speeds for hoisting, and single speed for traveling and traversing. The construction and materials shall be in accordance with Maker's standard.

The capacity of the engine room overhead crane is to be at least 4.0 t and can be hoist the heavisest spare part of the main engine.

Overhead lifting beam shall be securely supported to ship's structure and to extend the full length of the main engine.

Lifting eyes or I-beam shall be provided for overhaul T/C of main engine in case E.R. crane can’t be utilized for overhauling.

Facilities to be provided to visit the engine room crane if it stops at any position of its travel.

Source cable layout to be of a suspension type and long enough to reach the lower floor.

A cargo light to be fitted on the top of the overhead crane beam.

6.8.1.2	Shafting Removal

One (1) lifting beam with two (2) trolley hooks of adequate capacity to hoist the propeller shaft (+25% margin) to be provided above the propeller shaft if the lifting beam installation available. If enable, builder provided eye plate with chain block.

Necessary lifting eye plates shall be fitted above the shafting alley for lifting of the intermediate shaft and withdrawing the propeller shaft inboard. Removal of propeller shaft from the vessel shall be carried out via a temporary opening out in the shell plating which shall be marked by welding bead in approved colored scheme. Equipment and pipes arranged at that place shall be easily dismantled.

6.8.1.3	Overhauling of Auxiliaries

Generator engines shall have adequately strength lifting beam to facilitate overhaul.

Provided for the generator engines to be of sufficient capacity to hoist the alternator’s rotor.

Lifting beam	: 3 sets

Chain block with roller	: 3 sets, manual type

Capacity	: according to the generator Maker’s recommendation.

Oil purifiers shall have lifting beam using chain blocks to facilitate overhaul.

Lifting beam	: 1 set

Chain block with roller	: 1 set, manual type

Capacity	: 1.0t

A lifting beam shall be provided for transport of the heavy parts from engine room to workshop.

Lifting beam	: 1 set

Chain block with roller	: 1 set, manual type

Capacity	: 2.0 t

Other larger aux. equipment above shall have eye plates to facilitate overhaul.

6.8.1.4	Heavy Element in/out Engine Room

Spare gears, tools and other machine parts and big strainers shall be carried in/out of engine room through the hatch on deck.

A lifting beam shall be provided for transport of the heavy parts from spare landing hatch to workshop and E/R store shop.

One trolley to be provided to transport heaviest engine spare like liner.

Lifting arrangements and transport ways from exposed deck to bottom floor in engine room shall ensure easy transport of heavy elements by or harbor crane.

Lifting eyes to be provided for pieces of machinery weighting more than 50kg.

Lifting eyes with chain block to be provided for main SW filter.

Lifting beam to be provided for oil purifier to facilitate overhauling.

6.8.2	TANKS IN ENGINE ROOM

Tanks in engine room shall be of welded steel plate, and provided with necessary fittings such as heating coils, thermal insulation, level measuring apparatus, thermometer socket, air escape pipe, overflow pipe, drain plug, hole for cleaning, valves, etc., to be according to tank purpose.

All of the oil tanks shall be provided with drip pans as practicable. Heating coils shall be provided for the following tanks:

Tank name 	Heating coil surface ratio

HFO, ULSFO/LSHFO settling tank	0.2m2/m3

HFO, ULSFO/LSHFO service tank	0.2m2/m3

M/E LO settling tank	0.1m2/m3

M/E LO sump tank	around bell mouth

Fuel oil drain tank	0.1m2/m3

Sludge tank	0.25m2/m3

Fuel oil overflow tank	0.08m2/m3

G/E lub. oil settling tank	0.1m2/m3

Lub. oil drain tank	0.1m2/m3

Bilge tank	0.1 m2/m3

6.8.3	VENTILATING OF MACHINERY SPACE

The ventilating ducts shall be so arranged that air can be supplied to engine room where necessary such as for main engine, generator engines, composite boiler, workshop store and working space etc. All ducts shall be of steel plate or galvanized steel sheet.

Total capacity of ER supply fan shall be calculated according to “ISO8861Shipbuilding-Engine room ventilation in diesel engine ships – Design requirement and basis of calculation (1998(E)).

The ventilating ducts shall be of rectangular cross section and made with flange or welding sleeve connection to allow replacement for repair.

In general, min. height at main passage beneath the air duct shall be 2.0m (as far as possible). The thickness of ventilating duct shall be as follows:

The biggest side ≥800mm to be made of steel plate Q235-B, about 5.0mm thick;

600 mm ≤the biggest side< 800mm to be made of steel plate Q235-B, about 3.0mm thick; The biggest side < 600mm to be of 1.0mm galv. steel plate.

The inside of main duct (not include galv. steel plate) to be painted with epoxy paint..

The air ducts shall be provided with adjustable dampers where necessary for proper ventilation keeping healthy conditions and even temperature distribution within the machinery space.

Pneumatically controller fire dampers or ventilation inlet to be provided for engine room supply fans.

The adjustable dampers in engine room shall be operated manually by chain if necessary.

The independent exhaust air duct shall be arranged for purifier area and ER workshop welding space. And the ventilating duct for purifier area should be fitted a fire damper.

Exhaust duct to be provided over heavy fuel oil purifiers in the purifier’s space.

Exhaust duct to be provided in welding space of workshop.

Ducts shall be securely supported or braced from ship's structure and shall be designed to avoid irregular bends or branches as far as practicable.

The pin, hinge, bolts and nuts in the fan casing and duct-connecting flange exposed to weather shall be stainless steel (SUS316 or brass).

Excess air from the engine room shall be discharged to atmosphere through louvers at the aft of the funnel.

The E.C.R. shall be air-conditioned and supplied with mechanical ventilation with damper.

The working ladders or step shall be provided for maintenance if necessary.

6.8.4	SILENCERS

The silencer shall be provided for generator engines and emergency generator engine.

6.8.5	ENGINE CONTROL ROOM, ENGINE ROOM WORKSHOP & STORES

6.8.5.1	Engine Control Room

The engine control room shall be heat and sound insulated, and shall be provided with engine control, monitoring console and main switchboard etc. A unit cooler for air conditioning shall be also provided.

One (1) escape with a continuous fire shelter and vertical ladders to open deck shall be provided in compliance with the requirement of MSC 93.

A clear walkway shall be arranged between consoles and window bulkhead.

6.8.5.2	E/R Workshop

A workshop shall be located in the engine room and provided with such as universal machine tool, etc. as listed in Section 6.2.15 and one (1) work bench, two (2) sets of vise, one (1) welding table, shelves, lockers, etc. lifting beam and a door with locking device shall be provided for the workshop. Workshop to be enclosed by steel wall.

Fuel valve test devices shall be located in workshop.

One (1) tool board to be provided in workshop.

Acetylene and oxygen bottles shall be installed outside the engine room and suitable piping with blow back valve shall be provided from bottle room to welding space in workshop.

One (1) escape with a continuous fire shelter and vertical ladders to open deck shall be provided in compliance with the requirement of MSC 93.

6.8.5.3	Electrical Workshop

A separate electrical workshop shall be located in the engine room.

One (1) motor driven table driller, one (1) bench with vice, and one (1) test panel shall be provided.

Electrical Workshop shall be enclosed by steel wall.

6.8.5.4	Store

Each one (1) engine store and electric store shall be arranged and suitable steel shelves laid wood plate shall be provided in the store. The electrical store will be isolated with electrical workshop by steel wire wall.

There are six(6) drawer style storage to be installed in the engine store.

6.8.6	LADDERS, HANDRAILS, GRATINGS AND FLOOR PLATES

The painted ladders, painted handrails, painted gratings and painted floor plates shall be provided in accordance with shipbuilder practice.

6.8.6.1	Ladder

Main access ladders shall be led fore and aft wherever possible and to slope of 55~60 degrees from the horizontal, and width of main ladder shall be of 700mm and that of sub-ladders 500 mm.

Ladders to be inclined type, except where it is unavoidable, vertical ladder shall be used for unimportant access provision. Galvanized steel dust guard shall be fitted at underside of main ladders.

Treads shall be a non-slip type.

All ladders shall be attached at top and bottom by bolts to provide ready portability for overhaul of machinery when necessary.

6.8.6.2	Handrail

Handrail shall be fitted at gratings, ladders and floors where necessary for the safety and convenience of operating personal. Handrails shall be of steel pipe carried in steel pipe stanchions and shall be fitted with portable section where required for overhauling machinery.

Handrails shall have a clear hand space of at least 50mm from adjacent equipment and bulkhead.

6.8.6.3	Floor plate

In general, the flooring shall be checkered steel plate of about 4.5 mm in thickness, supported and fastened to suitable bars of angle frame by not less four (4) stainless steel or brass screws.

Removable sections shall be provided as required for proper access for handling and inspection of the equipment located under floor and for manholes.

Floor plates sections not to be larger than what can be handled by one man as far as practicable.

Coaming of height 50mm shall be provided around permanent openings.

Opening in the checkered steel plate for valve access shall be cut by a tool on site.

6.8.6.4	Grating

Adequate grating of steel shall be provided for machinery space walkways, platforms and floors only where specifically required for ventilation and continuous visibility purposes. Gratings to be made of twisted bar and steel plate, suitable spaced and properly fitted in steel flat frames.

6.8.6.5	Funnel

Ladder and grating also shall be installed in funnel. The flue louver shall be controlled outside the funnel by compressed air and also can be controlled by manual.

6.8.6.6	Protection Device

All moving parts such as flywheel, coupling shall be protected by safety cover or hand rail.

6.8.7	DISTINGUISHING AND UNIT

In case of two or more sets of identical machinery units are installed, those machinery shall be distinguished by numbering. In general, the numbering shall be carried out in this way:

From up to down

From fore to aft.

From port to starboard

The measuring instruments used for equipment and piping shall be according to International Standard.

Name plates shall be made of brass plate and marked by etching and painted black for illustration plate, red for warning plate and valve name plate respectively.

| Item | Language | Measuring unit |
| --- | --- | --- |
| Instruments: Thermometer Temperature indicator Pressure gauge Compound gauge Vacuum gauge Level indicator Electric indicator Salinity indicator Flow meter Indication of tank volume Measuring instruments | English | Metric or SI |
| Name plates & drawings: Name plate (fitted to machinery) Plan for approval Finished plan Instruction book Inspection record Test record Sea trial record | English | Metric or SI |
| Name plate (fitted to valve) | English |  |

6.9	INSULATION AND PAINTING

6.9.1	INSULATING

6.9.1.1	General

The surface of the equipment, tanks and piping which shall treat or contain the fluid with temperature over 60C except for cooling F.W. piping and L.O. piping should be insulated.

Main engine, generator engines, boiler, incinerator and calorifier shall be insulated according to the Maker’s standard.

Where it is necessary to fit removable and replaceable insulation on flanges, fittings, and covers, etc. The tailor-made pads or blankets shall be used.

The outer face temperature after insulation shall be not exceeded by 60oC when the engine room temperature is 45C.

6.9.1.2	Insulation for Thermal Equipment.

(1)	Boiler and Heater

Maker will deliver the boiler with insulation.

The minimum thickness of insulation material for surfaces of large extent such as heaters, etc. shall be met the following requirements or according to the Maker’s standard.

| Temperature range of internal fluid | Ceramic fibre moulding or rock wool |
| --- | --- |
| 60 to 155C | 25mm |
| 156 to 260C | 50mm |

(2)	H.F.O., U.L.S.F.O./ L.S.H.F.O. Tanks

- HFO and LSHFO/ULSFO settling tanks

- HFO and LSHFO/ULSFO service tanks

In case of structure tank, front and side walls faced to engine room area only shall be insulated and to be covered with galvanized steel plate up to 2.3m in height from bottom of deck. The remaining upper part shall be covered with aluminum foil coated fiber glass cloth.

|  | Material | Dimension | Remark |
| --- | --- | --- | --- |
| 1 | Ceramic fibre moulding or rock wool | 25 mm |  |
| 2 | Steel plate | 0.5 mm | Galvanized |

(4)	Silencers

The exhaust gas silencers for generator engine and emergency generator engine should be insulated with min. 50mm thickness of Ceramic fibre moulding or rock wool and covered with 0.5mm galvanized steel sheet.

6.9.1.3	Insulation for exhaust gas piping

The exhaust gas piping and uptake in engine room shall be insulated and the requirements are as follows:

| Name of equipment | Insulation |  | Cover |
| --- | --- | --- | --- |
|  | Thickness (mm) | Material |  |
| Main engine | Pipe 75 Flange75 | Ceramic fibre moulding or rock wool | Glass cloth + galv. steel sheet 0.5mm thick |
| Generator engine | Pipe & flange 75 | Ditto | Ditto |
| Incinerator & boiler | Pipe & flange 50 | Ditto | Ditto |

The exhaust gas piping and uptake from bottom to nav. deck above three (3) meters in funnel shall be insulated with thickness of 50mm and covered with galvanized steel sheet of 0.5mm thickness.

In all case where the pipe insulation contacts on the flanges and fittings, the insulation shall be suitably terminated to permit free removal of bolt and to allow for movement of the pipe at supports.

6.9.1.4	Insulation for Thermal Fluid Piping

The following piping system should be insulated.

| Name of pipe | DN mm | Insulation |  | Cover |
| --- | --- | --- | --- | --- |
|  |  | Thickness mm | Material |  |
| H.F.O.，/LSHFO/ULSFO | <50 >50 <10 | 30 40 | Mineral wool Two layers of glass cloth | glass cloth glass cloth |
| Steam, condense | <50 >50 | 30 40 | Mineral wool h | glass cloth |
| Hot water |  | 20 | Mineral wool | glass cloth |

In principle, the H.F.O. ,LSHFO/ULSFOpiping (including HFO,LSHFO/ULSFO transferring pipe line, HFO,LSHFO/ULSFO purifying pipe line, HFO,LSHFO/ULSFO service pipe line, HFO,LSHFO/ULSFO drain pipe line and HFO,LSHFO/ULSFO return pipe line) should be provided with steam tracing pipe and the connection of steam tracing pipe should close up to the flange of the H.F.O. ,LSHFO/ULSFO pipe.

The following pipes shall be insulated with glass cloth.

Boiler blow pipe

Exhaust steam pipe for safety valves

Drain pipe after steam drain valves and drain traps

6.9.2	PAINTING

Machinery and piping in engine room shall be painted except following parts.

1. Working surface of machinery, brass surface, and other parts, which are finished bright.

2. Surfaces of insulation, which is not covered with steel sheet or equal.

External surface of pipes and valves fitted below floor level (E/R grating of tank top) shall be painted with 2 coats anti-corrosion paint only.

Emergency Exit path to be marked with Florescent paint at all platforms / levels.

Other machinery and equipment shall be in accordance with Maker's standard.

The color of finished coat for external surface shall be in accordance with the following table. Maker's standard surface color shall prevail if there is conflict with the following table for individual equipment.

| Name of machinery or equipment | Color (Yard proposal) |
| --- | --- |
| Diesel engine Air compressor Centrifugal pump Screw and gear pump Oil purifier (acc. to Maker’s standard) Fan Oily bilge separator Sewage treatment plant F.W. generator Overhauling crane Workshop machine Intermediate shaft bearing Steel box for spare parts L.O. and F.W. coolers | Light blue green (Munsell notation: 7.5 BG 7/2) |
| Turbocharger for main engine Turbocharger for generator engine Boiler Incinerator Oil heater Atmospheric condenser | Silver |
| Air reservoir Tanks not to be insulated Pipes not to be insulated Air trunk Water pressure tank Valves not to be insulated Strainer Top rail of hand rail Ladder except step | White |

The piping shall be marked as per Buyer’s standard with color vinyl tapes or color paints to indicate the various mediums.

The medium flow direction shall be indicated by arrow shown outside the piping.

6.10	 PIPING SCHEDULE IN ENGINE ROOM

6.10.1	GENERAL

Piping workmanship to satisfy the requirement of the Classification Society and this section concerned.

Piping is to be led as direct as practicable, with a minimum number of bends and with suitable number of flange joints for removal. Sleeve joints or butt welding joints shall be used where practicable.

All piping shall be supported according to good practice in order to prevent damages from vibrations and other causes.

Pipes to have suitable thickness for each service, and shall be tested in accordance with the requirement of the Classification Society.

Expansion shall be compensated for by means of bend except where otherwise described.

Pipes of galvanized steel or brass or copper shall be bent in cold process. In case that cold bending machine is not applicable, high frequency induction bender or hot bending process shall be used.

The radius of bends of steel pipes, in general, not to be less than 2.5 times the outside diameter except narrow space of structure or crowding of pipes, where steel bend pieces hall be used. Bends of copper pipes except coils shall be not less than twice the outside diameter as far as practicable.

Commercial bend pieces will be used as far as practicable.

For the following cases, butt welding connection or fabrication shall be carried out.

1.	When the cold bending machine is not applicable.

2.	Where welding piece such as tube turn or bend piece is used.

3.	To make fabricated steel pipe of large diameter such as main engine exhaust pipe.

Oil piping and electric cables to the greatest possible extent shall be kept well clear of not to be installed close to steam lines and diesel engines exhaust pipes.

Water piping not to be installed close to diesel engines exhaust pipes as far as possible.

Piping in way of the machinery and equipment shall be arranged to permit overhaul of the unit with no dismantling of piping as far as practicable.

Where possible, piping at tank top level shall be arranged so that any length may be removed without disturbing adjacent pipelines.

Air pockets in pipe lines shall be avoided as far as practicable. Where pockets do occur, bosses with a cock or screwed plug for drainage are to be provided.

Pipes exposed to weather are to be fitted with drain plugs where necessary.

Where galvanized piping is specified, galvanizing shall be carried out after pipes are fabricated and flanges are attached, as far as practicable. Galvanized pipes shall be fitted with galvanized bolts, nuts and clamping devices.

All piping shall be cleaned and free of welding slag before installation.

Following piping system shall be flushed with the under-mentioned medium after being fitted on board, and these piping systems to be in whole accordance with maker recommendations.

| System | Application | Method |
| --- | --- | --- |
| Lub. oil piping | Main engine line | To be circulated by flushing oil |
|  | Line from cylinder oil tank to engine | To be blown out by compressed air |
| Fuel oil piping | Main engine line Diesel generator line | To be circulated by diesel oil |
|  | Boiler F.O. burning line Incinerator fuel oil line | To be blown out by compressed air |
| Comp. air piping | Main engine starting air line Diesel generator starting air line Cont. air line | To be blown out by compressed air |
| Steam & condensate | Steam supply line Condensate return line | Steam blow off |
| Hydraulic oil | Bow thruster Steering gear | Flushing oil |
| Boiler feed water line |  | Compressed air blow off |
| Refrigerant piping (if necessary) | Refrigerate plant AC plant | To be blown out by N2 gas |

If flushing of pipes are done by yard supplied lub.oil, builder should present the lab test certificate done prior usage for flushing. Flushing lub. oil not to be reused.

Where pipes pass through the water/oil tight bulkhead and deck, penetration pieces of two flanges with double plate and sleeve joint are to be used.

Piping is to be marked with color vinyl tape or paint for easy recognition. Color marking for various pipe systems shall be according to ISO standard.

After finishing of steel pipes, the acid pickling and neutralizing shall be carried out for the following pipes before installation on board and coated inside of pipes with suitable oil for rust protection. Pipes after acid pickling shall be well covered at ends to keep clean.

a.  Lubricating oil pipes except oil bilge pipe and air vent pipe.

b.  Fuel oil service pipes for main engine, diesel generator and boiler burning line only.

c.  Hydraulic oil steel pipe.

Liquid pipe lines shall be kept away from the switchboard and other electrical equipment.

Where the piping is unavoidable and flanges to be led above the electrical appliances, the welded joint to be provided.

All detachable pipe connections and valves of fuel oil piping shall be at a safe distance from boiler, exhaust pipes or other heated surface.

All connecting dimension of flanges shall be according to ISO standard. Asbestos material is not to be used for gasket and insulation.

Brass or stainless steel name plates with engraved letters are to be referred to piping system drawings for all valves, pumps, heat exchangers etc.

6.10.2	VALVE AND FITTING

All valves and cocks shall be in accordance with Chinese standard, but the connecting dimension of flange shall be in accordance with ISO standard.

In general, stop valves, gate valves and angle valves shall be used for all piping system unless specially specified. Butterfly valves shall be used for the pipe lines of 80 mm bores and above where applicable for fresh water, sea water, lubricating oil, fuel oil and others in accordance with Builder’s practice.

The material of the butterfly valve shall be as follows:

Body	Cast steel for ship side valves, cast iron for others

Disc	Aluminum bronze

Seat	Synthetic rubber (NBR) spindle stainless steel

The wafer type butterfly valve shall be adopted except ship side valves, which are to be of flanged type or lugged type. That in ER (DN250mm above) to be lugged type.

All butterfly valves (except oil valve) shall be provided with synthetic rubber (NBR) lining or renewable NBR seat except that fluoro rubber lining shall be used for high temperature fresh water cooling system.

Butterfly valves for oil piping line shall be applied to oil resistance material instead of NBR seat.

Manually operated butterfly valve shall be provided with mechanical indicator (open or close) actuated by the revolution of the hand wheel or hand notch

In case where drain traps are served continuously, a by- pass line shall be provided.

Each pressure reducing valve is to be provided on its discharge side with a pressure gauge and a safety valve having sufficient discharge capacity to prevent excessive pressure in the low pressure line.

Temperature control valves are to be provided with a manually operated handle. In case where manual handle is not fitted, a suitable by-pass line is to be provided.

Valves are to be fitted as extensively as possible instead of cocks.

Cocks may be used for low pressure gauge and drain lines.

Remote quick-closing valves in fuel oil lines are to be of pneumatic type. These valves are to be capable of being operated locally.

Valves below engine room floor are to be arranged for easy access with wheel spindle upward vertically near to floor plate as far as possible.

Suitable handles having engraved brass name plate with arrow indicator in English are to be fitted to each manually operated valve.

6.10.3	VELOCITIES IN PIPE LINES

The fluid velocities in pipelines shall be as per follows:

Sea water cooling line, suction side:	abt. 2.5m/s

Sea water cooling line, discharge side:	abt.3.5m/s

Fresh cooling water line:	abt.3m/s

ballast and bilge line :	abt. 3 m/s for steel pipes

FO(excluding filling line):	abt. 2.0m/s

LO(excluding filling line):	abt. 2.0m/s

6.10.4	MATERIALS & SIZE OF PIPING

Materials and size of pipes, valves and fittings shall be in accordance with following tables.

| System | Nominal size (mm) | Pipe |  | Pipe joint | Valve |  | Fitting |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Material | Thickn ess |  | Body | Disc. & seal Stem |  |  |
| Steam 0.7MPa | 50 & above | Seamless steel | A | flange or butt welded or sleeve | Cast steel flgd. | As per GB/CB standard | Cast steel or welded steel flgd. or welded branches |  |
|  | 40-15 |  |  |  |  |  |  |  |
|  | 10 & below | Seamless copper |  | Brass union or bite joint or sleeve | Bronze or brass screwed |  | Brass screwed |  |
| Steam 0.4MPa and Drain | 50 & above | Seamless steel | A | flange or butt welded or sleeve | Cast steel flgd. | As per GB/CB standard | Cast steels or welded steel flgd. or welded branches |  |
|  | 40 |  |  |  |  |  |  |  |
|  | 25-15 |  |  |  |  |  | Brass screwed |  |
|  | 10 & below | Seamless copper |  | Brass union or bite joint or sleeve | Bronze or brass screwed |  |  |  |
| Heating coils in oil tank | All size | Seamless steel | B | sleeve | -- | -- | - |  |

| System Aux. boiler feed pump suction | Nominal size (mm) 25& above | Pipe |  | Pipe joint flange or butt welded or sleeve | Valve |  | Fitting Cast irons or welded steel flgd. or branches |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Material Seamless steel | Thickn ess A |  | Body Cast steel flgd. | Disc. & seal Stem |  |  |
|  |  |  |  |  |  | As per GB/CB standard |  |  |
|  | 20 & below | Seamless steel | A | Flange or butt welded or sleeve | Bronze screwed | As per GB/CB standard |  |  |
|  | 10 & below | Seamless copper |  | Brass union or bite joint or sleeve | Bronze screwed |  | Brass screwed |  |
| Aux. boiler feed pump Delivery and boiler blow off. | 25 & above | Seamless steel | B | flange or butt welded or sleeve | cast steel | As per GB/CB standard | Cast irons or welded steel flgd. or branches |  |
|  | 20 & below | Seamless steel | B | Flange or butt welded or sleeve | Bronze screwed | As per GB/CB standard |  |  |
|  | 10 & below | Seamless copper |  | Brass union or bite joint or sleeve | Bronze screwed |  | Brass screwed |  |
| Cooling sea water | 60 & above | Seamless steel galv. | B | flange or butt welded or sleeve | Cast iron flgd or wafer. | As per GB/CB standard | Cast irons or welded steel flgd. Or welded branches |  |
|  | 50 & below | Seamless steel galv. | B |  | Bronze or brass flgd or wafer |  |  |  |
|  | 10 & below | Seamless copper |  | Brass union or bite joint or sleeve | Bronze or brass |  | Brass screwed |  |
| Cooling fresh water | 25& above | Seamless steel | A | Flange or butt welded or sleeve | Cast iron flgd. or wafer | As per GB/CB standard | Cast irons or welded steel flgd. Or welded branches Brass screwed |  |
|  | 20 & below |  |  |  | Bronze or brass flanged |  |  |  |
|  | 10 & below | Seamless copper |  | Brass union or bite joint or sleeve | Bronze or brass screwed |  |  |  |
| Water ballast, bilge | 25& above | Seamless steel galv. | B | flange or butt welded or sleeve | Cast iron flgd.or wafer | As per GB/CB standard | Cast iron or welded steel flgd. or welded branches |  |
|  | 20 & below |  |  |  | Bronze or cast steel |  |  |  |
|  | 10 & below | Seamless copper |  | Brass union or bite joint or sleeve | Bronze or brass screwed |  | Brass screwed |  |
| Fresh water in E/R. | 25& above | Seamless steel galv. | A | flange or butt welded or sleeve | cast iron flgd. or brass screwed | As per GB/CB standard | Cast iron or welded steel flanged or welded branches |  |
|  | 20 & below |  | A | flange or butt welded or sleeve | cast iron flgd. or brass screwed |  |  |  |
|  | 10 & below | Seamless copper |  | Brass union or bite joint or sleeve | Bronze screwed |  | Brass screwed |  |

| System | Nominal size (mm) | Pipe |  | Pipe joint | Valve |  | Fitting |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Material | Thickn ess |  | Body | Disc. & seal Stem |  |  |
| Fire main | 50& above | Seamless steel galv. | B | flange or butt welded or sleeve | Cast iron flgd. | As per GB/CB standard | Cast iron or welded steel flgd. or welded branch |  |
|  | 40& below |  |  |  | bronze or brass flgd. |  |  |  |
|  | 10 & below | Seamless copper |  | Brass union or bite joint or sleeve | Bronze or brass screwed |  | Brass screwed |  |
| F.O. supply | 20& above | Seamless steel | B | flange or butt welded or sleeve | Cast steel flgd. | As per GB/CB standard | Cast iron or welded steel flgd. or welded branch |  |
|  | 15 |  |  |  |  |  |  |  |
|  |  |  |  |  | Bronze or brass flgd. |  |  |  |
|  | 10 & below | Seamless copper |  | Brass union or bite joint or sleeve | Bronze or brass screwed |  | Brass screwed |  |
| F.O. transfer | 50 & above | Seamless steel | A | flange or butt welded or sleeve | Cast steel flgd. | As per GB/CB standard | Cast iron or flgd. or welded branch |  |
|  | 40-15 |  |  |  | Ditto |  |  |  |
|  | 10 & below | Seam less copper |  | Brass union or bite joint or sleeve | Bronze or brass screwed |  | Brass screwed |  |
| Lub. oil | 25& above | Seamless steel | A | flange or butt welded or sleeve | Cast iron flgd. | As per GB/CB standard | Cast iron or welded steel flgd. or welded branch |  |
|  | 20& below |  |  |  | Bronze or brass flgd. |  |  |  |
|  | 10 & below | Seamless copper |  | Brass union or bite joint or sleeve | Bronze screwed |  | Brass screwed |  |
| Com- pressed air (3 MPa) | 40 & above | Seamless steel | B | butt welded or sleeve or flange | Cast steel flgd. | As per GB/CB standard | Cast steel or welded steel flgd. or welded branch |  |
|  | 25-15 |  |  |  | Cast steel flgd. |  |  |  |
|  | 10 & below | Seamless copper |  | Brass union or bite or sleeve | Forged steel screwed |  | Brass screwed |  |

| System | Nominal size (mm) | Pipe |  | Pipe joint | Valve |  | Fitting |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Material | Thickn ess |  | Body | Disc. & seal Stem |  |  |
| Com- pressed air (0.7MPa) | 25 & above | Seamless steel galv. (control air piping do not need galv.) | A | flange or butt welded or sleeve | Cast steel flgd. | As per GB/CB standard | Cast steel or welded steel flgd. or welded branch |  |
|  | 20 & below |  |  |  | Bronze or brass flgd. Forged steel screwed |  |  |  |
|  | 10 & below | Seamless copper |  | Brass union or bite joint or sleeve |  |  | Brass screwed |  |
| Main & Aux. Engine exhaust gas, boiler exhaust gas | 600 & above | Welded steel plate | 6mm | flange or butt welded or sleeve | -- | -- | Welded steel flgd. or welded branch | - |
|  | 250-600 | Welded steel plate | 4mm |  |  |  |  |  |
|  | 250 & below | Seamless steel pipe | A |  |  |  |  |  |
| Air vent & overflow attached to double bottom & deep tank & E/R tank, void space, located on exposed deck | 100 & above | Seamless steel | B | flange, sleeve, butt welded | -- | -- -- | Cast iron or welded steel flgd. or welded branch |  |
|  | 80-65 | Seamless steel | B |  |  |  |  |  |
|  | 50-25 |  | B |  |  |  |  |  |
| Air vent & overflow except mentioned above open drain scupper | 50 & above | Seamless steel * | B | flange or butt welded or sleeve | Cast iron flgd. | As per GB/CB standard | Cast irons or welded steel flgd. or welded branch |  |
|  | 40-15 |  |  |  | Bronze flgd. |  | Brass screwed |  |
|  | 10 & below | Seamless copper |  | Brass union or bite joint or sleeve | Bronze screwed |  |  |  |
| Sounding for double bottom, deep tank & void space |  | Seamless steel * | B | flange or butt welded or sleeve | -- | -- | Cast irons or welded steel flgd. or welded branch |  |

Notes:

a)	All galv. pipes shall be of hot dip type.

c)	* All air vent, overflow & sounding pipes shall be galvanized except for oil system, and the air pipe to be added thickness for open deck area.

d)	Materials of automatic control valves for all system shall be of Maker's standard.

e)	Start air system for ME, G/E and so on shall not be galvanized or as per Maker recommendation.

f)	Number, positions, and type of pipes to be finally approved during plan approval.

g)  The clamping for copper pipes will be according to the builder standards.

Remarks:

A .......... Ordinary	B .......... Medium	C .......... Heavy

The thickness of 16mm material with epoxy painting after hot dip galvanized should be applied for the cross sea water pipes between the sea chests.

Table of steel pipes thickness

| Nominal diameter (mm) | External diameter (mm) | Thickness (mm) |  |  |
| --- | --- | --- | --- | --- |
|  |  | A | B | C |
| 10 15 20 25 32 40 50 65 80 100 125 150 200 250 300 350 400 450 500 | 17 22 27 34 42 48 60 76 89 114 140 168 219 273 325 377 406 457 508 | 2.0 3.0 3.0 3.03.0 4.0 4.0 5.0 5.0 5.0 5.0 6.0 6.0 7.0 8.0 8.0 8.0 8.0 8.0 | 2.5 3.5 3.5 4.0 4.0 5.0 5.0 6.0 6.0 6.0 7.0 8.0 8.0 9.0 10.0 10.0 10.0 10.0 10.0 | 3.5 4.0 4.0 5.0 5.0 6.0 6.0 7.0 8.0 9.0 10.0 11.0 13.0 13.0 13.0 13.0 13.0 13.0 13.0 |

Table of copper pipes thickness

| Normal diameter (mm) | Thickness (mm) |
| --- | --- |
|  | Copper pipe |
| 8-12 14-20 25 32-40 50 | 1.0 1.5 2.0 2.0~3.0 2.0~3.0 |

Table of stainless steel pipes thickness

| Normal diameter (mm) | Thickness (mm) |
| --- | --- |
|  | Stainless pipe |
| ≤12 15-40 50-80 100-150 | 1.0 2.0 3.0 4.0 |

6.10.5	STRAINERS

Strainers shall be provided for various systems according to the following table.

Strainers not listed below and supplied together with equipment shall be according to Maker’s recommendation.

| System | Place of installation |  | No. of set | Type | Mesh of element | Material |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Body | Elem. |
| Fuel oil | Main engine | Inlet | 1 | Auto. back wash with manual by-pass filter | As per Maker’s recommendation | Cast iron or fabricated steel plate | Brass wire or stainless steel |
|  | Generator engine | Inlet | 1 | Auto. back wash with manual by-pass filter | As per Maker’s recommendation |  |  |
|  | FO purifier | Suction | 1 each | Simplex | 30 |  |  |
|  | HFO , LSHFO/ULSFO trans pump | Suction | 2 | Simplex | 24 |  |  |
|  | M.G.O. trans. pump | Suction | 1 | Simplex | 24 |  |  |
| Lub. oil | Lub. oil pumps (for main engine) | Suction |  |  |  |  |  |
|  |  | Delivery | 1 | Auto. back wash with manual by-pass filter | As per Maker’s recommendation |  |  |
|  | L.O. purifier | Suction | 1 each | Simplex | 30 |  |  |
|  | L.O. trans. pump | Suction | 1 | Simplex | 32 |  |  |
|  | Cylinder L.O. transfer pump | Suction | 1 | Simplex | 32 |  |  |
| Water | Sludge pump | Suction | 1 | Simplex | 4 mm | Galv. steel | Stainless steel |
|  | Sea chests | Suction | 1 each | Simplex | 5 mm |  |  |
|  | Daily bilge pump | Suction | 1 | Simplex | 4 mm | Galv. steel | plate |
|  | Fresh water pumps | Suction | 1 each | Simplex | 24 | Galv. steel | Stainless steel wire |

6.11	 PIPING SYSTEMS IN ENGINE ROOM

6.11.1	COOLING SEA WATER SYSTEM

The system shall consist of three (3) sets of main cooling S.W. pump, two (2) sets of sea chest, scrubber sea water pump, two (2) sets of central cooler and valves, fittings and pipes.

The three (3) main cooling sea water pumps shall be of same model. Two (2) sets shall be operated at normal sea going and the third shall be as stand-by. Each one of them can be used for harbor service.

Main cooling S.W. pumps shall take suction from sea through sea chests stop valves and strainers and discharge to central coolers, then overboard or recirculation to high sea chest in engine room.

Two strainers at outlets of sea chests to be provided, made of steel plates and the inside to be galvanized. The bucket to be made of SUS with a mesh size of 5 mm diameter.

The back washing piping and inserted filter for central coolers shall be arranged.

Pump outlets to have plate non return valve and butterfly shut off valves.

The condenser of F.W. generator shall be directly cooled by sea water.

6.11.2	COOLING FRESH WATER SYSTEM

The system shall consist of L.T. cooling F.W. circuit and H.T. cooling F.W. circuit. The L.T. cooling F.W. circuit includes three (3) sets of L.T. cooling F.W. pump, L.T. F.W. expansion tank, valves, fittings and pipes. The H.T. cooling F.W. circuit includes two (2) sets of M/E jacket cooling F.W. pump and one (1) set of M/E jacket cooling F.W. cooler, M/E H.T.F.W. expansion tank and accessories etc.

The three (3) L.T. cooling F.W. pumps shall be of same model. Two (2) sets shall be operated at normal sea going and the third shall be as stand-by. Each one of them can be used for harbor service.

The various consumers for L.T. circuit listed as follows:

(1)	M/E air cooler

(2)	M/E L.O. cooler

(3)	M/E jacket cooling F. W. cooler

(4)	G/E air coolers and L.O. coolers

(5)	Main air compressor

(6)	Intermediate shaft bearing

(7)	Air conditioning plant

(8)	Packaged air cooler for galley, ECR

(9)	Ref. condensers

(10)	 Atmospheric condenser

(11) Sample cooler for boiler

(12) MGO cooler in M/E & G/E F. O. supply modular unit

The L.T. F.W. temperature at central cooler outlet shall be designed of 36℃. The H.T. fresh water shall be used for heating of F.W. generator.

The electronic type temperature regulating valves shall be provided for H.T. and L.T. fresh water circuits.

Feed water shall be auto supplied by float valve to M/E H.T. F.W. expansion tank and L.T. F.W. expansion tank from fresh water service system of the F.W. hydrophore tank.

One (1) set of M/E jacket F.W. pre-heater shall be provided in H.T. fresh water circuit, which capacity to be 1% of main engine SMCR.

G/E H.T. F.W. pre-heater unit shall be provided according to the manufacturer’s recommendation.

6.11.3	LUBRICATION OIL SYSTEM

6.11.3.1	L.O. Filling and Transfer Systems

One (1) each separate deck filling lines with stop valve and blank flanges shall be arranged on bunker station port & stb side. for main engine and generator engine system oil. The main engine cylinder oil filling line shall be arranged on bunker station port & stb. side.

The lub. oil transfer pump shall be arranged to transfer L.O. between the lub. oil tanks in engine room or to discharge overboard via “on deck” filling connections.

A simplex filter shall be provided at suction side of lub. oil transfer pump.

6.11.3.2	Lub. Oil Service System

Lubrication of the M/E main bearings, thrust bearings, bottom-end bearings, crosshead bearings, together with piston cooling, is carried out by the main lubricating oil system.

The lubricating oil pump shall draw from the lub. oil sump tank and through LO cooler, a thermostatic control valve and auto. back wash filter or a by-pass filter then enter to the main engine. Lubricating oil shall flow out directly from engine crankcase into the lub. oil sump tank after lubrication or piston cooling.

M/E turbocharger shall be lubricated from the main engine system oil.

The lub. oil sump tank shall be arranged in double bottom.

Cylinder oil from cylinder oil storage tank(s) shall be transferred to cylinder oil service tank by a pump and a high level auto cut off for the pump to be provided.

Cylinder lubricating oil shall be fed to the main engine by means of a pump station mounted on the engine from the cylinder oil service tank.

Each generator engine shall have its own internal lubrication oil system with components built on the engine.

Lub. oil cooler shall have a thermostatic valve for temperature control. The return of the back flush filter shall be arranged close to the purifier suction and far from the main L.O. pump suction.

6.11.3.3	L O. Purifying System

The main engine and the generator engines shall have independent L.O. purifying system.

The main engine L.O. purifying system shall have one (1) set ME L.O. purifier. Lub. oil purifier shall be connected so that purification may be carried out from the main engine lub. oil sump tank or the lub. oil settling tank to the lub. oil settling or the main engine sump tank.

The generator engines L.O. purifying system shall have one (1) set AE L.O. purifier. Lub. oil purifier shall be connected so that purification may be carried out from the G/E or the lub. oil settling tank to the lub. oil settling tank or G/E.

Sludge and water from the purifier shall be led to the sludge tank, and shall be shifted to waste oil tank for incinerator or to shore by the sludge pump.

6.11.3.4	Lub. Oil Drain

Dirty oil collected from the M/E scavenge box is led to the M/E scavenge box drain tank and then to waste oil tank.

Save-all drip pans shall be provided for lub.oil pumps, purifiers and tanks, and shall be led to the waste oil rank.

6.11.4	FUEL SYSTEM

Fuel oil system including the equipment such as purifiers, F.O. supply unit, heating coils, etc. to be designed for operation with HFO of 380 cSt at 500C.

6.11.4.1	Fuel Oil Filling and Transfer System

The FO filling line shall be led to the HFO , LSHFO/ULSFO bunker tank and MGO bunker tank.

Each FO filling connection shall be provided with shut off valve, blank flange, pressure gauge, thermometer  and sample taking device

One safety valve for HFO and LSHFO/ULSFO filling line to protect filling pipe shall be provided and to be connected to F.O. overflow tank. One safety valve with same function shall be provided for MGO filling line.

Overflow during filling shall be routed to overflow tank with a high level alarm.

One (1) sets of HFO/LSHFO/ULSFO transfer pump in the engine room shall be arranged to transfer HFO/LSHFO from bunker tanks to the engine room HFO and LSHFO/ULSFO settling tanks.

Automatic start/stop pumping signal of HFO and LSHFO/ULSFO settling tanks shall be provided for HFO/LSHFO/ULSFO transfer pump by level switches.

The HFO/LSHFO transfer pump shall have suction also from HFO/LSHFO service tanks.

One (1) set of MGO transfer pump in the engine room shall be arranged to transfer MGO from MGO bunker tank to MGO service tank.

A simplex filter shall be provided at the suction side of each transfer pumps.

Manual changed over from LSHFO/ULSFO settling tank to HFO settling tank. And automatic start/stop pumping. Long time run stop function will be supplied.

6.11.4.2 	Fuel Oil Service System

6.11.4.2.1	Main Engine and Generator Engines System

One (1) common F.O. supply module unit shall be provided for main engine and generator engines. The unit consists of two (2) fuel oil supply pumps, two(2) fuel oil circulating pumps, two(2) fuel oil heaters, one(1) auto back flushing filter, one(1) viscosity control equipment, one(1) venting tank and etc.

Fuel oil supply pump shall draw fuel oil from the HFO and LSHFO/ULSFO service tank or MGO service tank through a strainer and then discharge to main engine through a flow-meter, F.O. circulating pumps, the heaters (for HFO and LSHFO/ULSFO) and an automatic back flush filter (to be installed on the hot side). The heaters shall be fitted with automatic regulating controller, governed by a viscometer. Excessive fuel oil from main engine shall be led to the fuel oil circulating pump suction side through a venting tank.

An emergency MGO pump shall be fitted in F.O. supply module unit for emergency use, the power source of the MGO pump shall be from emergency switchboard. And the emergency MGO supply line shall be separate from the main fuel oil line.

The automatic change-over device between HFO and LSHFO/ULSFO and MGO shall be arranged in the FO supply module.

6.11.4.2.2 Composite Boiler

Two (2) sets of composite boiler FO supply pump shall be provided and arranged near the LSHFO/ULSFO service tank.

LSHFO/ULSFO for composite boiler shall be drawn from the LSHFO/ULSFO service tank by the composite boiler fuel oil supply pumps and discharge to the burner of the oil fired section.

A connection of MGO shall be fitted at the suction side of the supply pump for cold starting.

During Vessel in emission control area, boiler’s fuel oil supply pump shall draw ULSFO/MGO from ULSFO/MGO service tank.

One (1) F.O. volume flow meter shall be provided.

6.11.4.3	Fuel Oil Purifying System

Two (2) sets of FO purifier shall be provided; one set of them shall be operated and the other as stand-by.

HFO and LSHFO/ULSFO after settled in the settling tank shall be drawn by the suction pump of fuel oil purifier and shall be delivered to the purifier through the fuel oil heater.

After purification, the clean oil shall be discharged into HFO and LSHFO/ULSFO service tank.

Overflow pipe from the bottom of the service tank back to settling tank shall be provided.

Sludge and water from purifier shall be discharged into the sludge tank for incinerator or to shore by the sludge pump.

6.11.4.4	Fuel Oil System besides the above

For the waste oil incinerator, MGO transfer line from the incinerator MGO tank shall be provided.

6.11.4.5	F.O. Drain

Save-all drip pans shall be provided for pumps, purifiers and tanks and shall be led to the fuel oil drain tank.

6.11.5	COMPRESSED AIR SYSTEM

Compressed air to be provided to the various services at the following pressure:

3 MPa	:	Main engine starting

Generator engine starting

0.7 MPa	: Miscellaneous usage including air horn

control air, including the safety air and maneuvering system

Deck service

0.55MPa	:	Hydrophore tank charging

0.2MPa	:	Sea chest

Main air reservoirs and aux. air reservoir shall be charged by main air compressors and emergency air compressor.

Deck service air reservoir shall be charged by the service air compressor.

Compressed air for starting the main engine shall be supplied from the main air reservoirs. Generator engines shall be supplied from the main air reservoirs or the aux. air reservoir.

The deck service compressed air to be supplied from service air reservoir and main air reservoirs via pressure reducing valve.

The pressure reducing valve shall be provided with a pressure relief valve and a pressure gauge at its outlet and the stand-by pressure reducing valve shall be provided.

Maneuvering and control air shall be supplied through one (1) pressure reducing valve and one (1) dehydrator from the main air reservoirs.

Separate airline shall be provided for air horns with locally open valve.

Air hose connections shall be provided at suitable locations in the engine room.

6.11.6	STEAM AND DRAIN SYSTEM

Following system shall be provided

a) Steam supply system

b) Condense system

Steam shall be supplied to various services at following pressure:

0.7 MPa:		HFO and LSHFO/ULSFO service and settling tanks heating

HFO,LSHFO/ULSFO heater for F.O. supply module unit

HFO,LSHFO/ULSFO &LO heaters for purifiers

Steam tracing

Other tank heating and heaters (in engine room)

0.4MPa: 		Calorifier

Air conditioning plant

Sea chests in engine room

Miscellaneous usage

Drain from HFO,ULHFO/LSHFO and lub. oil tank heating coil and oil heaters shall be led to the cascade tank through atmos. condenser and the inspection tank.

Drain from the calorifier and accommodation also shall be led to the cascade tank.

Drain traps with by-pass and check cock shall be provided for HFO,LSHFO/ULSFO and lub. Oil tank’s coil and oil heaters.

Escape lines for steam safety/relief valves shall be led to the funnel.

Flange type or screw type steam hose connections shall be provided for cleaning oil tanks, sea chest etc. in the engine room.

No shore steam connection shall be provided.

The inspection tank shall be integrated with cascade tank or separately arranged.

Excess steam from M/E exhaust gas section of the composite boiler shall be led to cascade tank through auto. steam dump and atmos. condenser.

6.11.7	FEED WATER SYSTEM

Condensate from atmos. condenser shall be discharged to cascade tank by gravity.

Feed water pump of the composite boiler shall draw water from the cascade tank and then discharge it to the composite boiler.

Feed water to cascade tank shall be manually from the F.W. hydrophore.

The bottom and surface blow down lines of the composite boiler shall be combined to led overboard.

The blow-down lines shall have double valves.

Each feed water pump shall have a capacity for max. evaporation when the main engine operating at SMCR and one (1) pump of them shall serve as stand-by.

6.11.8	SHIP'S SERVICE SYSTEM OF ENGINE ROOM

6.11.8.1	Bilge System

The engine room bilge system shall be served by two (2) sets of fire, bilge & G.S. pump, one (1) set of daily bilge pump and one (1) set of Clean Drain Tank Pump

One (1) set of dangerous goods bilge eductor for cargo holds shall be arranged in suitable place outside engine room, the pump shall be used for pumping the bilge of cargo holds when carry dangerous goods.

Each bilge well in engine room shall have a suction line with mud box and a stop check valve and shall be connected to the bilge main. The daily bilge pump shall take suction from the bilge main to the bilge holding tank. The oily bilge separator pump shall take suction from the oily bilge holding tank, and discharge overboard through the oily bilge separator and separated oil shall be automatically drained to the sludge tank by gravity.

The IBTS shall be complied.

Clean bilge in engine room shall be led to the clean drain tank, and discharge overboard by the Clean Drain Tank Pump. An independent 15 PPM monitor to be provided on pump discharge side.

Bilge suction shall be provided for double bottom cofferdams in engine room and main engine recess on tank top.

In addition to the suction from bilge main suction, one of the two fire, bilge & G.S. pumps shall have an independent bilge suction from the bilge well in the engine room.

The emergency suction line with non-return stop valve shall be connected to a ballast pump or a cooling sea water pump as per the Rule requirement.

Bilge high level alarm for each bilge well in engine room and cargo holds shall be provided. And before the suction pipe there is a mud box installed for each well.

In general, the valves in engine room bilge system shall be manual operation.

6.11.8.2	Water Fire Fighting System

6.11.8.2.1 	Sea Water Fire Fighting System

The fire main shall be served by the fire, bilge & G.S. pumps and emergency fire pump (in emergency case).

The fire hydrants with manual operation and hose boxes shall be arranged on platforms.

6.11.8.2.2 	Water Mist Fire Fighting System

The fixed low pressure water mist local application system shall be provided in engine room to protect following fire hazard space such as main engine, generator sets, boiler, fuel oil purifiers, fuel oil booster and incinerator.

The system consists of water pump unit, F.W. tank, water nozzles, pipes, alarm system and release station etc.

The system shall have both automatic release located outside of the engine room and local manual release.

Power for system must be routed through the Emergency Generator Switch Board.

6.11.8.3	CO2 Fire Fighting System

The system refers to Hull Piping Chapter.

6.11.8.4	Ballast System

Two (2) sets of ballast pump shall be used for ballasting and de-ballasting via ballast water treatment plant. Generally one pump to work, and the other to be stand-by. Ballast stripping shall be carried out by one (1) ballast stripping eductor.

The detail refers to Hull Piping Chapter.

#### 6.11.8.5	Fresh Water System

Fresh water shall be supplied by two (2) sets of fresh water pump with one (1) set of fresh water hydrophore tank.

Fresh water shall be supplied from hydrophore tank for domestic service in living quarters, technical fresh water service and calorifier in engine room.

The engine room technical water service shall be supplied directly from the fresh water hydrophore tank.

The fresh water pump to take suction from fresh water tank through a strainer and discharge to the fresh water hydrophore tank, fitted with limit pressure switch adjusted to start and stop the fresh water pump motor within a preset pressure range.

One of two fresh water pumps shall be used as stand-by.

Fresh water to main deck shall be provided with isolating valve and drain valve located at the lowest point (accommodation face).

The hot water shall be supplied by a calorifier with steam and electric heating.

One (1) set of F.W. sterilizer and one (1) set of mineralizing shall be provided for potable fresh water system.

Y- type filter shall be arranged before the F.W. sterilizer.

For fresh water system description, also refer to Chapter 6 - Hulling Piping , 6.1.8.

6.11.8.6	Sea Chests

There are one (1) high suction sea chest and one (1) low suction sea chest in engine room and one (1) suction sea chest in emergency fire pump room. Each suction sea chest shall be fitted with a sea valve of butterfly type.

Each suction sea chest shall be of welded steel construction. A steam and compressed air blow out connection and an air vent to be equipped for engine room sea chests. But only a compressed air blow out connection and an air vent to be equipped for the sea chest of emergency fire pump.

Each suction sea chest shall be equipped with grid, made of galvanized steel flat bars, with a clear area of at least two (2) times the area of the sea valves.

Grid shall be secured to the chest so that no part extends beyond the shell surface.

Fitting bolts and nuts of sea chest grids shall be of stainless steel and locked by stainless steel wire.

One (1) marine growth preventing system shall be installed in main SW strainers for main sea chests in engine room only.

6.11.9	EXHAUST GAS SYSTEM

Exhaust gas from the turbocharger of the main engine shall be led to the M/E exhaust gas section of the composite boiler , scrubber unit and discharged directly to open air.

No exhaust gas by-pass line to be provided for exhaust gas section of the composite boiler.

The oil fired section and incinerator shall be led up independently to open air.

Exhaust gas from G/E sets  engine shall be led up independently to scrubber unit through each silencer.

Stainless steel expansion elements for exhaust gas pipes of main and gen. engines shall be fitted immediately after turbocharger through transition connection.

Adequate supports shall be fitted so that the weight of the piping shall not rest on the turbocharger, boiler, incinerator, silencers and expansion elements, etc.

Recommend amend to Exhaust gas uptakes in the funnel to be insulated up to about at least 1,000 mm from the top plate of funnel.

Necessary drains from the exhaust gas pipes and silencers shall be arranged with drain traps and led to bilge well.

6.11.10	INSTRUMENTATION

Following instrument shall be provided for each machinery or on the piping side, and exclusive of the instruments used for automatic control in ECR.

6.11.10.1	Pressure, Compound and Vacuum Gauges

Pressure, compound and vacuum gauges shall be manufactured in accordance with the Chinese Industrial Standard or equivalent for marine use.

All gauge to have black figures, white faces, letter in English and units in metric or SI system (Red figures for vacuum), and working range shall be clearly marked for each gauge. Following gauges shall be furnished.

|  | Place of installation | No. of set | Diameter of gauge (mm) | Remark |
| --- | --- | --- | --- | --- |
| Water | Centrifugal pumps suction & delivery | 1 each | 100 |  |
|  | Reciprocating pumps suction & delivery | 1 each |  |  |
| Oil | Rotary pumps suction & delivery | 1 each |  |  |
|  | Oil strainer inlet & outlet | 1 each |  |  |
| Steam & Condensate | Boiler drum | 2 | 150 |  |
|  | Heaters (steam side) | 1 each | 100 |  |
|  | Steam line after reducing valve | 1 each | 100 |  |
| Compressed air | Air line after reducing valve | 1 each | 100 |  |
|  | Air reservoir | 1 each | 150 |  |
| Others | Various pressure gauge fitted at main engine, generator engines, main air compressors, and boiler burning set, etc. U-tube difference-pressure gauge fitted at exhaust gas section of composite boiler. | Assorted | Manufacturer's standard |  |

6.11.10.2  Thermometers

Thermometers shall be indicated in Centigrade and shall be manufactured in accordance with the International Standard or equivalent, and working range shall be clearly marked for each gauge. Thermometers of liquid in glass type with separable sockets shall be provided as follows.

|  | Place of installation |  | No. of set | Remarks |
| --- | --- | --- | --- | --- |
| Water | Fresh water cooler | Fresh water & sea water inlet & outlet | 1 each | Fresh water: after temp. control valve |
|  | Atmospheric condenser | Cooling water inlet & outlet | 1 each |  |
|  | Lub. oil cooler | Cooling water inlet & outlet | 1 each | Lub. oil: after temp. cont. valve |
| Oil | Heaters | Oil inlet & outlet | 1 each |  |
|  | Heated tanks | Oil pocket on tank wall or outlet | 1 each |  |
| Others | Various thermometers fitted at main engine, generator engines, air compressors, stern tube bearing, boiler |  | Assorted |  |
|  | Oil purifiers | Acc.to Maker's standard |  |  |

6.12    scrubber system

The SOX Scrubber system shall be of open loop type and shall be able to treat the exhaust gas from one (1) main engine and three (3) generator engines to achieve the 0.5%S (Option 0.1%S) equivalency to meet the “Global Sulphur Cap 2020”.

The open loop system is typically used in open waters where the alkalinity of the seawater is higher than 2,200μmol/l for effective scrubbing.

The SOx scrubber system is designed to remove sulphur oxides from main engine and generator engines exhaust gases. The removal efficiency for SOx of the open loop system is from max. 3.5%S to 0.5%S. (Option: to achieve the 0.1%S equivalency to meet the SECA requirement)

One(1) common scrubber unit with a combine box for ME and three(3) sets of GEs.

M.G.O. will be used as stand-by for main engine and generator engines when SOx scrubber system is fault, and when local regulations prevent or limit the use of the open loop system due to low alkalinity or restricted discharge criteria, the MGO could be used.

Boiler exhaust gas pipe will not connect to SOx scrubber system.

The open loop system uses pure sea water for neutralizing the exhaust gas. The water is taken from sea chest, pumped to the scrubber tower and distributed to the active spray nozzles. Drain is led directly via sea water overboard discharge line, the discharge criteria set by the IMO guidelines should be met (the PH should be equals or above 6.5 at a distance 4 meters from the discharge point.

The open loop SOx scrubber system capacity is based on the following operational profile:

- M/E SMCR

- Two (2) sets G/E at 85% load

The boiler is only connect to the scrubber tower, not enlarge the scrubber system’s capacity, and the boiler is only working in port.

The backpressure limit for M/E and G/Es will be increased as per maker’s standard, the fuel oil consumption may suffer penalty as per maker’s calculation, no exhaust gas fan is provided for M/E and G/E in principle.

Continuous emission monitoring system and water monitoring unit shall be provided, the materials, accessories and fittings will be in accordance with Maker’s standard and requirement of Classification Society.

The discharge pipes to be GRE and the last connection to the ship side should be duplex S32750

6.13	  SPARE PARTS AND TOOLS

All spare parts and special tools shall be supplied according to the Rules of Classification Society and manufacturer’s standard.

Normally only 1 set spare for each type equipment, such as Aux engines, just one set spare for three Aux engines.

The spares list will be supplied to the owner after the yard receive the approved drawing from maker. The list will be checked by yard, if have any deviation, the shipyard will inform the owner.

General tools and outfits for machinery part are supplied according to builder standard.

`

CHAPTER	7

AUTOMATION

Version:A

Key words: Breadth 27.5m, Gearless, 6S60ME-C10.5 Tier II, 220ReeferPlugs, Open loop scrubber

CONTENT

CONTENT	2

CHAPTER 7	AUTOMATION AND REMOTE CONTROL	3

7.1.	General	3

7.1.1.	Principle	3

7.1.2.	Control standS	3

7.1.3.	Control room	5

7.2.	Automation System	6

7.2.1.	Engine’s Alarm and Monitoring System	6

7.2.2.	Automation System	8

7.2.3.	Main Propulsion Control	8

7.2.3.1.	Main Engine Control System	9

7.2.3.2.	Protecting and safety devices	10

7.2.3.3.	Telegraph System	11

7.2.3.4.	Shafting Revolution Indicators	12

7.2.4.	Control of electric gneeratring plant	12

7.2.4.1.	Main generator sets	12

7.2.4.2.	Emergency diesel generator set	14

7.2.5.	Composite Boiler	14

7.2.6.	Control of Oil Purifiers	14

7.2.7.	Control of Air Compressors	15

7.2.8.	Scrubber Operation and Control	15

7.2.9.	Special Control Functions	16

7.2.10.	Local Automatic Control	16

7.2.11.	Miscellaneous Controls	16

7.2.12.	Running Lamps	17

7.2.13.	Remote Quick Closing Valve	17

7.2.14.	Ballast Control Station	17

CHAPTER 7	AUTOMATION AND REMOTE CONTROL

  - 7.1	General

    - 7.1.1 Principle

Automatic control, remote operation, instrumentation and central supervision shall be so designed and arranged that the propulsion plant is capable of being operated for 24 hours under normal sea-going condition without manual attendance in engine room to comply with the requirement for the ship with Classification notation (refer to Chapter 1 General 1.3.1).

One (1) control room shall be arranged in the engine room, where the control console of main engine and the main switchboard shall be installed.

The instrument gauge and alarm shall be arranged in good order for safety operation and easy maintenance.

Automatic combustion control and equipment concerned with the boiler shall be arranged adjacent to the boiler.

Automatic controllers and control valves shall be provided on principal systems for safety operation and easy maintenance.

The entire automation system shall consist of several decentralized control systems.

The decentralized control systems shall be used separately for main engine remote control, power management control, stand-by pump changeover control, and engine room monitoring and alarm system, ballast and bilge system.

The machinery and equipment not specified in this specification shall be served locally by the engine personnel on site, according to needs.

In case that the contents in the following parts of this section differ from Maker’s standard for specific equipment, Makers’ standard shall be applied in general.

UPS for AMS are to be provided. Duration of the batteries shall be minimum 30 min and tested during sea trials.

Serial and analog signals to AMS are to be isolated in equipment / device / sensor side as maker’s standard. Digital signals are to be voltage free contact signals.

    - 7.1.2 Control standS

The following control stands shall be provided:

-	Wheelhouse control console (WHC) including Bridge Maneuvering Console (BMC) and Navigation Signal Console (NSC) in wheelhouse

-	Engine control room console (ECC) in engine control room.

-	Ballast control station in ship office.

-	Local stands for local controls of machines.

-	Bridge wing control console at navigation bridge deck wings.

1.	Bridge maneuvering console

Refer to “Chapter 8 Electric”.

2.	Engine control room console

The console shall be of steel construction self-standing type, with hinged doors in front side and removable doors in rear side if applicable.

The console shall be supplied from main switchboard and emergency switchboard by means of each one feeder 3 x 220V AC, and from charging & discharging board by means of one feeder 24V DC.

Inside the console AC220 V socket outlets shall be provided.

Inside the console 2 pcs of AC220V 20W lights should be provided.

The following equipment shall be included in/on the console, but not limit to:

-	Main propulsion–main operation panel / safety panel

-	Main engine indication panel

-	Engine telegraph (receiver / transmitter)

-	2 pcs of PC workstation with 24  inch TFT VDU and other engine room monitoring and alarm equipment

-		Alarm & Data logging printer

-	Slave clock

-		Dead man alarm main switch or push button (combined in extension alarm panel)

-	Remote water level analogue or digital indicator for boiler

-	Main engine auxiliary blower, and start / stop switch with running indication lamp as per main engine Maker’s standard

-	Composite boiler emergency stop push button

-	Composite boiler running (firing on) lamp

-	Composite boiler steam pressure gauge

-  	Main engine RPM indicator and revolution counter

-	Main engine turbocharger RPM indicator

- Scrubber monitoring and shut-down control in emergency

-	Rudder angle indicator

-	Analogue measuring instruments for essential pressures and temperatures

-	Main air compressor auto/start/stop switch and running indication

-	Fuel oil viscosity indication

-  HFO, LSHFO and MGO change-over position indicator

-	Duty engineer selector

-	Auto. telephone

-	Speed log indicator

-	Sound power telephone

-	Suitable place for writing

-  Lubricator controller .

-  Temperature controllers of HT, LT & LO.

-  M/E running hour counter

Water mist alarm repeater panel

Manual emergency trip buttons with cover of necessary

-	Other necessary equipment specified elsewhere

3.	Ballast control station in ship office. Refer to item 1-14.

4.	Local stand for local control of main engine. Refer to “Chapter 6 Machinery”.

5.	Bridge wing console at navigation bridge deck wing. Refer to “Chapter 8 – Electric”.

    - 7.1.3 Control room

1.	Engine control room (ECR)

Engine control room surrounded with steel walls shall be sound insulated and air conditioned.

In the ECR the following equipment shall be provided:

-	Main switchboard

- Engine control console; One monitor for Valve remote control system along with tank level remote gauge shall be combined.

-	Uninterrupted Power Supply unit for automation system (UPS)

- Auto telephone and sound power telephone

-  Electronic modules and operator panels for M/E remote control system

-  ECR manual control panel of M/E

-  Fire alarm repeater panel

-	Air condition unit

-  Remote valve operation and tank sounding shall be possible from ECR and ship office.

-	Other necessary furniture specified elsewhere

2.	Ship office

In the ship office the following equipment shall be provided:

-	Ballast control station

-	Auto. telephone

-	AMS Extend alarm panel

-	Reefer container monitoring panel

-	Loading computer

- 1 pcs of 24 inch TFT VDU for ballast control and level measure system

- Anti-heeling system control

- Ballast water treatment system panel

- Cargo hold fans remote control and running indicator

  - 7.2 Automation System

    - 7.2.1 Engine’s Alarm and Monitoring System

The micro-processor aided integrated alarm and monitoring system (AMS) to be provided.

The system consists of the following equipment:

| Equipment | QTY. | Location |
| --- | --- | --- |
| PC workstation with24 inch TFT VDU | 2 | ECC |
| PC workstation with24 inch TFT VDU | 1 | WHC ( for monitoring ) |
| Alarm printer &Data logging printer | 1 | ECC |
| Extension alarm panel | 11 | Accommodation |
| Alarm lamp | * | ECC |
| UPS | * | ECC |

Comments: AT with mark “*”is specified hereinafter.

The system shall be based on a concept of totally distributed processing which various processes are controlled by input/output modules located near equipment. Ample hardwired I/O and serial I/O shall be provided as per Rule and Maker’s standard.

The system shall have the following functions:

- Data processing function

- Data storage function

- Data indicating function

- Alarm function

- Data logging function

- Alarm logging function

- Extension alarm function

1.	Main parts of Engine’s alarm and monitoring system (AMS) shall have redundant functionality, the system shall be based on computer with alarm, and monitoring signals connected to I/O units arranged in the engine room and in the control room or other spaces.

2.	All input channels shall be made in “self-check-off” principle

3.	The particular measured analogue data shall be continuously indicated on individual analogue indicator.

4.	Alarm shall be initiated when measured analogue data exceeds corresponding alarm preset analogue value as well as deviations from the mean value for the main engine exhaust gas.

5.	Common sound alarm in ECR with flashing light or indication in each particular channel shall signalize the alarm conditions. After acceptance the failure condition, the flashing light or indication signal shall be turned to steady one and sound signal shall be silenced.

The steady light shall be automatically cancelled after normal condition has been restored and the alarm signal has been acknowledged.

When the arrangement is stopped intentionally, the respective alarm circuits shall be switched off automatically.

6.	Each alarm condition shall be transmitted to the E/R alarm column system in ECR and engine room as well as to extension alarm system panels.

7.	The failure alarms shall be divided at least in the following groups:

a.	“Trip” group: alarms signaling faults, which require the propulsion system, shall be shut down immediately.

b.	“Reduce” group: alarms signaling faults, which require a power reduction in the propulsion system.

c.	“Common” group: alarms signaling faults which do not require actions as described in a or b.

d.  Individual alarm: The alarm shall be individual transmitted to extension alarm system, including the engine room fire, bilge level high, system power source fault etc.

8.	Extension alarm system panels with sound and light signals, installed in

-	Wheelhouse

-	Chief engineer's day room (with additional buzzer in bed room)

-	2nd, 3rd, 4th Engineer’s cabin

-	Engineer’s office

-	Officer’s mess room

-	Recreation room

-	Gymnasium

-	Ship office

-  Electrician’s cabin

The extension alarm system panels shall be able to indicate different alarm groups.

9.	Engine room “Dead man” alarm

The system shall be switched-on in activating boxes at two main ER entrances and shall be reset within preset time by reset push-buttons with indication lamp, situated in:

-	engine control room

-	engine room (6 pcs)

10.	Power supply arrangement

The entire system with operator stations I/O units shall be powered from 24V DC.

Note: The alarm and monitoring system (AMS) to be in accordance with rule requirement and may be changed according to final selected Maker’s standard.

    - 7.2.2 Automation System

All pneumatic valves / items for main engine control should be tagged by stainless steel name plate with valve position deeply etched on it. This should exactly match with control drawing.

Automation system consists of the following subsystems:

1.	DG Power Management System Refer to item 1-5.

2.	Automatic change-over of stand-by pumps

The following pumps to be automatically changed-over by low pressure or low level and abnormal trip:

-	Cooling S.W. pump

-	M/E jacket cooling F.W. pump

-	L.T. cooling F.W. pump

-	Composite boiler feed water pump

-	F.O. pumps attached to the F.O. supply module unit

-	Composite boiler F.O. supply pump

-	Main L.O. pump

3.	Automatic regulation of temperature and viscosity

The following equipment and systems to be provided with automatic regulation of temperature or/and viscosity.

-	HFO/LSHFO settling and service tanks (refer to Chapter 6 Machinery 6. 2.17)

-	Oil purifier’s heater (acc. to Maker’s recommendation)

-	F.O. supply module unit (acc. to Maker’s recommendation)

-	Generator engines (acc. to Maker’s recommendation)

-	Calorifier

-	M/E and G/E pre-heaters (acc. to Maker’s recommendation)

-	Following ancillary system for main engine (if necessary)

M/E jacket cooling F.W. circuit

Central F.W. coolers outlet

M/E L.O. cooler outlet

4.	Ballast/bilge/fuel oil transfer pumps and valves/control system

Refer to item 1-14.

5.	Automatic start and stop

HFO/LSHFO/ULSFO transfer pump

    - 7.2.3 Main Propulsion Control

Main propulsion control shall consist of main engine control system, safety device and engine telegraph system, Telegraph logger to be connected to master clock.

      - 7.2.3.1 Main Engine Control System

A circuit breakers as protective device shall be prepared for the control circuits in the switchboards.

1.	Local control

Manual, emergency controls from M/E local operating panel placed on the engine in engine room with following functions:

Start/stop, reversing, speed control, and the most important engine data displayed.

2.	Remote control from engine control room

Main operating panel shall be used for carrying out engine commands, adjust the engine parameters, select the running modes, and observe the status of the control system.

3.	Remote control from wheelhouse or navigation bridge deck wings

The following controls shall be sequentially performed according to a preset RPM time loading program by operating the engine telegraph in wheelhouse or navigation bridge deck wings:

Starting and stopping

Speed increasing and decreasing as per engine design

Reversing

The reversing and speed setting of M/E shall be controlled by engine telegraph lever which is used as maneuvering lever.

Main engine speed control shall be carried out through the electronic speed control system.

In case of the first starting failure, two or more staring attempts shall be automatically carried out (total three attempts). An alarm shall be given after three attempts without firing. Re-starting shall be carried out after setting the telegraph lever at the “STOP” position.

The auxiliary blower shall be automatically started and stopped by detecting the scavenging air manifold pressure, and also shall be capable of being started and stopped from engine control room.

4.	Control position transfer system

Engine running modes shall be done from the main operating panel in ECR.

Control shall be done from one station only at a time.

(1)	Local – engine control room

Maneuvering position change-over between the engine side and the engine control room shall be carried out at the engine side.

(2)	Engine control room – wheelhouse

Maneuvering position change-over between the wheelhouse and the engine control room shall be carried out in the engine control room.

(3)	Wheelhouse – navigation bridge deck wings

Maneuvering position change-over between the wheelhouse and the both bridge wings shall be carried out in the wheelhouse.

      - 7.2.3.2 Protecting and safety devices

a.	Emergency trip

Main engine shall be automatically tripped with an alarm according to Rules and propulsion plant manufacturer's requirements.

Each one manual emergency trip push button with cover shall be provided in engine control console, local emergency control station, navigation bridge deck wings and W/H control console.

Emergency trip system shall be independent of the remote control system and separated from Engine’s alarm and monitoring system.

Reset of trip circuit shall be carried out by setting the maneuvering/telegraph lever in engine control room at stop position after recovering to normal condition.

b.	Critical speed protection

Automatic quick passing program shall be applied over the critical revolution zone during bridge control (if any).

Critical speed alarm to be provided in AMS for the long time running.

c.	Program speed control

When controlled in wheelhouse, the engine speed shall be controlled by load-up / load down program over certain ahead speed by program device with readjusted time up to the set speed of the maneuvering / telegraph lever.

One override push button for program speed up shall be provided in each of wheelhouse control console and navigation bridge deck wings.

d.	Turning interlock

While the turning gear is engaged, a starting pilot air valve for starting shall be locked.

e.	Automatic slow-down

The M/E speed shall be automatically reduced according to Rules and propulsion plant Manufacturer's requirements.

One by-pass push button of this system shall be provided in each of W/H control console and navigation bridge deck wings or as per maker standard.

Reset of the automatic slow down circuit shall be carried out by setting the telegraph lever in selected main engine control position below a predetermined position after recovering to normal condition.

Alarm and separate indicator for above as well as emergency trip shall be provided on engine control console in engine control room and W/H control console.

      - 7.2.3.3 Telegraph System

Telegraph system shall consist of engine telegraph, sub-engine telegraph and telegraph logger.

1.	Engine telegraph

One (1) set of engine order telegraph system shall be provided, and it is to consist of:

1 -	transmitter with receiver with buzzer and illuminating lamp and dimmer in wheelhouse control console

1 –	transmitter (remote control lever) with buzzer and illuminating lamp and dimmer in each of navigation bridge deck wings

1 –	receiver / reply with bell in engine control console

1 –	receiver / with gong in engine side

During wheelhouse control or navigation bridge deck wing control of main engine, the buzzer shall sound for abt. 2 seconds when the order partition is changed.

2.	Sub-engine telegraph

One (1) set of sub-engine telegraph of push button with lamp type shall be provided in wheelhouse and engine control room.

Partitions of the sub-telegraph shall be as follows:

AT SEA

STAND-BY

FINISHED WITH ENGINE

Audible devices of the sub-engine telegraph shall be one buzzer in each of wheelhouse and engine control room.

3.	Telegraph logger

One (1) telegraph logger shall be provided in wheelhouse. The following items shall be recorded:

Engine telegraph order & reply

Sub-engine telegraph order & reply

Time

The recorded time shall be synchronous with Master clock.

The achieved revolutions and time corresponding to the revolution

Note: The M/E remote control system and telegraph system to be in accordance with rule requirement and may be changed according to final selected Maker’s standard.

      - 7.2.3.4 Shafting Revolution Indicators

One (1) set of shafting revolution-indicating system shall be provided and installed at

following places.

One (1) pick-up type transmitter on M/E flywheel.

One (1) flush type RPM indicator with an illumination on wheelhouse ceiling console with

separate dimmer switch on W/H control console.

Each wall type RPM indicator (with dimmer) on each bridge wings.

One (1) flush type RPM indicator on the engine control console and Chief Engineer’s day

room.

One (1) electric or mechanical type indicator at main engine machine side according to

engine Maker's standard.

One (1) shaft revolution counter of 8digits shall be provided on engine control console.

    - 7.2.4 Control of electric gneeratring plant

      - 7.2.4.1 Main generator sets

The main generator sets shall be either manually remote controlled or automatically control, and also be capable of being started and stopped locally. Three(3) engine generators shall be capable of being set in stand-by with the selecting switch on main switchboard.

- Manual control

The generator's engines shall be provided with remote manual starting and stopping facilities, EMCY stop and speed control switch in MSB, and local start/stop shall be also provided.

Manual synchronizing shall be available by means of synchronoscope and synchronizing lamps and the governor control switches for diesel generators on main switchboard.

Manual load sharing shall be carried out with the governor control switches for generators on main switchboard.

- Automatic control

The main generating plant shall be automatically controlled by means of Power Management System (PMS) and safety system i.e.

-	Automatic start/stop

-	Monitoring of critical parameters

-	Safety system with shutdown

-	Automatic synchronizing

-	Automatic load sharing

-	Frequency control

-	Load dependent start / stop

-	Bow thruster start inquiry

1)	Automatic start and stop of stand-by generator

When loading of a running generator is over about 90% of rated load, the predetermined generator shall be started, synchronized and the circuit breaker of started generator shall be closed and the load shall be shared automatically.

When two (2) or more generators are running in parallel and the total power is below about certain percentage of one generator’s rated load, the predetermined generator in running shall shift the load and the circuit breaker shall be opened and the engine shall be stopped after a time delay automatically.

Stand-by generator shall be automatically started in the following conditions:

(1)	Power failure of main bus

(2)	Low frequency of main bus

(3)	Low voltage of main bus

(4)	High voltage of main bus

(5)	Trip of any air circuit breaker of running generators

(6)	Over load (when one generator is running)

2)	In case of above items (1) to (4)

The circuit breaker of running generator shall be automatically opened, and the circuit breaker of started generator shall be closed.

Then, some essential electric equipment formerly operated shall be automatically connected in sequence.

3)	In case of above items (5) and (6)

The started generator shall be automatically synchronized and connected to main bus, and then the loads of generators shall be automatically shared.

The stand-by set shall be pre-lubricated according to Maker’s requirement. Automatic/manual selecting switch for pre-lubricating pump of each set shall be arranged in the starter.

When main generators operate in parallel and one of them fails, the preference trip system to be initiated and trip the non-essential loads include Bow thruster. The non-essential consumers shall be manually restarted after tripping.

In case of “black-out”, the stand-by set shall be started automatically.

Interlock between PMS operation and Generator standby system to be provided to prevent PMS selection when no generator is put on standby.

      - 7.2.4.2 Emergency diesel generator set

The emergency generator set shall be automatically started, the bus tie breaker in emergency switchboard shall be automatically opened, and the air circuit breaker shall be automatically closed in case of failure in supply feeder from main switchboard.

And then, when electric source on main bus is recovered, the air circuit breaker shall be automatically opened and the engine shall be stopped after a time delay automatically, and the bus tie breaker shall be automatically closed.

Manual start and stop of the generator set shall be carried out on site.

The automatic and manual control system shall be according to the Maker’s standard.

    - 7.2.5 Composite Boiler

1.	Automatic combustion control

The fuel oil burning set for oil fired section of the composite boiler shall be provided with electric modulating controlling system.

The burner shall be automatically lit and extinguished according to the boiler steam pressure.

The burner shall be turned off automatically according to the rules. The manual stopping device shall be provided in ECC.

2.	Automatic control of surplus steam

When the steam pressure generated by M/E exhaust gas section of the composite boiler is over the predetermined limit, the surplus steam shall be led to the atmospheric condenser automatically through a surplus steam dump valve.

3.	Automatic control of boiler water level

Water level of the boiler shall be regulated automatically by level switch on the boiler.

4.	Alarm and protection

The alarm signals for pressure, temperature and combustion shall be connected to alarm and monitoring system (AMS) in compliance with the Rules and Maker’s standard.

    - 7.2.6 Control of Oil Purifiers

All purifiers shall be manually ON/OFF control on site operation and cleaning process –fully automated.

Alarm and recirculation of oil feed to purifier upon loss of seal shall be provided for the purifier according to Maker's standard. Feed oil will be re-circulated by three way shifting valve, as per Maker’s standard.

All oil purifiers shall be designed to have manual emergency stop function, the emergency stop button shall be locate at the suitable place by the designing.

Running indication shall be in ECC. Alarm shall be in alarm and monitoring system (AMS). Auto. sludge discharging shall be performed.

Hour counter for each purifier shall be provided or displayed in control board.

    - 7.2.7 Control of Air Compressors

1.	Starting of main air compressors

The starting of main air compressors shall be automatically ON/OFF controlled, depending on pressure of common inlet pipe of the main air reservoirs in the following sequence:

-	At abt. 2.8 MPa – start the first leading main air compressor

-	At abt. 2.6 MPa – start the second leading main air compressor (operating both compressors)

-	All compressors stop at 3.0 MPa

Any compressor can be selected as first leading, second leading.

The main air compressor shall be automatically started and stopped by detecting the pressure of common inlet pipe of main air reservoirs. The relocation of start/stop setting points shall be possible.

A common failure alarm for automatic tripping shall be sent to alarm and monitoring system (AMS). Safety installation for the compressors shall be provided in accordance with the maker's recommendation.

Hour counter of each compressor shall be provided in control board.

A remote control for the main air compressors shall be arranged.

Main air compressor auto/start/stop switch and running indication to be provided in ECC

2.	Control air treatment station

Control air installation shall be supplied from air treatment station with pressure 0.8MPa

The air treatment station shall be supplied from main air reservoir by means of reduction valve 3.0 / 0.8 MPa and consists of:

-	Air dehydrator

-	Reduction valve and safety valve

-	Pressure gauge at each pressure output

    - 7.2.8 Scrubber Operation and Control

The operation and control system shall be the type of programmable logic controller (PLC) Following parameters shall be continuously monitoring for the cleaned exhaust gas and water emissions:

- Wash water turbidity, pH value, etc. of outlet;

- CO2/SO2 analysis measuring for exhaust gas outlet

- ME/AE power signal and GPS signal shall be supplied by Yard for logging

- etc.

An operating station shall be in the ECC for the system operation.

    - 7.2.9 Special Control Functions

The important equipment, if formerly operated, shall be automatically, sequentially restarted when electric power on main bus is recovered after “black-out”.

    - 7.2.10 Local Automatic Control

The following auxiliaries to be operated in local automatic system:

- Fresh water generator abnormal alarm shall be connected to AMS and high salinity alarm in monitoring system.

- Salinometer shall be provided with automatic control to re-circulate distillate on high salinity.

- F. W. pumps shall be automatically operated in dependence on the tanks pressure

- Oily bilge separator with failure/15ppm alarm in alarm and monitoring system (AMS).

- Incinerator with failure alarm in alarm and monitoring system (AMS).

- Sewage treatment plant with failure alarm in alarm and monitoring system (AMS).

- Ballast water treatment plant with failure alarm in alarm and monitoring system and remote control from ships office.

    - 7.2.11 Miscellaneous Controls

- Start/stop control of fire, bilge & G.S. pump in W/H control console and machine side and in fire control station also.

- Engine room ventilation fans – start / stop control in MSB.

- START / STOP control of sludge pump.

- Engine room bilge pumps and valves – locally controlled.

- Remote emergency stop.

- Remote start-stop switches and running indicating lamps for cargo hold ventilating fans shall be installed in wheel house along with one common isolation switch (key type) to isolate non-Ex. Proof vent fans during dangerous cargo loading through the AMS mimic diagram..

Emergency stop push button shall be arranged near shore discharge station for sludge pump and bilge pump.

Ventilating fans and oil pumps shall be stopped in an emergency case of fire.

-	Ventilating fans in engine room, fuel and lub. oil pumps, boiler, purifiers, incinerator in engine room shall be remote stopped. Emergency stop push-buttons shall be provided in fire control station, wheelhouse and entrance of engine room.

-	Separate emergency stop push-buttons of ventilation fans for accommodation and ventilation fans for cargo hold shall be installed in wheelhouse and fire control station.

-	Boiler plant and incinerator can be emergency stopped in engine control room. In addition, Boiler Running, boiler water levels and steam pressure indications are to be provided in the ECR console

Builder shall prepare a list to Buyer for the memory batteries of individual equipment control circuit, which shall including the reference data of type and replacement dates etc. that necessary for the batteries replacement work.

    - 7.2.12 Running Lamps

As a Rule, machinery controlled from MSB or ECC shall be equipped with running lamps there.

    - 7.2.13 Remote Quick Closing Valve

Quick closing facilities for pneumatic remote closing valves in fuel oil and lub. oil systems shall be provided.

Low pressure alarm shall be provided to AMS.

    - 7.2.14 	Ballast Control Station

Entire system shall be based one (1) pc of PC workstation with 24 inch TFT in engine control room, one (1) pc of PC workstation in ship office.

The following system shall be provided in above monitor:

-	Ballast control system

-	Fuel oil filling control system (only indication)

-	Valves remote control system

-	Level and draught gauging system

-	Bilge control system.

All measured data in level and draught gauging system shall be transmitted to loading computer via interface.

1.	Ballast system

Display of mimic diagram of the following:

-	Valve control for remote control valves and open/shut indication

-	Start/stop control for ballast pumps

-	Running indication for ballast pumps

-	Tank level and content display (water ballast tanks and fresh water tanks)

-	Ship's draught display

2.	Fuel oil filling control system

The fuel oil filling system shall be remote controlled from engine control room as follows:

-	Remote control of HFO/LSHFO transfer pump

-	Running indication for HFO/LSHFO transfer pump and MDO/MGO transfer pump

-	Remote control valves in fuel oil filling system and open/shut indication

-	Remote level measurement in fuel oil tanks and alarm of overflow and filling up

Start / stop push buttons shall be installed near the HFO/LSHFO transfer pump, MDO/MGO transfer pump separately.

Stop push-button for HFO/LSHFO transfer pump in bunker station to be provided. The auto start/stop shall be applied to HFO/LSHFO settling tank by high/low level switches.

3.	Valves remote control system

The valve remote control system shall be of pneumatic type.

The valve relay cabinet(s) shall be installed in engine room and cargo hold.

All remotely controlled valves shall have an emergency operation facility.

4.	Level and draught gauging system

The system shall consist of electronic pressure transmitter type.

Installation of tank remote level measuring able to display tank volumes shall be provided in:

-  F.O. tanks and L.O. tank in E/R (refer to part VII-Machinery, paragraph 2-17)

-  Ballast tanks

-  F.W. tanks

5.	Bilge control system

The bilge control system shall ensure:

-  Opening and closing of bilge system valves outside the engine room

-  Alarm in the bilge wells of cargo hold when filled up

6.	Anti-heeling system

One (1) set of separated anti heeling system shall be provided; the common alarm shall be connected to AMS.

The anti-heeling system includes as follows:

- Anti-heeling pump

- Heeling sensor and control unit in ship office

- Anti-heeling control valves

Control functions and indications shall be provided as bellows:

- Start/Stop function for automatically heel adjusting

- Start/Stop function with running indication for heeling pump

- Lower level alarm and stop for heeling pump

CHAPTER 8

ELECTRIC

Version:A

Key words: Breadth 27.5m, Gearless, 6S60ME-C10.5 Tier II, 220 Reefer Plugs, Open loop scrubber

CONTENT

CHAPTER 8	ELECTRIC PART	6

8.1 	GENERAL	6

8.1.1.	GENERAL	6

8.1.2.	ARRANGEMENT OF EQUIPMENT	7

8.1.3.	NAME PLATE	7

8.1.4.	PAINTING COLORS	8

8.1.5.	MARK AND ARRANGEMENT OF BUS BAR	8

8.1.6.	EQUIPMENT AND MATERIAL	9

8.2.	WIRING	10

8.2.1.	WIRING SYSTEM	10

8.2.2.	CABLE APPLICATION	11

8.2.3.	CABLE INSTALLATION	12

8.3.	PRIMARY SOURCE	13

8.3.1.	GENERAL	13

8.3.2.	MAIN GENERATORS	13

8.3.3.	EMERGENCY GENERATOR	14

8.3.4.	MAIN SWITCHBOARD	15

8.3.4.1.	General	15

8.3.4.2.	Generator control panel	17

8.3.4.3.	Synchronizing Panel	17

8.3.4.4.	AC440 V Feeder Panel	18

8.3.4.5.	AC 220 V Feeder Panel	18

8.3.4.6.	Bow Thruster Feeder Panel	19

8.3.4.7.	Group Start Panels	19

8.3.5.	EMERGENCY SWITCHBOARD	19

8.3.5.1.	Generator Panel	20

8.3.5.2.	AC 440 V Feeder Panel /AC220 V Feeder Panel	20

8.3.6.	SHORE CONNECTION BOX	20

8.3.7.	TEST PANEL	21

8.4.	SECONDARY SOURCE	22

8.4.1.	TRANSFORMERS	22

8.4.2.	STORAGE BATTERIES AND BATTERY CHARGING BOARD	23

8.4.2.1.	For General Use and for Engine Room Area	23

8.4.2.2.	Other	23

8.5.	DISTRIBUTION SYSTEM	24

8.5.1.	ELECTRIC DISTRIBUTION	24

8.5.2.	SECTION AND DISTRIBUTION BOARDS	24

8.5.3.	CONNECTION BOXES AND BRANCH BOXES	25

8.5.4.	RECEPTACLE FOR REEFER CONTAINER	25

8.6.	ELECTRIC MOTOR AND CONTROL GEAR	25

8.6.1.	GENERAL	25

8.6.2.	MOTORS	26

8.6.2.1.	General	26

8.6.2.2.	Insulation and terminal box	26

8.6.3.	STARTERS	26

8.6.3.1.	General	26

8.6.3.2.	Ammeters and Running Hour Counters	27

8.6.3.3.	Emergency Stop Device	27

8.6.3.4.	Steering Gear Motors	28

8.6.3.5.	Bow Thruster Motor and Control	28

8.6.3.6.	Cargo Hold Fans Control	29

8.6.3.7.	Preferential Tripping Arrangement	29

8.6.4.	ELECTRIC WARMER FOR ROOM (AC 220 V)	30

8.7.	LIGHTING SYSTEM	30

8.7.1.	GENERAL	30

8.7.2.	LIGHTING – ACCOMMODATION AREA AND PASSAGE	32

8.7.3.	LIGHTING – MACHINERY SPACE	33

8.7.4.	LIGHTING – WHEELHOUSE	33

8.7.5.	LIGHTING – STORE	34

8.7.6.	LIGHTING – CARGO HOLD	34

8.7.7.	OUTDOOR FLOOD LIGHTING	34

8.7.8.	PORTABLE LAMPS	35

8.7.9.	CONTROL OF LIGHTING	35

8.7.10.	SWITCHES	36

8.7.11.	RECEPTACLES	36

8.7.11.1.	Non-water-proof Type Receptacles	36

8.7.11.2.	Water-proof Type Receptacles	36

8.8.	NAVIGATION LIGHTS AND SIGNAL LIGHTS	37

8.8.1.	NAVIGATION LIGHTS	37

8.8.1.1.	Anchor Lights: (AC 220 V)	37

8.8.1.2.	"Not-Under-Command" Lights: (AC 220 V)	37

8.8.1.3.	Manoeuvring/Morse Signal Lamp: (AC 220 V)	37

8.8.2.	Signal lights	38

8.8.2.1.	Daylight Signal Lamp (DC 24 V)	38

8.8.2.2.	SUEZ CANAL SIGNAL LIGHTS & SEARCHLIGHTS (AC 220 V)	38

8.8.2.3.	SUEZ CANAL STERN LIGHT (AC 220 V)	38

8.8.2.4.	DANGEROUS CARGO LOADING LIGHT (AC 220 V)	38

8.8.2.5.	Panama CANAL Stering LIGHT (AC 220 V)	38

8.8.2.6.	Singapore strait lighT (AC 220 V)	38

8.9.	INTERIOR COMMUNICATION, ALARM AND MEASURING SYSTEM	39

8.9.1.	8.9.1 SOUND POWER TELEPHONE	39

8.9.2.	AUTOMATIC TELEPHONE (AC 220 V & DC 24 V)	39

8.9.3.	HOSPITAL CALL SYSTEM (DC 24 V)	40

8.9.4.	ENGINEER CALL SYSTEM (AC 220 V)	40

8.9.5.	Bridge navigation WATCH alarm SYSTEM（BNWAS）	40

8.9.6.	Refrigerated Chamber ALARM	40

8.9.7.	ENGINE ROOM CO2 ALARM	40

8.9.8.	EMERGENCY GENERAL ALARM SYSTEM	41

8.9.9.	E/R SIGNALING AND ALARM SYSTEM (LIGHT COLUMN)	41

8.9.10.	ELECTRIC TACHOMETER FOR MAIN ENGINE	42

8.9.11.	RUDDER ANGLE INDICATOR	42

8.9.12.	PUBLIC ADDRESS WITH TALK BACK SYSTEM	43

8.9.13.	SHIP'S HORNS	44

8.9.14.	FIRE DETECTING AND ALARM SYSTEM	44

8.9.15.	FIXED GAS DETECTION SYSTEM	45

8.9.16.	ELECTRIC CRYSTAL CLOCK	45

8.9.17.	PC LOCAL AREA NETWORK	46

8.9.18.	E/R local water mist alarm system	46

8.10.	NAUTICAL INSTRUMENT	46

8.10.1.	Magnetic Compass (AC 220 V, DC 24 V)	46

8.10.2.	GYRO COMPASS (AC 220 V, DC 24 V)	47

8.10.3.	AUTO PILOT (AC 220 V, DC 24 V)	47

8.10.4.	ECHO SOUNDER (AC 220 V)	48

8.10.5.	SPEED LOG (AC 220 V)	48

8.10.6.	RADAR PLANT (AC 440 V/AC 220 V)	48

8.10.7.	ELECTRONIC CHART DISPLAY & INFORMATION SYSTEM (AC 220 V)	49

8.10.8.	DGPS NAVIGATOR	49

8.10.9.	ANEMOMETER and ANEMOSCOPE (AC 220 V)	50

8.10.10.	WINDOW WIPER and CVS (AC 220 V)	50

8.10.11.	VOYAGE DATA RECORDER VDR (AC 220 V, DC 24 V)	50

8.10.12.	AUTOMATIC IDENTIFY SYSTEM (ais) (DC 24 V)	51

8.10.13.	WHEELHOUSE CONTROL CONSOLE (WHC)	51

8.10.13.1.	Navigation Signal Console:	51

8.10.13.2.	Bridge Maneuvering Console (BMC)：	52

8.10.14.	Docking working station	52

8.10.15.	MEASURING INSTRUMENT PANEL	53

8.10.16.	CHART TABLE	53

8.10.17.	RADIO TABLE	53

8.10.18.	STEERING STAND	53

8.11.	RADIO COMMUNICATION EQUIPMENT	54

8.11.1.	GENERAL	54

8.11.2.	MF/HF RADIO EQUIPMENT (AC 220 V, DC 24 V)	54

8.11.3.	VHF RADIO TELEPHONE	54

8.11.4.	SATELLITE COMMUNICATION SYSTEM	54

8.11.5.	NAVTEX RECEIVER	55

8.11.6.	UHF handset	55

8.11.7.	WEATHER FACSIMILE RECORDER	55

8.11.8.	8.11.7 SATELLITE EPIRB	55

8.11.9.	RADAR TRANSPONDER	55

8.11.10.	TWO-WAY RADIOTELEPHONE APPARATUS	56

8.11.11.	SHIP SECURITY ALERT SYSTEM (SSAS)	56

8.11.12.	Long range Identification and Tracking (LRIT)	56

8.11.13.	BROADCAST & TELEVISION RECEIVING ANTENNA SYSTEM (AC 220 V)	56

8.11.14 vsat	56

8.11.14.	SUPPLEMENTARY OUTFITS	57

8.11.15.	SPARE PART	57

CHAPTER 8	ELECTRIC PART

  - 8.1 GENERAL

    - 8.1.1 GENERAL

The whole electrical installation shall comply with IEC and ISO standard, and to meet relative rules, regulations and convention which are listed in the “Part I General” of the Specifications.

Electric works on the vessel shall be carried out in accordance with these Specifications and Builder's practice.

Electric equipment and function shall be satisfied with specification and related recent rule.

Earth wire for equipment in/outside shall be prepared as per IEC standard.

More than terminal or connector with protective cover in electric panel shall be applied in order to prevent electrical shock of the operator.

Enough working space and door stopper shall be prepared when door of electric equipment open.

Pipe joints near side electric equipment shall be provided by sleeve or equivalent method for electric equipment protection. No piping or components containing liquids (including drains) shall be arranged inside rooms where switchboards, transformers, batteries, rectifiers, variable frequency controllers, etc. are installed. If piping passing through cannot be avoided, piping welding joints (sleeves) must be used, and shields must be installed above the electrical equipment / panels.

Final electric connection diagram with pocket shall be prepared in the panel door for easy maintenance.

If any items in the hull or machinery Specification necessitate the installation of electrical equipment not mentioned in this part, such equipment shall be provided by Builder.

Voltage and Frequency:

All electric equipment to function at variations of:

Frequency:      ±5% at the continuous load condition

±10% at the transient load condition

Voltage:        +6/-10% at the continuous load condition

±20% at the transient load condition

An ambient temperature of 45deg.C shall be assumed for electrical equipment in all spaces. Where ambient temperature is in excess of these values, the temperature specified by the Rule shall not be exceeded.

The meters to be of Class 1.5 and to have scale of the following range with red color mark at rated value.

Voltmeter:              0 to abt. 120% of rated voltage

Ammeter:               0 to abt. 130% of rated current

Wattmeter:              -15% (reverse power) to abt. 130% of rated wattage

Frequency meter:         55 - 65 Hz

The type and construction of electrical equipment incorporated or packed with the appliances, shall be in accordance with the manufacturers' standard unless specially described and may be changed by their Maker’s type or model changes.

All electrical machines and apparatus delivered to the shipyard and waiting installation are to be kept in a clean, dry and well ventilation store.

Electric equipment will be inspected and tested in accordance with the Rule, the Regulations and the Builder's usual practice.

Trial

The trials for all electrical equipment in manufacturer’s shop and or on board shall be carried out according to the requirements of Classification Society and the Builder’s standard and approved by Buyer.

“Trials program for electrical equipment” shall be provided for Owners review one month before sea trial.

After installations on board have been finished, all electrical equipment shall be tested to demonstrate their condition for satisfactory operation in accordance with the requirement of Rules and the detail design document of “Trials program for electrical equipment”.

    - 8.1.2 ARRANGEMENT OF EQUIPMENT

The arrangement of the electrical equipment throughout the Vessel shall be such as to provide ready and safe access to all parts for inspection, maintenance and repair as far as practicable.

All electrical equipment shall be so located that as far as practicable they are not exposed to risk of mechanical injury or damage from water, steam, oil or excessive heat and where inflammable gases cannot accumulate. Where necessarily exposed to such risks, the equipment shall be suitably constructed or enclosed.

All junction boxes shall be clearly marked with name by the drawings, necessary access hatches and space around repairing and etc. shall be considered.

Where electrical equipment is installed in the hazardous area such as battery store or paint store or cargo hold carried dangerous goods in, the equipment shall be of explosion proof type or intrinsically safe type.

    - 8.1.3 NAME PLATE

Flame retardant and non-corrosive material are to be used for nameplates, labels, etc. Nameplates on open decks shall be made of brass or stainless steel.

All nameplates are to be fixed with screws or rivets. The nameplates shall include at least item number and functional description

Type of plate shall be as follows:

| - | Rotating machine | : | Flame retardant and non-corrosive metal plate |
| --- | --- | --- | --- |
| - | Controller and panel | : | Plastic plate or Maker’ standard, except non-corrosive metal plate for those exposed to weather |
| - | Others | : | Maker’s standard |

The letter on the nameplate shall be in English. The fitting type of nameplate should be screw fitting or according to maker’s standard.

The caution plates shall be in English.

    - 8.1.4 PAINTING COLORS

Metal surfaces of the enclosure of electrical equipment shall be painted with the following colors or according to the Buyer’s specified.

| Generators, motors | Munsell code 7.5BG 7/2 |
| --- | --- |
| Transformers | Munsell code 7.5BG 7/2 |
| Switchboards, distribution panels and starter boards, engine control console etc. | Munsell code 7.5BG 7/2 |
| Bridge control console | Munsell code 7.5BG 7/2 |
| Interior communication, navigation and radio equipment | Manufacturer's standards |
| General alarm and other emergency operating equipment | Munsell code 5R4/13 (Red) |
| Others | Manufacturer's standard |

Interior metal surfaces of electrical equipment shall be painted with manufacturer's standard colors.

Resin made surface of electrical equipment shall be of manufacturer's standard colors.

Emergency lights to be provided with red mark.

    - 8.1.5 MARK AND ARRANGEMENT OF BUS BAR

Bus bars and connections of major power equipment shall be marked with tapes, paints, letters or equivalent and shall be arranged as follows:

(A)	A.C. system

| Source side | Load side | Color | Arrangement |  |  |
| --- | --- | --- | --- | --- | --- |
| Phase R | Phase U | Black | Left | Top | Front |
| Phase S | Phase V | Brown | to |  |  |
| Phase T | Phase W | Grey | Right | Bottom | Back |

(B)	DC. system

| Pole | Color | Arrangement |  |  |
| --- | --- | --- | --- | --- |
| Positive (+) | Brown | Left | Top | Front |
| to |  |  |  |  |
| Negative (-) | Blue | Right | Bottom | Back |

    - 8.1.6 EQUIPMENT AND MATERIAL

Electrical equipment and material shall be marine use one in general.

Control box with contact and emergency stop box (bunker station) on the open deck to be provided with space heater. Not including switches, reefer container sockets junction boxes, navigation and radio equipment.

Fuse, terminal and lamp shall be as mentioned below, except for those of special equipment such as navigation instrument, radio equipment and other electronic equipment shall be of Maker’s standard.

Fuse	In general, tubular or screw type fuse to be used except special type fuses for electronic equipment.

Terminal	Solderless type except for small electric terminal for lighting fixtures, switch and branch box etc.

Lamp

|  | Purpose |  |
| --- | --- | --- |
| For fluorescent lamp | General light | Two pin type |
| For incandescent lamp | General light, floodlight and signal light | . E-27 (200 W) |
|  | Pilot lamp of switchboard, starter, console, etc. | Manufacturer’s standard |
|  | Floodlight and searchlight | E-40 (>200 W) |
| For sodium and halogen lamp | Floodlight | E-40, Manufacturer’s standard |

Electrical equipment and wiring shall in general not to be installed in hazardous areas unless essential for operational purposes. The type of equipment and installation requirements shall comply with class rules.

Breaker

(A) Air Circuit Breaker (ACB)

Air circuit breakers may be used for protection of main generators, bow thruster. Air circuit breaker (ACB) to be of 3-pole hand and automatic operated, trip free, with breaker operation facilities in the main contact isolated position. Air circuit breakers (ACB) in Main and Emergency Switchboards shall be with draw-able type.

(B) Moulded Case Circuit Breaker (MCCB)

Moulded case circuit breaker will be used for protection of feeder circuits and may be fitted with magnetic instant trip, thermal trip, or shunt trip coil and/or under voltage trip coil depending on each application. Moulded Case Circuit Breakers (MCCB) is to be of plug-in type in Main- and Emergency Switchboards.

Miniature circuit breaker (MCB) will be used for protection of feeder circuits in lighting system and domestic equipment.

(C) Bus-tie switch (as per Maker)

Due to total electrical power of generators to be very large, bus bar and bus-tie switch of MSB must have enough capacity as per rules and Manufacturer’s standard.

  - 8.2 WIRING

    - 8.2.1 	WIRING SYSTEM

All cables shall be marked at both ends with the cable reference numbers as per drawings. Marks shall be flame retardant with impressed letters. The individual conductor ends in the cables shall be marked with closed ring marks. All internal wiring in switchboards, panels, consoles, etc. shall also be marked with closed rings at both ends with reference numbers as per drawings. (according to maker standard)

The distribution system shall consist of three-phase AC. three-wire insulated system, single phase AC. two-wire insulated system and DC two-wire insulated system.

The distribution system shall be as follows:

General power system:

1) AC 440 V system: 3 PH, 60 Hz, 3-wire insulated system

2) AC 220 V system: 3 PH, 60 Hz, 3-wire insulated system and single phase two-wire insulated system

3) DC 24 V system: 2-wire insulated system.

Galley and domestic equipment (large capacity AC440V):

AC 440 V/220 V, 3 PH/1PH, 60 Hz, 3-wire/2 wires

Lighting main circuit:

AC 220 V, 3 PH, 60 Hz, 3-wire

Branch circuits of lighting, small capacity domestic equipment:

AC 220 V, 1 PH, 60 Hz, 2-wire

Nautical instruments and radio equipment:

AC 220 V, 3 PH /1 PH, 3 wire/2 wire, 60 Hz, and DC 24 V 2-wire

Interior communication and alarm system and automation:

AC 220V, 1 PH, 60 Hz, 2-wire or DC 24 V, 2-wire

Bow thruster motor:

AC 440 V, 3 PH, 60 Hz

Reefer container:

AC 440 V, 3 PH, 60 Hz

The distribution system shall be insulated against hull throughout the vessel except for ground detecting circuits, secondary circuits of potential and current transformers will be earthed at switchboard or control panel.

The voltage for some portable equipment such as portable control box for turning gear motor and traveling crane and so on should be lower than AC 50 V using two (2) windings type transformer of which the secondary winding shall not be earthed.

The metal frame or body for portable equipment shall be earthed by the earth connection, if the voltage for portable equipment is AC 220 V.

    - 8.2.2 CABLE APPLICATION

All cables accepted by the Classification Society shall be provided.

In general, electric cable shall be of IEC marine cable (IEC 60092 series) ,and flame retardant type tested according to IEC 60332-1 and IEC 60332-3 Category, low smoke and halogen-free or equivalent type that has been approved by the Classification Society, except special cable which including: coaxial cable, compensate cable and LAN net cable etc. Cables in accommodation spaces to be of XLPE insulated, Metallic halide free sheathed (extruded or lapped).

Cables shall be of EP rubber or XLPE insulated, metal wire braided with polyolefin (PO) or polychloroprene rubber (PCP) covering.

Cables for lighting system in accommodation area shall be of EP rubber or XLPE insulated with polyolefin (PO) polychloroprene rubber (PCP)) covering.

For portable appliances, cables to be of flexible rubber insulated and polyvinyl chloride (PVC) or polychloroprene rubber (PCP) sheathed, and for table lamps to be of flexible vinyl cord.

The other insulating material cables may also be adopted, but permissive operating temperatures of cable conductor should be 90 oC.

Kind and size of cables supplied by manufacturers together with equipment to be accordance with manufacturer’s standard.

All cables throughout the vessel, except special cables such as coaxial cables, compensating cables, network cables, cables supplied together with equipment, etc. will be type approved by Classification Society.

    - 8.2.3 CABLE INSTALLATION

All cables shall be installed according to builder's practice and in compliance with the requirement of the Classification Society.

In general, cables shall run in groups and parallel and be fixed on steel hangers or trays.

Where the cables are exposed to mechanical damage, they shall be protected with steel pipe or steel cover.

Cables installed on mast or post on upper deck to be laid in galvanized steel pipe up to about 1m height from the deck.

The cables installed under the engine room lower floor shall be protected with flexible conduit or steel pipe or steel cover.

Cables passing through water-tight decks or bulkheads, Heat shrunk for all water-tight cable glands or other appropriate methods shall be employed.

Cables passing through beams, non-water-tight decks and bulkheads etc., suitable bushing or coaming with round edge shall be used for their protection.

The cables in living quarters shall be secured by nylon straps and plastic covered stainless steel straps, at wet space and exposed weather space, only plastic covered stainless steel straps to be used, unless the cables secured by conduit or steel pipe.

Penetration of fire protection bulkheads or decks of steel structures shall be protected by pipes, cable glands or coaming stuffed with fire retardant compound according to the fire protection class and number of cables.

In rooms and inner passages, where lining walls over steel walls and ceiling over overhead decks are provided, cables shall be concealed behind the lining walls as far as practicable.

Signal cables shall be always separated from power cables. Yard to describe same in shipyard standard for electrical practices; Sharp bend of cables not allowed.

The cable trays to be galvanized in general, except hanger to be the builder's practice,inside water ballast tanks, PVC pipe with stainless clamps can be adopted by builder’s practice.

Foundation for electrical equipment, small outfitting materials and fittings will be blasted and painted in general.

The support of cable trays & electric equipment brackets should be welded directly on deck plate, frames, girders and bulkhead.

The installation of cable trays and electric equipment brackets will be in accordance with Builder’s actual practice.

  - 8.3 PRIMARY SOURCE

    - 8.3.1 GENERAL

Three(3) sets of main generator AC 450 V, 3 PH, 60 Hz, each driven by diesel engine shall be installed in the engine room with the shaft parallel to ship's centerline.

One (1) set of emergency diesel generator AC 450 V, 3 PH, 60 Hz shall be installed in the emergency generator room. The emergency generator shall be used for supplying the loads such as radio equipment, interior communication equipment, nautical instruments, emergency lights, one set steering gear and emergency fire pump, one engine room fan, the emergency air compressor, etc. when the main power station fails.

    - 8.3.2 MAIN GENERATORS

| Type: | Drip-proof, self-ventilated type with air filter, and the air filter as per Maker’s standard |
| --- | --- |
| Output: | abt. 1,250 kW x 3 sets (DG) |
| Voltage: | AC 450 V |
| Phase: | 3 PH |
| Frequency: | 60 Hz |
| Power factor: | 0.8 (lagging) |
| Rating: | Continuous at full load |
| Revolution: | 900 r/min or as per Maker’s standard； |
| Insulation: | Class “F” |
| Exciting system: | Brushless, self-excitation |
| Protection: | With differential protection function |
| Enclosure: | IP 23 |

Note:

Protection grade IP 23 of main generators will be suitable in respect of water based fire- fighting system and will be certified by Classification Society.

Other particulars and construction of the main generators shall be in accordance with manufacturers' standard.

Each main generator shall have a space heater within the generator frame to prevent the condensation of moisture while the generator is stopped.

An automatic voltage regulator for each main generator shall be provided and located in MSB.

The main generator bearing type and lubrication function shall be manufacturers' standard. Temperature sensor shall be provided for bearing of the generator. Two temperature sensors shall be fitted in the stator winding of each phase of generators for alarm, one for working and the other for spare.

The main generators shall be capable of parallel operation. However, main generators neither to be run in parallel with emergency generator nor the shore source.

Each main generator shall be used as follows:

|  | Without reefer container | With reefer containers |
| --- | --- | --- |
| Normal sea going | DG x 1 | DG x 2 |
| Arrival or departure with bow thruster | DG x 2 | DG x 3 |
| Cargo handling | DG x 1 | DG x 3 |
| Rest in port | DG x 1 | DG x 2 |

The reefer container load for electric power balance may be considered as follows:

Total 220 sets of reefer containers socket(CEE type, 32A) on deck .

The reefer containers power demand for electric power balance shall be determined as follows:(4.5 kW/FEU x 220 RFEU = 990 kW.)

If bow thruster cannot be operated with ship's loads including reefer containers, some of reefer container loads to be off manually during bow thruster operation at port in/out condition.

If dangerous goods loaded on deck, the feeder breaker in main switch board must be locked and take off power source of reefer containers by locking device at ship office.

Proper procedure to be provided. System must then automatically block start of Bow Thruster till load is brought down to acceptable limits..

    - 8.3.3 EMERGENCY GENERATOR

| Type: | Drip-proof, self-ventilated type |
| --- | --- |
| Output: | Abt. 150 kW |
| Voltage: | AC 450 V |
| Phase: | 3 PH |
| Frequency: | 60 Hz |
| Power factor: | 0.8 (lagging) |
| Rating: | Continuous at full load |
| Revolution: | 1,800 r/min； |
| Insulation: | Class “F” |
| Exciting system: | Brushless, self-excitation |
| Protection class: | IP 23 |

Other construction and characteristics of the emergency generator shall be in accordance with the manufacturer's standard.

Two starting methods will be provided as per rule and manufacturer's standard such as to be used by one(1) set battery and hydraulic device or other equivalent differ starting method.

The emergency generator shall have a space heater within the generator frame to prevent condensation of moisture while the generator is stopped.

An automatic voltage regulator for emergency generator shall be provided.

This generator shall be started automatically in case of occurrence of the power failure of the main power supply and shall automatically supply electric power to the emergency loads.

After restoration of electric power from the main generator, the power supply system shall be automatically disconnected from the emergency generator and connect to the main generator. The emergency generator engine shall be automatically stopped with time delay.

Emergency generator can’t be run in parallel with main generators.

    - 8.3.4 MAIN SWITCHBOARD

In case that the contents in the following parts of this section differ from manufacturer's standard for specific equipment Makers’ standard shall be applied

in general.

IEC standards or wording “national standards” for the Main Switchboard.(8.3.4 Main Sitchboard) to be reference., such as rule: IEC61111

      - 8.3.4.1 General

The main switchboard to be consist of: (or according to manufacturer's standard)

Three(3)	-- Generator control panels

One (1)	-- Synchronizing & bus-tie panel

One (1)	-- Bow thruster feeder panel

Two (2)	-- AC 440 V feeder panels (or four panels as per manufacturer)

Two (2)	-- Group starter panels (or four panels as per manufacturer )

One (1)   -- AC 220 V feeder panel

One (1)   -- Empty cold-iron panel for AMP ready in future

Two (2)   --Reefer container panels

Each generator panel and bow thruster feeder panel to be protected by a three pole air circuit breaker (ACB).

The main switchboard, for receiving power from main generators and for the power distribution, shall be installed in engine control room with clear maintenance access to the front and rear of the panel as per rules, insulating mat shall be provided at front and rear of the main switchboard.

The main switchboard shall be multi cubicle, metal enclosed self-supporting, dead front type (IP20) with hinged front panels that can be opened without disturbing installed equipment and complete with necessary fittings and instruments.

Between cubicles shall be closed by flame retarding insulating material to prevent fire hazard passing to adjacent.

All bolts, screws etc., used in connection of current carrying conductors shall be adequately locked with lock nuts and spring washers or as manufacturer's standard.

The top and each side of the switchboard shall be protected with steel plate and suitable hand rail shall be provided at front and rear. The main switchboard shall have hinged front doors and removable rear side covers. Main cable shall entry through cable hole from under side.

The preferential tripping system shall be provided and so arranged that in the event of the main generators becoming overloaded the non-essential loads are disconnected automatically before the generators' circuit breaker is opened. The preferential tripping arrangement shall be in two stages sequential (the first stage to be general non-essential loads and the second reefer containers), as necessary.

Voltage circuits for control and instrumentation to be earth free and protected by double pole fuse.

Air circuit breakers (ACB) shall be of electrically operated trip free type and controlled by a control switch and additionally a manual operating handle shall be provided.

All AC440V out-going feeder circuits and shore source circuit to be protected by a three pole molded case circuit breaker (MCCB) with over-load trip and instantaneous trip on each pole, except for steering gear motor circuit which to be only equipped with an instantaneous trip. And all AC 220V out-going feeder circuits to be protected by a three/ two pole molded case circuit breaker (MCCB) or three/two pole miniature type circuit breaker (MCB) with over-load trip and instantaneous trip on each pole.

Each feeder circuit on the main switchboard shall be distinctly marked by label giving service name of each circuit and frame/set current for the circuit breaker.

The molded case circuit breaker (MCCB) for feeder circuits of motors as described in item 6-3.3 and 6-3.6 shall be provided with a shunt trip coil for emergency stop or preferential trip.

In case that the contents in the following each panel of this section differ from manufacturer's standard for specific equipment, manufacturer's standard shall be applied in general.

Certified insulation mat (IEC61111) to be provided in front and back of MSB and in front of ESB

The accuracy of meter to be within 1.5 (error: within 1.5% of full scale), except the meters for motor starter for which class 2.5 shall be applied.

Air from ventilation ducts shall not be discharged directly on the switchboard.

The bus bar of main switchboard shall be provided with bus-tie switch (manual control)

      - 8.3.4.2 Generator control panel

For proper operation and maintenance, the following instruments and apparatus shall be provided for each generator panel:

1 - Air circuit breaker (ACB)

1 - AC ammeter

1 - Change-over switch for AC ammeter (for each phase of generator)

1 - AC voltmeter

1 - Change-over switch for AC voltmeter (between each line of generator)

1 – Power factor meter

1 – Running hour meter

1 - Frequency meter

1 - Manual voltage regulator.

1 - ACB open/close control switch (or push button)

1 - Switch for space heater

1 - Generator running indicating lamp (white)

1 - ACB close indicating lamp (green)

1 - ACB open indicating lamp (red)

1 - Space heater indicating lamp (orange)

1 - Reverse power relay

1 - Running hour meter

1 – Diesel gen. remote control ready indicating lamp

1 –set of diesel gen. start/stop switch

1 – set of power management unit

1 – set of differential protection transformer (supplied by manufacturer)

1 – set standby indicator light in blue

1- Switch for External excitation and De Excitation

Etc.

And other necessary equipment;

The generators system must automatically block start of Bow Thruster till the load is brought down to acceptable limits.

      - 8.3.4.3 Synchronizing Panel

The following apparatus shall be provided:

1 -Dual voltmeter and frequency meter for main bus bar and each generator with generator selection switch

3 -kW meters

3-Governor control switches

1 -Synchroscope with a control switch

1 -Set of synchronizing lamp

1 -Set of stand by gen. select switch

1 –Set gen. Stand-by indicating lamp

1 –24 V DC source indicating lamp

1 –Emergency gen. running indicating lamp

1 –Emergency gen. in auto mode indicating lamp

1 –Alarm unit

1 set Red lamps for the following alarm:

- - MSB preferential & emergency stop circuit source fail

- Preferential trip

- Low/high voltage

- Low frequency

- Insulation low for AC 440V circuit

- Insulation low for AC 220V circuit in main switchboard

- 1- Lamp test switch/alarm reset button

And other necessary equipment;

      - 8.3.4.4 AC440 V Feeder Panel

The following device shall be provided for AC440 V feeder panel:

Necessary quantity - module case circuit breakers (MCCB) (with abt. 10% spare breakers for AC440V load only). All MCCB shall be of plug-in type.

*1 -Molded case type circuit breaker(MCCB) for shore supply

*1 -Shore supply indicating lamp (white lamp) and kWh meter

*1 -Insulation monitor with alarm lamp for checking AC440 V system

*1-Ammeter, it will be share with other equipment.

      - 8.3.4.5 AC 220 V Feeder Panel

The following device shall be provided for AC 220 V feeder panel:

Necessary quantity - Miniature circuit breakers (MCB, with abt. 10% spare breakers based on amount of circuit breakers on feed panel);

1 - Insulation monitor with alarm lamp for checking AC 220 V system

1 - AC ammeter

1 - Change-over switch for AC ammeter (for each phase of power transformer secondary circuit)

1 - AC voltmeter

1 - Change-over switch for AC voltmeter (between each phase of power transformer secondary circuit)

And other necessary equipment;

      - 8.3.4.6 Bow Thruster Feeder Panel

The bow thruster panel including:

1 –Air circuit breaker (ACB) (draw-out type)

1 –ACB open/close control switch (or push button)

1 –ACB close indicating lamp

1 –ACB open indicating lamp

1 –Ammeter

And other necessary equipment

      - 8.3.4.7 Group Start Panels

The starters for the essential pumps and ventilators in the engine room shall be assembled into group starter panels. Each unit (fixed type) to have circuit breaker, AC 440V/220V transformer, contactor, thermal relay, indicating pilot, push buttons, ammeter and running hour meter etc. which to be of fixed type.

Automation change-over function shall be provided for primary essential duplicate pumps.

The exact item and layout of starter unit shall be determined later during detail design.

Following items to be agreed for detail design:

The main switch is to be lockable by pad lock (to inhibit start).

When starter is not visible at the motor, a separate start/stop switch shall be installed near the motor.

Starters for redundant motors shall be installed in different GSP’s, supplied from the different sections of the main switchboard.

    - 8.3.5 EMERGENCY SWITCHBOARD

In case that the contents in the following each panel of this section differ from manufacturer's standard for specific equipment, Manufacturer's standard shall be applied in general.

Emergency switchboard shall be installed in emergency generator room.

In principle, the construction of the emergency switchboard shall be similar to the main switchboard, but service access at front.

The emergency generator shall be protected by a three-pole air circuit breaker (ACB), which to be of draw-out type.

Interlock circuit shall be provided between emergency generator ACB and the space heater circuit.

A socket of portable welding machine shall be supplied in the ECR, which shall be powered from the ESB.

The switchboard shall be consisted of the following:

      - 8.3.5.1 Generator Panel

1 -Air circuit breaker (ACB) (draw-out type)

1 -AC ammeter

1 -Change-over switch for AC ammeter (for each phase of generator)

1 -AC voltmeter

1 -Change-over switch for AC voltmeter (between each phase of generator)

1 -Frequency meter

1 -kW meter

1 -Bus tie for link with the main switchboard

1 –Running hour meter

1 -Switch for space heater

1 -Generator running indicating lamp (white)

1 -ACB close indicating lamp (green)

1 -ACB open indicating lamp (red)

1 -Space heater indicating lamp (orange)

1 -Manual voltage adjuster (if necessary)

1 -Red lamp for ACB abnormal trip

And other necessary equipment

      - 8.3.5.2 AC 440 V Feeder Panel /AC220 V Feeder Panel

Necessary quantity-Molded case circuit breaker (MCCB) or miniature circuit breaker (MCB) (with abt.10% spare breakers based on amount of circuit breakers on feed panel).

Each panel:

1 - Insulation monitor with alarm lamp (for AC 440V & 220V).

1 - AC ammeter

1 - Change-over switch for AC. ammeter (for each phase of emergency power transformer secondary circuit)

1 - AC voltmeter

1 - Change-over switch for AC voltmeter (between each phase of emergency power transformer secondary circuit)

1 - Mega-ohm meter with change-over switch

    - 8.3.6 SHORE CONNECTION BOX

One (1) set of shore connection box for receiving AC 440V volts, 400 amperes, 3 phase 60 Hz shore source shall be provided and wired to the main switchboard (MSB).

The box shall be of drip-proof type with a three (3) pole moulded case circuit breaker (MCCB), connecting terminals and phase sequence indicator; and:

1 - White lamp for shore source alive

1 - Green lamp for shore MCCB closed

1 - White lamp for shore MCCB open

1 - Voltmeter for shore power

1 - Ammeter for shore power

Interlocking shall be arranged to disable any parallel operation with the ship’s generators.

The shore connection box shall be of wall mounted type and shall be installed near emergency generator room or other suitable place.

On the bulkhead of laid room, a hole with water tight cover shall be provided for passing through the shore connection cables.

    - 8.3.7 AMP ready

MSB with additional empty panel(cold-ironing) , and cable tray shall be installed which shall route from the cold-ironing panel to the port side on upper deck of AMP ready position with water tight penetration; packaged container solution with all other AMP device will be separated and supplied by the Buyer (refer to the installation of 40ft container which will include relevant transformer and AMP connection box inside). Appropriate space for future AMP device ready shall be considered on board.

MV transformer and cable reel shall not be installed on board. The Buyer will rent the container, which shall be contained with MV transformer(1.6MVA)  and reel.

The cable for shore connection shall not be supplied by the Builder

    - 8.3.8 TEST PANEL

One (1) set of wall mounted type electrical test panel shall be installed in the electrical workshop. The panel shall be provided with following devices: (or according to manufacturer's standard) switches, pilot lamps, voltmeter and ammeter.

| Each one (1) | Set of test terminals of AC 440V. 3 PH and AC 220V. 3 PH both up to 20A, AC110V and AC 24V 5A |
| --- | --- |
| One (1) | Set of test terminals of DC 24V up to 5A |
| One (1) | Fuse checker |
| Each one (1) | Lamp holder for various fluorescent lights of 40W, 20W, 8W |
| Each one (1) | Lamp holder for various incandescent lamps of E40, E27, E14, B15, B22 |
| Each one (1) | Receptacle of non-water-tight and water-tight for AC 220V, 10A |

  - 8.4     SECONDARY SOURCE

    - 8.4.1 TRANSFORMERS

- For General Service

Two (2) sets of 3 phase main transformer (i.e. d-d or y-y) shall be installed in the engine room. One (1) transformer shall be capable of supplying to all AC220V service so that only one (1) transformer is for working and the other is for spare. Working transformer manually changing-over shall be provided only.

Principal particulars shall be as follows:

Type:	Drip-proof, dry type

Output: 	Abt.99 kVA (as per final E-balance)

Number: 	2 (one as stand-by)

Voltage: 	AC 450/230V

Frequency:	60 Hz

Insulation:	Class F

Rating: 	Continuous

Cooling: 	Natural-cooling

- For Emergency Service

Two (2) sets of 3 phase transformers (i.e. d-y11) shall be installed in the emergency generator room for emergency service. One (1) transformer shall be capable of supplying all emergency service so that only one (1) transformer is for working and the other is for spare. Working transformer manually changing-over shall be provided.

Principal particulars shall be as follows:

Type: 	Drip-proof, dry type

Output: 	Abt. 50 kVA (as per final E-balance)

Number: 	2 (one as stand-by)

Voltage: 	AC 450/230 V

Frequency:	60 Hz

Insulation: 	Class F

Rating: 	Continuous

Cooling: 	Natural-cooling

- For Galley Service

One (1) set of 3 phase transformers (i.e. y-d11) shall be installed in galley room or near place for galley service. The transformer shall be double wiring and capable of supplying all AC440V & AC220Vservice.

Principal particulars shall be as follows:

Type: 	Drip-proof, dry type

Output: 	Abt. 99 kVA (450V 65kVA / 230V 30kVA) (as per final E-balance)

Number: 	1

Voltage: 	AC 450/450V/ 230 V

Frequency:	60 Hz

Insulation: 	Class F

Rating: 	Continuous

Cooling: 	Natural-cooling

    - 8.4.2 STORAGE BATTERIES AND BATTERY CHARGING BOARD

      - 8.4.2.1 For General Use and for Engine Room Area

The maintenance free lead-acid type batteries to be installed as follows:

|  | No. of set | Capacity (Ah) | Voltage (V) | Rating | Location |
| --- | --- | --- | --- | --- | --- |
| For general service | 1 | 200 | 24 | 10 hours | Engine room |
|  | 1 | 200 | 24 | 10 hours | Battery room |
| For automation system | Manufacturer's standard |  |  |  | Engine room |
| For radio equipment | Manufacturer's standard |  |  |  | Radio space |
| For emergency generator starting | Manufacturer’s standard |  |  |  | Emergency generator room |

Each one (1) set of charging and discharging board for general service batteries and engine room battery may be of floating charging type.

All necessary charging apparatus i.e. source switch, transformer, voltage adjuster, silicon rectifier, voltmeter, ammeter, indicating lamps, miniature circuit breaker (MCB), fuses, etc. to be provided.

24V DC distribution board (with feeder breakers, insulation monitor, ammeter, voltmeter and other necessary fittings;

A safety factor of 1.3 is to be used for the calculation of the capacity of all batteries to compensate for capacity loss caused by ageing.

1. All battery chargers are to be capable of supplying the total load of all consumers and simultaneously charge totally empty batteries.

2. Chargers are to be able to operate with batteries disconnected. A low voltage alarm for the battery and battery charger output is to be provided in AMS.

      - 8.4.2.2 Other

Battery for emergency generator engine starting and for radio emergency source shall be provided according to the Rules and Manufacturer's standard.

Emergency generator engine starting battery shall be charged by a charger supplied by emergency’s generator maker.

Radio battery shall be charged by a charger incorporated in radio console.

The power socket, cables with plugs etc. for life boat and rescue boat battery charging shall be equipped (if necessary) and the chargers shall be fixed on boat.

  - 8.5 DISTRIBUTION SYSTEM

    - 8.5.1 ELECTRIC DISTRIBUTION

In general, separate main feeders shall be fed from the main switchboard to essential pump motors, bow thruster, each reefer power distribution panel, each power distribution panel, and one (1) of steering gear motor and other large capacity services.

Motors of which starters are assembled into the group starter panels, however, shall be connected to the AC 440 V feeder bus bar of the main switchboard.

Non-essential services and other small motors shall be connected to each distribution board or local group starter board fed from the main switchboard.

Separate feeders shall also be fed from the emergency switchboard to one (1) of the steering gear motor, one (1) set of the E/R exhaust fan and emergency fire pump motor, emergency air compressor motor, generator engine pre-lub. oil pump etc.

The general lighting system and AC 220 V general consumers shall be fed from AC 220 V feeder panel of the main switchboard.

Emergency lighting system and AC 220 V system of radio equipment and nautical instruments shall be fed from AC 220 V feeder panel of the emergency switchboard.

DC 24V system of alarm, DC 24V emergency lighting, some interior communication equipment and nautical instruments shall be fed from the battery charging & discharging board.

    - 8.5.2 SECTION AND DISTRIBUTION BOARDS

All section boards and distribution boards shall be enclosed with steel sheet case.

Certified rubber insulation mats (IEC61111) shall be provided around reefer container distribution panels.

Distribution boards and section boards fitted in the living quarters and other similar places shall be of non-water-proof type, and those fitted in the engine room, steering gear room, stores and other similar places shall be of drip-proof type.

Reefer container distribution panels shall be fitted in under deck passageway of cargo hold region and shall be non-water-proof type.

In general, distribution boards and section boards shall be equipped with two or three pole miniature circuit breaker (MCB).

An electric circuit drawing shall be provided in a vinyl envelope in each distribution boards or boxes.

The circuit label shall be fitted on the inside of each board door.

    - 8.5.3 CONNECTION BOXES AND BRANCH BOXES

All boxes shall be of drip-proof or water-proof construction depending on their locations. The water-proof type connection box exposed to weather deck to be brass-made or stainless steel.

    - 8.5.4 RECEPTACLE FOR REEFER CONTAINER

Total 220 reefer container sockets (CEE type, 32A) will be provided on open deck.

Suitable number multi-gong type receptacle box shall be installed on operation platform.. Each gang should include circuit breaker, and power receptacle etc. these receptacle box shall be connected to some reefer distribution panels, each reefer distribution panel shall be fed from the main switchboard.

One (1) set of reefer container monitoring system, which to be of power cable data transmission (PCT) type with operating and display (24” TFT) unit shall be installed in ship office.

The communication between the PCT system and loading computer shall be established, therefore, necessary hardware and software shall be provided.

Where the water-proof socket outlets on upper deck are exposed to rain, sea water splashes, etc. the socket outlet to be protected with sheet steel casing. But Reefer container sockets below hatch coaming no need protection.

Each individual reefer socket shall be capable of being switched on/off.

  - 8.6 ELECTRIC MOTOR AND CONTROL GEAR

    - 8.6.1 GENERAL

The specification described in this section shall be applied to the power motors and their control equipment except for following electrical equipment which are constructed according the maker's standards:

(1)	 Galley and laundry equipment

(2)	Governor motors

(3)	Instrumentation and automation equipment

(4)	Motors and their control equipment supplied as a complete set with auxiliaries machinery such as deck machinery, aux. blower, turning gear, small and household appliances etc.

    - 8.6.2 MOTORS

      - 8.6.2.1 General

All motors, unless otherwise specified, shall be marine type squirrel cage induction motors.

Motors shall be designed for AC 440 V, 3 phase, 60 Hz, except for that the motors of 0.5 kW rating and less may be of AC 220 V, 3 phase or single phase type according to manufacturer's standard.

Enclosure of motors shall be generally provided as follows:

Water-proof type (IP 56) for motors exposed to weather.

Totally enclosed type (IP 44) for steering gear, emergency fire pump and motors in engine room, provision refrigerating room, purifier area, galley and similar spaces.

Drip-proof type (IP 23) used for motors in accommodation and other dry spaces.

The enclosed type (IP23) will be for bow thruster motor.

Space heater shall be provided for all motors on open deck and motors of steering gear, emergency fire pump, M/E turning gear, bow thruster and motor rated power over 10 kW in engine room.

      - 8.6.2.2 Insulation and terminal box

In general, class “B” or “F” insulation shall be adopted, except special service motors for which insulation are in accordance with the manufacturer's standard.

In general, all motors shall be provided with waterproof terminal box of cast iron or steel plate construction with gland and removable cover. All terminals shall be marked.

    - 8.6.3 STARTERS

      - 8.6.3.1 General

All motors with reversible feature (eg. ER Vent fans) shall have timer interlock to prevent immediate reversal.

In general, starters shall be of magnetic control type with thermal protect device except for that small motors (0.5 kW and less) may be manually operated by line switch or miniature circuit breaker (MCB).

In general, motors shall be arranged to start direct on-line (DOL), but for 100 kW and above large capacity motors, star – delta starting, auto-transformer starting reduced voltage starting means shall be adopted.

In general, voltage for control circuits shall be isolated from main power source via transformer.

The wiring diagram of starter shall be fitted on the back of each starter door.

In general, starters located out of sight the motors shall have local interlock type with running indication lamp start/stop push button box near respective motors, except for the E/R fans, accommodation fans and cargo hold fans.

Starters for motors of steering gear and those for essential services will be provided with under voltage release protection (UVR) will permit these motors being restarted automatically on restoration of main power, and starters for other motors to be provided with under voltage protection (UVP) feature.

All motors with reversible feature (eg. ER Vent fans) shall have timer interlock to prevent immediate reversal.

(A)	Group Starter Board:

In general, starters shall be assembled in group starter boards as far as practicable.

The group starter board shall be in combination with starters according to their location and/or purpose.

These group starter boards shall be of bulkhead type or floor mounting type and shall be installed near the respective motors as far as practicable.

Each starter unit to be fixed type and built into one enclosed metal box of drip-proof construction with hinged door. Starters for redundant motors shall be installed in different GSP’s, supplied from the different sections of the main switchboard.

(B)	Individual Starter Cabinets:

Starters which are not contained in the group starter board shall be mounted in the drip-proof metal cabinet.

The cabinet shall be arranged for bulkhead mounting type near the respective motor.

      - 8.6.3.2 Ammeters and Running Hour Counters

Ammeters will be equipped on the starters for important motors in MSB and motor of 15 kW and above.

The running hour counters will be equipped on the starters for important motors in MSB and motor of 7.5 kW and above.

The ammeter on the starter shall be of overload type, the rated value shall be marked in red color.

      - 8.6.3.3 Emergency Stop Device

- For fire

All ventilating fans and oil pumps shall be capable of being stopped in case fire.

Emergency stop push button shall be provided for each group and their locations shall be as follows:

(a)  Fuel oil pumps, lub. oil pumps, oil purifiers and ventilation fans in the engine room.

Emergency stop push button box shall be provided in the entrance of engine room, fire control station

(b)  Ventilating fans in accommodation and cargo hold:

Each one (1) emergency stop push button box shall be provided in the wheelhouse and fire control station for stopping accommodation fans and cargo hold ventilating fans.

- Other

Emergency stop button for engine room bilge pump and sludge pump include other system which may connect to shore cause potential pollution shall be provided on upper deck (port and starboard) according to the requirement of USCG Regulation.

Emergency stop device shall have self-monitoring features like power failure and cable break. Emergency stop push button to be fitted with plastic covers. Box to be red painted

Sea water discharge pumps having a water discharge outlet beneath the life boat stowing place above light ballast waterline, if any, shall be stopped in case of boat launching at the location of boat launch.

      - 8.6.3.4 Steering Gear Motors

The steering gear motors shall be IP44 squirrel cage type and which power fed by separate feeder circuits, one (1) from main switchboard and another one (1) from emergency switchboard, according to the requirements of the Rules.

The feeder cables shall be run so as to minimize the probability of simultaneous failure of both feeders.

Both independent motor starters shall be located in the steering gear room. The motors can be controlled locally or from the bridge.

The steering gear alarm shall be installed on wheelhouse and in the ECR in accordance with the requirements of the rules.

      - 8.6.3.5 Bow Thruster Motor and Control

Bow thruster motor shall be interlocked with bow thruster room fan and the pitch of blades is in zero position.

The motor shall be of single speed and non-reversible type. The motor rating is S2-30 min as per manufacturer’s manual.

The motor can be started/stopped from local starter and wheelhouse and can be emergency stopped from both bridge wings. The running indication shall be provided in wheelhouse.

PT100 temperature sensors will be supplied in main motor winding.

The starter for bow thruster is located in bow thruster room or suitable place, BT starter to be in upper deck level of BT room and not in the same level as BT motor..

Bow thruster pitch order & response signal will be supplied to VDR system from bow thruster remote control system.

Adjustable timer shall be provided in the starter to prevent immediate re-starting of bow thruster.

Load indicator to be provided in wheel house panel. MSB Bow Thruster Feeder Panel to include Running indication and Abnormal Condition Lights.

Following shall be provided for the control of bow thruster.

| Location | Start/stop | Pitch control & indicator | Em'cy stop | Ammeter |
| --- | --- | --- | --- | --- |
| Starter | O | - | - | O |
| Wheelhouse control panel | O | O | O | - |
| Wing control panels (fixed type) | - | O | O | - |
| Local (motor side) | O | - | - | - |

      - 8.6.3.6 Cargo Hold Fans Control

In general, each one starter shall be provided for each cargo hold fan, which can be combined in one group starter board together at suitable place; the power shall be feed from the MSB.

      - 8.6.3.7 Preferential Tripping Arrangement

Preferential tripping arrangement shall be as follows:

First stage unload:

- Galley and laundry powers;

- Air conditioner compressor room warmers etc.

- Ventilation fan for accommodation

- Lathe, drilling machine, tool grinder

- Air conditioner for engine control room;

Second stage unload:

- Reefer containers

    - 8.6.4 ELECTRIC WARMER FOR ROOM (AC 220 V)

Electric warmers for room shall be installed as follows:

2 x 2 kW – Steering gear room

2 x 2 kW - Emergency generator RM

2 x 2 kW – Bow thruster room

1 x 2 kW - Bosun store

2 x 2 kW- Wheelhouse

1 x 0.5 kW - Each public toilet

1 x 2 kW – Hospital

  - 8.7 LIGHTING SYSTEM

    - 8.7.1 GENERAL

The vessel shall be adequately illuminated throughout with fluorescent lights or energy saving light and supplemented by incandescent lights. Lighting fixtures for flood lighting shall be of halogen type lamps.

In general, incandescent lamps must be used as little as possible unless otherwise specified.

The lighting system shall be divided into normal lighting, emergency lighting.

Machinery space light should be arranged/supplied in such a manner that the failure of anyone branch circuit not to cause any space to be entirely dark.

In general, emergency lights shall be arranged without local switches, except for wheelhouse and crew cabins.

All emergency lights must be marked with a red mark.

Anti-vibration mountings to be used in areas subject to local vibration (if necessary), except hanging type lighting fixtures.

The general features and dimensioning of lighting fixtures, switches and receptacles to be submitted for Owner's approval. Quantity and size of lighting fixtures, switches and receptacles may be changed in accordance with Owner's approved General Arr't of Electrical Equipment during plan approval procedure.

a) The lighting fixtures fitted in the weather part to be water-proof and to have the body of cast brass or brass plate or stainless steel or synthetic resin.

b) Lighting fixtures fitted under the risk of mechanical damages to be protected with guards for (energy saving tube) lights and polycarbonate globes for fluorescent lights.

c) For protection guards, steel guards to be used for the lamps fitted indoor space and brass guards for the lamps exposed to weather

Incandescent Lamps to be replaced by energy saving tube wherever is feasible.

Emergency light may be normally used as part of the general lighting system and 1/3 quantity at least in the following rooms & spaces :.

| Wheelhouse | Inner and outer passages |
| --- | --- |
| Electric equipment room | Stairs in accommodation |
| Emergency generator room | Workshop |
| Hospital | Engine control room |
| Offices | Engine room |
| Public rooms | Steering gear room |
| Public toilet and deck toilet | Emergency fire pump room |
| Navigating light and signal lights | Chart light |
| Fire control station | Navigation Equipment |

In general the enclosure of the lighting fixtures and fittings shall be of the following type dependent on their location:

| Waterproof type: (IP 55) | Spaces exposed to the weather, pipe funnel, refrigerated chamber, E/R below floor, oil purifier area, machinery spaces, store room, laundry, bow thruster room etc. wet spaces. |
| --- | --- |
| Enclosed type (IP 34 or 44) | Galley and pantry |
| Non-waterproof type: (IP 22) | All cabin, mess room, engine control room, wheelhouse, control room and office etc. dry spaces. |
| Explosion proof type: | Battery room, paint store, acetylene bottle store and other dangerous spaces. |

    - 8.7.2 LIGHTING – ACCOMMODATION AREA AND PASSAGE

| Lighting fixture | Space/location |
| --- | --- |
| Fluorescent ceiling light (20 W x 2, flush type) | Crew’s cabin, mess room, pantry, hospital, conference room, office, gymnasium etc. |
| Fluorescent ceiling light (20 W x 2, surface type) | Galley, laundry, public toilet, deck toilet |
| Fluorescent corner light (20 W x1) | Inner passage, staircase |
| Fluorescent desk light(15W) | On the writing table in each cabin and office |
| Fluorescent berth light(8W) | Over the head of each berth (fed by emergency power supply) |
| Fluorescent mirror light (with shaver socket) | On each private lavatory room |
| Decorative type wall light | Mess room, gymnasium, recreation room |
| Fluorescent light (20 W x 2, waterproof type) | Outer passageway, every 3~5 meters or a proper distance around superstructure and along on deck passage ways below/at hatch coaming |

    - 8.7.3 LIGHTING – MACHINERY SPACE

| Lighting fixture | Space/location |
| --- | --- |
| Fluorescent pendant light (40 W x 2, 20 W x 2) | Engine room over floor, steering gear room, emergency generator room, bow thruster room and other machinery space |
| Fluorescent ceiling light (20W x 2, flush type) | Engine control room |
| Incandescent pendant light (40W~60W) , energy saving light (11W) | Pipe duct, escape trunk, under the E/R lower floor |
| Incandescent spot light (100W, with adjustable arm) | On work face of machine tool and worktable in workshop |
| Halogen type floodlight (500W, 8 sets) | E/R upper flat top of M/E & A/E |

    - 8.7.4 LIGHTING – WHEELHOUSE

| Lighting fixture | Location |
| --- | --- |
| Fluorescent ceiling light (20W x 2, flush type) | Top of wheelhouse |
| Incandescent chart lamp (60 W, with dimmer switch) | On the chart desk |
| Incandescent light (down light, 40 W, red) | Above the wheelhouse control console for night navigation |

    - 8.7.5 LIGHTING – STORE

| Lighting fixture | Space |
| --- | --- |
| Fluorescent pendant light (20 W x 2) Or incandescent bulb pendant light (60 W) | General store, CO2 room |
| Incandescent bulb pendant light (60 W) | Refrigerated chambers |
| Incandescent bulb pendant light (60 W, explosion-proof type) | Battery store, paint store, acetylene store etc. dangerous spaces |

    - 8.7.6 LIGHTING – CARGO HOLD

| Lighting fixture | Location |
| --- | --- |
| Fluorescent pendant light (20 Wx2),explosion-proof type | Entrance, companionways and stairway, walkways |
| Incandescent light (60W) | Each hatch coming (3 sets/each side) |

    - 8.7.7 OUTDOOR FLOOD LIGHTING

| Lighting fixture | Qty. | Location | Illuminated area |
| --- | --- | --- | --- |
| 1000W halogen floodlight | 3 2 | Compass deck front Foremast-behind | Main deck cargo space |
| 500W halogen floodlight | 2 | Foremast forward | Fore mooring space |
| 500W halogen lamp | 2 | Navigation deck after | Funnel mark |
| 500W halogen lamp | 2 | Navigation deck both side | Ship’ name |
| 500W halogen lamp | 2 | Stern | Stern mooring space |
| 500W halogen lamp | 2 | Navigation deck | Main deck cargo space |
| 500W halogenlamp | 6 | Fore, midship and stern both sides | Ship surrounding area for ISPS requirement |
| 500W halogen lamp | 2 | Main deck side | Pilot ladder |
| 500W halogen lamp (manual swivel type) | 2 | Boat deck side | Lifeboat embarkation and outboard (commonly used for accommodation ladder) |
| 200W halogen lamp (bay lamp) | 2 sets /passage | Main deck side, between container line | Operation platform walkway |

    - 8.7.8 PORTABLE LAMPS

Total ten (10) sets of 24 V, 60 W portable working lamps (no-explosion proof type) with protection cover shall be provided. Each portable lamp shall have flexible cord of fifteen (15) meters with plug.

Five (5) sets of explosion proof type, chargeable, portable battery lamp shall be provided.

    - 8.7.9 CONTROL OF LIGHTING

The ceiling lights in the engine room shall be generally controlled by miniature circuit breakers (MCB) of the distribution boards in the engine room.

In mess rooms, recreation room, wheelhouse and engine control room etc. spaces, in case multiple access are arranged, the two-pole two-way switches shall be installed in each entrance for control the ceiling lights in common.

The lights in workshops and engine & electrical stores shall be controlled by a switch located near the respective entrances.

The lights in the machinery compartments such as steering gear room, emergency generator room, etc. shall be controlled by a switch located near respective entrances.

Cargo hold lights shall be controlled by switches on main deck suitable location.

Cargo hold light (except inside C/H) and outside lights shall be controlled by switches in the wheelhouse.

The lights in the living quarters, public spaces, stores, etc. shall be controlled by respective switches located near the entrances in general.

The lights in the inner passages shall be controlled by the circuit breakers of the distribution boards.

The lights for the outside passages shall be controlled from the wheelhouse.

The outside floodlights shall be controlled by respective switches on the group switch board in the wheelhouse.

The lights in the stair case of the wheelhouse entrances shall be controlled by a door limit switch (if necessary) also.

The lights of the explosion-proof type shall be controlled by respective switches located in the safety place.

The lighting fixture in pipe duct shall be controlled (on/off) from the pipe duct entrance in engine room.

Emergency lights shall be arranged without local switches, except for wheelhouse and crew cabins.

    - 8.7.10 SWITCHES

Switches in living quarters shall be of non-water-proof type, and switches in the machinery spaces, galley, laundry, toilets etc. shall be water-proof type in general.

Switches shall be constructed for wall mounting in general, and flush type switches shall be provided for the spaces where woody linings are fitted.

The switch for cold provision stores lighting shall be with a red indicating lamp to indicate when the inside lights are on.

Switches for berth lights and desk lights shall be fitted on their own fixtures.

In general, all tumbler switches and rotary switches for lighting branch circuits shall be of two-pole type.

    - 8.7.11 RECEPTACLES

      - 8.7.11.1 Non-water-proof Type Receptacles

Non-waterproof receptacles for small electric appliances shall be of, two (2) poles with earth contact type AC 220 V, 10 ampere capacity and shall be fitted as follows:

At least two (2) free receptacles shall be provided respectively in wheelhouse, chart space, radio space, mess room, hospital, engine control room, all crew's cabins, offices, inner passageway and other public room, at least 4 for after wall of wheelhouse.

In addition, each one (1) receptacle of two (2) poles, DC 24 V shall be provided at the rear of the main switchboard and the emergency switchboard.

      - 8.7.11.2 Water-proof Type Receptacles

AC 220 V/24 V water-proof, two (2) poles with earthling contact, with switch receptacles boxes shall be fitted in steering gear room (2 sets), engine room (10 sets), bosun store (1 set), outside of ref. provision chamber (1 set), and emergency generator room (1 set), etc. used for portable lamps or tools.

Receptacles fitted in weather space shall be of brass-made

Two (2) AC 440 V receptacles for electric welding will be provided. One in E/R workshop, one in the bosun store forward.

Two (2) AC220V/10A receptacles protected by water-tight boxes will be provided for reefer container maintenance in each side of the reefer container cargo bay.

  - 8.8 NAVIGATION LIGHTS AND SIGNAL LIGHTS

    - 8.8.1 NAVIGATION LIGHTS

The following navigation lights shall be of double lamp type and connected to a panel with control switches and indicating lamps graphically arranged, audible and visible signal alarm, and this panel shall be incorporated into the wheelhouse control console.

Navigation lamps should be installed according the requirement of Rules. Navigation lights and its controller should meet the performance standard requirement MSC.253 (83) and as per rules.

Two separate feeder circuits shall be provided for the panel, one (1) from the AC 220V feeder panel of main switchboard directly and one (1) from the AC 220 V feeder panel of emergency switchboard directly.

Two (2) - Mast head lights	AC. 220 V, 60-65 W

Two (2) - Side lights	AC. 220 V, 60-65 W

One (1) - Stern light	AC. 220 V, 60-65 W

      - 8.8.1.1 Anchor Lights: (AC 220 V)

Each one (1) 60 W anchor light of lamp fixed type shall be fitted at the forward and after end of the vessel.

The source of anchor lights will be from navigation light control panel.

      - 8.8.1.2 "Not-Under-Command" Lights: (AC 220 V)

Two (2) 60 W red all round lamps shall be fitted on the radar mast.

The source of NUC lights will from navigation light control panel.

      - 8.8.1.3 Manoeuvring/Morse Signal Lamp: (AC 220 V)

A Manoeuvring/Morse signal lamp shall be mounted on the top of radar mast and four (4) keys shall be located at both bridge wings and on front wall of wheelhouse.

The Manoeuvring/Morse signal lamp shall also be controlled by the ship's horn system.

The control method of the Morse & manoeuvreing signal lamp should also meet the performance standard requirement MSC.253 (83) and as per Rules.

The source of maneuvering/morse signal lights shall from navigation light control panel.

    - 8.8.2 Signal lights

      - 8.8.2.1 Daylight Signal Lamp (DC 24 V)

One (1) set of portable type daylight signal lamp (60 W) shall be provided with battery and charger.

One (1) source receptacles shall be installed on each bridge wing.

      - 8.8.2.2 SUEZ CANAL SIGNAL LIGHTS & SEARCHLIGHTS (AC 220 V)

One (1) set of 60 W fixed type Suez Canal signal light shall be provided, that is six (6) red, five (5) white and two (2) green. These signal lights shall be controlled by respective switches graphically arranged on the signal light control panel in the wheelhouse control console.

One (1) set of receptacle with switch used for Suez Canal searchlight (searchlight 3kW, Owner supply) shall be installed on the fore part.

Each one AC 220V 1000 W searchlights (with manual operation rotatable bracket) shall be installed on both wings of bridge deck.

      - 8.8.2.3 SUEZ CANAL STERN LIGHT (AC 220 V)

One (1) set of 40 W fixed type red stern light shall be provided as the Suez Canal stern signal light.

      - 8.8.2.4 DANGEROUS CARGO LOADING LIGHT (AC 220 V)

One (1) flash type red dangerous cargo loading light (40 W) shall be fitted on radar mast.

      - 8.8.2.5 Panama CANAL Stering LIGHT (AC 220 V)

25W blue steering lights shall be fitted on the foremast behind and controlled from wheelhouse through dimmer switch.

      - 8.8.2.6 Singapore strait lighT (AC 220 V)

One (1) set of Singapore strait light for vessel passing traffic separation scheme TSS (three all-round green lights 60W).

  - 8.9 INTERIOR COMMUNICATION, ALARM AND MEASURING SYSTEM

    - 8.9.1 SOUND POWER TELEPHONE

One (1) set of selective calling type sound power telephone system shall be provided as follows:

1-	Wheelhouse

1-	Emergency generator room

1-	Fire control station

1-	Steering gear room

1-	Bow thruster room

1-	Engine control room/main engine side.

1-   Fuel oil filling station (one portable telephone and two sockets).

The sound power telephone in engine room, steering gear room, bow thruster room and emergency generator room shall be of anti-noise type (head set type).

    - 8.9.2 AUTOMATIC TELEPHONE (AC 220 V & DC 24 V)

One (1) set of forty-eight (48) line automatic telephone system shall be provided. The number and location of telephone to be properly corrected according to the final general arrangement without capacity upgraded; Parallel connection shall be adopted for the engine room, workshop, and ECR.

Automatic telephone to be interfaced with Inmarsat Fleet broadband system for incoming and outgoing calls as per Maker’s standard.

The telephone sets shall be arranged in crew’s room, offices, public spaces, all control station, hospital, galley, steering gear room and other spaces where necessary for operation..

The telephone sets shall be adopted desk type, wall type or flush type according to their installed location.

The general telephone number list shall be supplied by builder and located beside each phone (paper with plastic coated).

The automatic telephone system shall be connected to the public addresser system to enable paging announcement spoken from wheelhouse, captain’s cabin, chief engineer’s cabin;

One (1) telephone socket shall be provided at each side on upper deck to connect with shore telephone line. The automatic telephone system shall be capable of receiving call from shore by this socket.

The priority function for the following sets shall be provided:

- Wheelhouse

- Captain cabin

- Engine control room

- Fire control station

Automatic telephone to be interfaced with satellite VSAT for incoming and outgoing calls as per Maker standard. Extensions that can make and receive VSAT calls shall be as per instructions during plan approval.

    - 8.9.3 HOSPITAL CALL SYSTEM (DC 24 V)

One (1) set of calling bell system shall be provided as follows: Call push button	Hospital (on berth side)

Buzzer with lamp	Wheelhouse & 2nd Officer cabin

    - 8.9.4 ENGINEER CALL SYSTEM (AC 220 V)

One (1) engineer call function shall be done in engine control console, each one (1) set of watch cabin panel with acknowledge push button shall be provided in chief engineer’s, 2nd engineer’s, 3rd engineer’s, 4th engineer’s, electrician’s cabin and recreation, Officer mess rooms.

Patrol man in engine room call alarm to be provided.

The system may be combined with both dead man alarm system and E/R monitoring & alarm system, i.e. this call system can be combined into extension alarm system.

    - 8.9.5 Bridge navigation WATCH alarm SYSTEM（BNWAS）

One (1) set of bridge navigation watch alarm system (BNWAS) will be supplied according to requirements of the IMO Res.MSC.128 (75).

One (1) main unit shall be installed in wheelhouse control console and each one (1) set of signal box with acknowledge push button shall be provided in wheel house and both wing of the bridge, alarm signal of notification shall be provided in Captain, Chief off., 2nd off., 3rd off. cabin, Officer mess room and ship's office, recreation room and Gymnasium..

    - 8.9.6 Refrigerated Chamber ALARM

One (1) set of refrigerated provision chamber alarm device shall be provided. Each one (1) switch with indicator shall be fitted in the refrigerated chamber and a bell with red lamp shall be fitted in the galley & a buzzer with lamp shall be fitted in wheelhouse so as to signal when a person is locked in.

    - 8.9.7 ENGINE ROOM CO2 ALARM

One (1) set of engine room CO2 alarm system shall be provided for warning before CO2 extinguishing gas flooding into the engine room, and shall consist of an activated switch in the CO2 room, some horns with rotating lamp in engine room (signal light columns) and bells with lamp in engine control room.

Engine room CO2 alarm sound shall be distinguished separately. The CO2 alarm is to be sent to AMS and VDR (direct to VDR).

The fire resistant type cable should be used in this system.

    - 8.9.8 EMERGENCY GENERAL ALARM SYSTEM

An electrically operated sounding alarm system consisting of bells and sirens, as appropriate, shall be provided with general alarm signals which shall be clearly audible in all conditions throughout the entire accommodation, navigating bridge, machinery space, and normal crew working spaces of the vessel.

The system with signal unit shall be used for sounding fire and general emergency alarm signal.

The system may be manually operated call points installed at the following locations:

- W/H control console

- Fire control station

- Embarkation

The general emergency alarm may be interface with public address system and in accordance with the Class requirement.

For the engine room and steering gear room, the light signal column shall be used for the alarm device. For main deck area, ship’s horns shall be used for this purpose.

The general emergency alarm may be interfaced with public address system also.

“ Ref. SOLAS Ch. III Reg. 6 Clause 4.2”

    - 8.9.9 E/R SIGNALING AND ALARM SYSTEM (LIGHT COLUMN)

Signaling and alarm system (alarm lighting column) shall be provided in engine room, alarming engine room staff if specified alarm condition happen or telephone is calling in engine room or in ECR.

In engine room, include boiler, AC room, steering gear room, suitable amount of signal light panels shall be installed, with the following illuminated signaling/alarm identification signs according to IMO Resolution:

-	Fire alarm – red globe with “FIRE” sign

-	General alarm – green globe with “ALARM” sign

-	CO2 alarm – red globe with “CO2” sign

-	Engine failure alarm – amber globe with “FAILURE” sign

-	Telephone communication call – white globe with “TELEPHONE” sign

-	Telegraph alarm – white globe with “TELEGRAPH” sign

-	Dead man alarm – amber globe with DEAD MAN ALARM sign

Simultaneously with alarm display in signal light panels, rotating lights (with red or amber globe) and electric sirens shall be actuated.

Telephone calling signal shall be stopped automatically if no acknowledgement follows in abt. 4 min.

The quantity and position of columns shall be an example which can be changed by the final designing:

4-	Upper platform in E/R

4-	Lower platform in E/R

2-	Floor in E/R

1-	Steering gear room

1-	Purifier room

1-	Engine casing

A mini column panel for alarm shall be installed in the engine control room.

Abt : 14 pieces.

Power supply – AC 220 V from emergency mains;

"Ref. Appendix of "Code on Alerts and Indicators, (2009)"

    - 8.9.10 ELECTRIC TACHOMETER FOR MAIN ENGINE

One (1) set of electric tachometer system shall be provided, measuring and indicating the speed in revolution per minute and the direction of rotation.

One (1) set transmitter shall be attached to the main engine and each one (1) receiver shall be installed as follows:

- Wheelhouse ceiling console (with separate dimmer)

- Both bridge wings wall type (with dimmer)

- C/Engineer’s day room

- Engine control console

- M/E machinery side

One (1) revolution counter and one (1) running hour counter shall be installed in the engine control console.

Illumination with dimmer switch shall be provided for the tachometer receivers in wheelhouse ceiling console and bridge wings.

    - 8.9.11 RUDDER ANGLE INDICATOR

One (1) set of electrical rudder angle indicator system shall be provided as follows:

- One (1) transmitter with link bar stick in the steering gear room

- One (1) three (3) face type indicator in the wheelhouse

- One (1) indicator on wheelhouse ceiling console

- One (1) indicator in the engine control console

- Each one (1) indicator of watertight type on both bridge wings

- One (1) indicator in the steering gear room

- One (1) indicator on the navigation control console

Illumination with dimmer switch shall be provided for the rudder angle indicators in the wheelhouse and bridge wings.

    - 8.9.12 PUBLIC ADDRESS WITH TALK BACK SYSTEM

One set of public addresser with talk back system shall be provided as follows:

One (1) - About 200 watts double amplifier ,the control panel/talk back shall be in the WHC.

Six (6) -	Talk back station, each one (1) at the bridge wings, life boat embarkation station, ECR, forecastle deck and main deck after.

One (1) -	50 watts of water-proof type speaker, on the compass deck forward.

Eight(8) - 10 watts of water-proof type speakers, each one (1) on the forecastle deck, life boat embarkation stations,  main deck after, four(4)  in the engine room;

Fourteen (14) - 5 watts of water-proof type speakers, each one (1) at steering gear room, galley, AC room, CO2 room, E/R workshop, emergency generator room, bow thruster room and purifier room

Necessary quantity – 2 watts speakers of indoor type, in the engine control room, pantry, mess room, gymnasium, each lavatory and inner passage of each deck and other cabin if necessary (The speakers for inner passages shall of double face type).

The switches for selection of the speaker group shall be provided on the control panels and the speakers shall be grouped as follows:

- Compass deck

- Upper deck fore and aft

- Accommodation

- Engine room

- Emergency

The system to be arranged to operate in following orders of priority:

1st	-Emergency broadcasting

2nd		-Emergency general alarm

3rd 		-Broadcasting or paging from telephone

4th 		-Broadcasting of entertainment programs

Note:

1. Talk back system maybe is separated type and to be in accordance with Maker’s standard.

2. The public address system maybe is changed according to final selected Maker's standard.

3. Radio tuner and CD player to be provided r. Any entertainment system to be automatically turned off when the general alarm system is activated.

    - 8.9.13 SHIP'S HORNS

One whistle system with air horn, electric motor horn complete with automatic fog signal control unit shall be provided.

Air horn with magnetic valve shall be mounted on the radar mast; electric motor horn shall be mounted on the fore mast; one (1) time controller shall be in the wheelhouse control console.

Manual operation facility shall be provided for the air horn.

The horn shall be capable of automatic blowing for fog signal by means of time controller; it may be operated by push button switches also as following:

2- water tight type for wings;

2- for bridge console near the OOW seat;

2- at the front bulk of wheelhouse where the OOW usually stay for watching

In addition, the air horn on the radar mast shall be operated by mechanical pull from the wheelhouse or one DC 24V solenoid valve to be provided for emergency operation purpose.

The horn light may be used in common with Morse signal light also. And the light control function will meet requirement of IMO MSC.253 (83)

Electric heaters for the air horn and electric motor horn shall be provided.

    - 8.9.14 FIRE DETECTING AND ALARM SYSTEM

(1)	One (1) set of fixed addressable type fire detection and fire alarm system shall be so installed and arranged as to provide smoke detection in all corridors, stairways and escape routes within accommodation spaces as per the SOLAS IC requirement:

One (1) - Fire alarm panel in the wheelhouse

One (1) –Fire alarm repeater in fire control station

One (1) –Fire alarm repeater in ECR

Heat detector - in galley, recreation room and engine room

Smoke detector - in passage, stair of accommodation, laundry, engine room, steering gear room and other machinery space; Smoke detector in E/R workshop shall be with blocking time controller (0~2 hours)

Push button - in passage near the exit, wheelhouse, engine control room, entrance to the E/R, steering gear room, emergency generator room etc.;

Fire detection and alarm system shall be independent and not to be combined with fixed water mist fire fighting system in engine room.

I.S. type smoke detector shall be provided in paint store, battery store, and other dangerous area.

Emergency batteries to be built in fire alarm panel, when main net failure, the emergency batteries to support the system working normally as per rules.

Stairway doors closer shall be connected to fire alarm system for automatic closing after 2 mins of fire alarm.

The fire alarm signal to be sent to VDR.

(2)	One set of smoke detection and alarm system for cargo hold shall be provided as follows:

Detecting and monitor panel:	CO2 room

Exhaust fan motor box:	Suitable location

Some smoke sample intake:	Suitable location in cargo hold

Detecting and monitor repeater:	Wheelhouse

Both systems should interface with the VDR.

    - 8.9.15 FIXED GAS DETECTION SYSTEM

None

    - 8.9.16 ELECTRIC CRYSTAL CLOCK

One (1) set of electric crystal clock system shall be provided as follows:

1 -Master clock with slave clock control panel in chart space.

The master clock unit shall be incorporated with GMT and local time indication and slave clock control device such as advance and reverse control switch etc.

The Master clock shall be interfaced with DGPS.

Two hands slave clock:

Decorative type:	Each captain class day room

Mess room

Recreation room

Gymnasium

Chief. Officer

Second engineer day room

Ship office

Ordinary type:	Each one (1) double side type in corridor for each deck of accommodation

Each office

Hospital

Pantry

Galley

Three hands slave clock:	Wheelhouse control console,

Engine control room

Three hands slave clock (UTC type):	   Radio space

    - 8.9.17 PC LOCAL AREA NETWORK

1- LAN system (Owner supply; 10 stand socket, CAT.6 type cable wiring and sockets shall be supplied by the Builder only)

This system to be connected to VSAT.

    - 8.9.18 E/R local water mist alarm system

One (1) set fire alarm control panel of local water mist system shall be in the fire control station.

One (1) set local pumps unit and starters in E/R.

One (1) set repeater panel in wheelhouse control console and alarms connected to the AMS

Other flame detector, manually operated call points, flash lamp & sirens etc. will be provided in each local mist area (i.e. M/E, A/E, F.O. purifier, boiler area etc.) as per rule.

  - 8.10 NAUTICAL INSTRUMENT

Equipment and arrangement of nautical instrument will be supplied as per the rules requirement.

    - 8.10.1 Magnetic Compass (AC 220 V, DC 24 V)

One set of transmitting type magnetic compass (TMC) with reflector binnacle shall be installed. Transmitting magnetic compass signal should be connected to auto pilot.

Accessory to be provided as per the Maker’s standard.

Heading signal of TMC should be sent to the steering unit for auto steering and off-course alarm in the event of a gyrocompass failure.

One (1) spare bowl will be provided. Heading signal of TMC should be sent to the steering unit for auto steering and off-course alarm.

A length of bypass tube and facilities shall be considered as per the maker’s standard if the binnacle had to locate a distance from the top of the helmsman position in the wheelhouse.

Dimmer at bridge console for magnetic compass lamps of both DC24V and AC220V shall be provided.

    - 8.10.2 GYRO COMPASS (AC 220 V, DC 24 V)

One (1) set of gyro-compass shall be installed as follows:

1 – Master compass

1 – Digital repeater on W/H front ceiling console

1 – Bearing repeater with pedestal on each bridge wings

1 –Bearing repeater in steering stand of W/H

1 – Repeater in steering gear room

1 – Digital indication in captain’s day room

1 – Course recorder (only heading signal to be recorded) in chart space of W/H

1 – Rate of turn indicator on W/H front ceiling console

1 – Set of azimuth mirror and azimuth circle

1 – Repeater of heading type for conning position 1.of PANAMA required.(acc.rules)

Heading output signals shall be sent to Inmarsat- Fleetboard Band if necessary, VDR, AIS, ECDIS, radars and auto-pilot etc. External signals from the speed log, DGPS etc. shall be received.

The rate of turn signal shall sent to the ECDIS

    - 8.10.3 AUTO PILOT (AC 220 V, DC 24 V)

One (1) complete set of adaptive auto-pilot shall be installed consisting of:

One (1) - Steering stand in the wheelhouse, which might be combined with the bridge consoles;

The steering stand shall be equipped with a steering mode select switch in order to select the operation mode of automatic steering or electric hand steering with follow-up or non follow-up control. Non follow-up control units for both bridge wings shall be provided.

The steering stand shall be provided with power failure alarm and off course alarm.

The feedback unit shall be arranged in steering gear room with level link rod as per the maker’s standard .

Auto pilot override tiller to be provided in main console on wheelhouse.

The heading signal from the magnetic compass and gyro compass shall be connect in the autopilot separately.

    - 8.10.4 ECHO SOUNDER (AC 220 V)

One (1) set of echo sounder with necessary accessories shall be installed as follows:

1- Transducer in bottom of fwd  working frequency type of transducer to be 200 kHz according to Maker’s standard,

(Option: 1- Transducer in bottom of stern, and the bridge screen at the same time can display the depth of F/A);

1- Main display in the chart space

1- Display in wheelhouse ceiling panel.

The maximum depth range scale is 300 meters or as per Maker’s standard and signal shall be transmitted to VDR and ECDIS.

    - 8.10.5 SPEED LOG (AC 220 V)

The speed log must meet the requirement of Resolution MSC.334 (90).

(Option:Speed log on the bridge display screen, there are speed of water or speed of ground, were displayed the direction of flow and arrows of velocity indicator.)

One (1) set of Doppler speed log shall be provided as follows:

1- Transducer with gate valve fitted in ship forward bottom

1- Electronic unit in fore room

1- Main unit in chart space

1- Speed indicator at wheelhouse ceiling console

The speed signal shall be sent to two radars, AIS and VDR and autopilot etc.

    - 8.10.6 RADAR PLANT (AC 440 V/AC 220 V)

Two (2) sets of marine raster scan radar equipment shall be provided at wheehlouse, (Option:both Radars were installed the Uninterruptible Power System(UPS) )

(Option: A monitoring screen in the Master’s room, and there should be a switch to change the screen to the S/band or X/band.)

The particulars to be as follows:

|  | No.1 | No.2 |
| --- | --- | --- |
| Frequency band: | X-band | S-band |
| Peak output power: | abt.25 kW | abt.30 kW |
| Min. Display area | 340 mm x 340 mm at least | 340 mm x 340 mm at least |
| Display mode | relative/true motion | relative/true motion |
| Scanner size | 7~ 9 feet | 12 feet |
| Performance monitor | fitted | fitted |
| ARPA | with | with |

The data from AIS shall be capable of being displayed on TFT of both radars.

Gyro, AIS, speed log and DGPS signals shall be sent to X-band and S-band radar.

Both screens will be at least24" color TFT screens.

Above radars performance standard should be to meet IMO resolution MSC.192 (79).

    - 8.10.7 ELECTRONIC CHART DISPLAY & INFORMATION SYSTEM (AC 220 V)

Electronic chart display & information system (ECDIS) will fully comply with the latest standards and resolutions of IMO, IHO, and IEC. The electronic chart is compatible with ENC (S57 Edition 3) charts, C-MAP, ARCS charts etc. The electronic chart can be overlaid with a variety of navigation data such as radar images, position, heading, speed, and others to facilitate safe and efficient navigation.

Two (2) independent set of ECDIS (master & back-up) shall be provided; (Option:both ECDIS were installed the Uninterruptible Power System(UPS)}

(Option:A monitoring screen shall be arranged in the Master’s room, which may be combined with the one screen of radar in the room (if it were be)

They shall be supplied via navigation power distribution box which to be fed from MSB and ESB auto-changeable.

The ECDIS shall have following main functions:

Electronic chart;

Radar image superimposition;

Continuous monitoring vessel position;

Route planning and route monitoring etc.

Display of ECDIS to be 24".

External signals from echo sounder, anemometer, AIS, DGPS, gyrocompass, speed log and radars etc. shall be received.

    - 8.10.8 DGPS NAVIGATOR

Two (2) sets of DGPS navigator with necessary accessories shall be provided: one set in wheelhouse control console and one set on the chart table.

Each 1 - Main display unit in chart space and on WHC

2 - Antenna on the after mast or compass deck

1 - Signal distribution box in wheelhouse

The GPS signal shall be sent to the following equipment: Inmarast-Fleet broadband , Inmarast-C, MF/HF DSC, VHF DSC, Radars, speed log, Echo sounder, VDR ,  gyro-compass,ballast water treatment, scrubber and other systems requirement etc.

    - 8.10.9 ANEMOMETER and ANEMOSCOPE (AC 220 V)

One (1) set of anemometer of propeller type (Option: ultrasonic type)shall be provided.

1-Transmitter on the radar mast

1-Wind speed and direction indicator with illumination on the W/H ceiling console

Interface facility to ECDIS shall be provided.

    - 8.10.10 WINDOW WIPER and CVS (AC 220 V)

Abt. Three(3) sets of window wipers with heater and Two(2) CVS shall be installed on the front windows of the wheelhouse according to rules requirements. (Refer to Accommodation Part)

The wiper shall be of horizontal movement type. Speed controller at the bridge console shall be provided for each window wiper.

Each CVS controller shall be located at the lower part of front bulkhead

    - 8.10.11 VOYAGE DATA RECORDER VDR (AC 220 V, DC 24 V)

One (1) set of voyage data recorder system VDR which performance standard will meet the requirement of Resolution MSC.333 (90) shall be provided as follows:

One (1) main electronics unit for data acquisition and recording shall be installed in the suitable space of bridge deck.

One (1) capsule (black box, auto floating type) and One (1) capsule (black box, fixed type) shall be fixed on compass deck and shall be capable of being removed but secure against tampering.

Interface box and power supply unit shall be provided. The data shall be easy download without any special needs to a hard drive and can be play back on a normal PC.

Six (6) sets of microphones to be provided, two (2) outdoor type microphones on both bridge wings and four (4) indoor type microphones in the wheelhouse.

VDR extension backup cable for captain day room computer will be provided and the computer shall be supplied by Owner.

Gyro compass, speed log, echo sounder, at least 2 stations radar video, DGPS, rudder order, engine order, wind speed and direction, VHF communication audio, alarm and other information requirement by rules shall be sent to the VDR.

    - 8.10.12 AUTOMATIC IDENTIFY SYSTEM (ais) (DC 24 V)

One (1) set of AIS with necessary accessories shall be provided.

One (1) set antenna shall be installed on compass deck and one (1) controller shall be installed in the wheelhouse.

One (1) interface for communicating between AIS and pilot’s computer shall be provided, and one (1) power receptacle with supply AC 110 V (USA type socket) for pilot’s computer shall also be provided.

AIS shall interface with Radar and ECDIS units to display the AIS information on Radar’s TFT and ECDIS display.

Gyro and DGPS signal and other information requirement by rules shall be sent to AIS. The system may be fed from DC 24 V and AC 220 V.

    - 8.10.13 WHEELHOUSE CONTROL CONSOLE (WHC)

In case that the contents in the following parts of this section differ from Maker’s standard for specific equipment, Makers’ standard shall be applied in general.

The wheelhouse control console should consist of three parts, the navigation signal console (NSC), bridge maneuvering console (BMC) and steering stand. The following equipment specified in other part shall be installed at the consoles but not limit to.

      - 8.10.13.1 Navigation Signal Console:

- Navigation light panel

- Signal light panel

- Outside light control panel

- Distribution boards for lighting, nautical equipment

- Emergency stop switch for the accommodation fans

- Emergency stop switch for the cargo hold fans

- Remote start and stop buttons for fire/G.S. pump and emergency fire pump

- Dimmer switches

- Hold fan emergency stop switch

- Hospital alarm buzzer

- Refrigerated chamber alarm buzzer with lamp

- And other necessary equipment

- AIS display unit

- X band radar display unit

- Extension alarm panel with group alarm indication

- General alarm control panel

- Window wiper controller (possible by the designing)

- Gyro compass component

- S-band radar display unit

- VDR control panel

- Engine order telegraph logger

- No.2 DGPS receiver

- No.2 VHF radiotelephone

- Bridge navigation watch alarm system (BNWAS)

- ECDIS

- Remote distress alarm push button for GMDSS

- Other necessary facilities

      - 8.10.13.2 Bridge Maneuvering Console (BMC)：

- Main engine remote control unit

- Maneuvering lever and telegraph

- Bow thruster control unit

- ECDIS (with conning function)Auto telephone set

- Ship’s horn controller

- Automatic telephone and common battery telephone sets

- Public address control unit. Dimmer switches

- And other necessary equipment

- No.1 VHF radiotelephone

The wheelhouse control console shall be supplied from the AC 220 V normal supply system, the AC 220 V emergency supply system and the DC 24 V supply system.

    - 8.10.14 Docking working station

At each bridge wing including:

| 1 | - | 5 W speaker |
| --- | --- | --- |
| 1 | - | Gyro repeater (pedestal type) |
| 1 | - | Whistle push button |
| 1 | - | Key for morse/manoeuvring light |
| 1 | - | Bridge wings search light and switch |

2  -	Sockets  for VHF handset.

1  - 	Panama type main engine revolution indicator

1  -  Panama type steering indicator

1  -  VDR microphone

1  -  BNWAS reset switch

Bridge wing console including:

-Main engine  control panel for wings

-Bow thruster control unit for wings

-Non-follow-up tiller of autopilot

-Microphone receptacle for talk back system

-Dimmer switch

And other necessary equipment;

    - 8.10.15 MEASURING INSTRUMENT PANEL

One (1) measuring instrument panel shall be provided at the wheelhouse ceiling console and following equipment shall be mounted:

- Electric slave clock

- Digital type gyro compass repeater

- Revolution indicator for ME

- Anemometer

- Ship’s speed indicator

- Depth indicator of echo sounder

- Rate of turn indicator

    - 8.10.16 CHART TABLE

One (1) chart table to be provided in the wheelhouse and following instruments to be mounted:

- Master clock with slave clock control panel

- Speed log indicator master panel

- No.1 DGPS

- Weather facsimile recorder

- Chart table lights with dimmer

- Course recorder

- Echo sounder main unit

- Navtex receiver

    - 8.10.17 RADIO TABLE

One (1) radio and safety plan table to be provided in wheelhouse and the following instruments to be mounted:

- Data terminal for Inmarsat C

- Printer for Inmarsat C

- MF/HF radio controller

- Battery charger

- Low voltage power distribution box

    - 8.10.18 STEERING STAND

One (1) steering stand to be provided as following:

| 1 | - | Manual hand- wheel for the helsman |
| --- | --- | --- |
| 1 | - | Gyro repeater |
| 1 | - | Heading monitor |
| 1 | - | Steering gear alarm panel |
| 1 | - | Dimmer switches |

The steering stand can be combined into the wheelhouse control console; a UHF handset of hand-free type shall be prepared for the intercom with the docking workstation.

  - 8.11  RADIO COMMUNICATION EQUIPMENT

    - 8.11.1 GENERAL

Radio communication equipment shall comply with the International Telecommunication Radio Regulations and the SOLAS Convention (GMDSS) for A1, A2, A3 sea area and adopt shore-based maintenance.

    - 8.11.2 MF/HF RADIO EQUIPMENT (AC 220 V, DC 24 V)

One (1) set of MF/HF radio equipment shall be installed in radio space.

1- 250W MF/HF transmitter

1- Self-supporting Tx antenna

1- MF/HF receiver

1- MF/HF receiving whip antenna

1- MF/HF DSC terminal

1- MF/HF DSC receiver and whip antenna

1- Radio controller

1- Battery charger

1- Set of necessary accessories according to Maker’s standard

    - 8.11.3 VHF RADIO TELEPHONE

Two (2) sets of VHF radiotelephone of DSC with 57 channels and USA channel shall be installed in wheelhouse control console.

Each set including:

1-VHF telephone with DSC and whip antenna

1-VHF DSC watch receiver (channel 70) with antenna

The No.1 VHF shall be installed at Navigation Signal Console in hands reach area near the OOW seat, with one(1) remote control handset at the front bulk of wheelhouse.

No.2 VHF shall be installed at Navigation Console in hands reach area near the OOW seat, with slave sockets for both wings .

One (1) portable handset will be provided for both wings.

DC 24V Power should be fed from the radio battery.

    - 8.11.4 SATELLITE COMMUNICATION SYSTEM

(1)	INMARSAT-Feet Broadband FB 500 (AC 220 V)

One (1) set INMARSAT Fleet broad band 500 standard will be installed, consisting of:

1- Main unit

1- Telephone (in captain day room)

1- Facsimile in radio space

1- Antenna on compass deck

1- AC/DC power supply unit

(2)	INMARSAT-C (AC 220 V / DC 24 V)

Two (2) sets of standard-C satellite communication SES with EGC reception (One of them may be combined with LRIT and SSAS function), each one shall be provided as follows:

1-Main unit

1-Terminal unit

1-Printer

- Antenna

DC 24 V Power shall be fed from the radio battery.

    - 8.11.5 NAVTEX RECEIVER

One (1) set of Navtex receiver (518 kHz & 490 kHz) with printer shall be provided in chart space. The system to be fed from DC 24 V and AC 220 V. Printer is according to maker’s standard.

    - 8.11.6 UHF handset

Six (6) sets of portable UHF handset (400 MHz, including two explosive type, one hand-free type with certificate) and rechargeable battery and charger and holster, microphone shall be provided.

    - 8.11.7 WEATHER FACSIMILE RECORDER

One (1) set of weather facsimile recorder shall be provided in chart space

    - 8.11.8 	SATELLITE EPIRB

One (1) set of self-float type emergency position indicating radio beacon operating on 406/121.5 MHz shall be fitted at the open part of bridge deck.

    - 8.11.9 RADAR TRANSPONDER

Two (2) sets of radar transponder operating on 9 GHz shall be provided. Four(4) brackets shall be supplied, which two of them shall be installed in the lifeboats.

    - 8.11.10 TWO-WAY RADIOTELEPHONE APPARATUS

Three (3) sets of two-way VHF with battery charger and 3 sets of primary batteries shall be provided as per the rules.

    - 8.11.11 SHIP SECURITY ALERT SYSTEM (SSAS)

One (1) set of ship security alert system in compliance with the requirement of SOLAS will be installed. SSAS may be combined with Inmarsat C system.

Each one (1) security alert button shall be installed at wheelhouse and captain day room. The system will be fed from DC 24 V and AC 220 V.

    - 8.11.12 Long range Identification and Tracking (LRIT)

One (1) set long range identification and tracking system (LRIT) will be provided.

LRIT may be combined with Inmarsat C system.

    - 8.11.13 BROADCAST & TELEVISION RECEIVING ANTENNA SYSTEM (AC 220 V)

One (1) set of broadcast & TV receiving antenna system shall be provided as follows:

1 – Omni-direction type antenna for TV/FM/AM

Each 1 – Antenna outlet for radio and TV receiver in each private room, office, hospital, mess room and recreation room.

This system should be compatible for future SAT TV jumping in.

In Cabins and Public areas where entertainment appliances are provided, an extra power socket should also be provided at each area.

Broadcast receivers and TV sets etc. shall be supplied by Owner, the Builder shall be in assistant to fix and secure the TV sets by practice.

8.11.14  vsat

Owner supply One set VSAT, shipyard in charge of cable and cable bracket installation.

SUPPLEMENTARY OUTFITS AND SPARE PART

    - 8.11.15  SUPPLEMENTARY OUTFITS

One (1) - universal meter

One (1) - 500 volts portable megger One (1) - portable clamp - on ammeter One (1) - diagonal cutting pliers

Two (2) - cutting pliers

One (1) - set of screw driver

One (1) - Jack knife

Each one (1) - soldering iron (AC 220 V 25 W, 75 W, 150 W and 1 kg soldering wire) One (1) - pistol drill

One (1) - set of steel file

Each one (1) - monkey wrenches (6”, 8”, 10”) One (1) - set of open end wrench

One (1) - set of socket wrench

One (1) - hand bow saw

One (1) - hammer

One (1) - wire stripper

One (1) - convolution ruler (3 m) One (1) - tool box.

    - 8.11.16 SPARE PART

The necessary number of spare parts shall be supplied according to requirements of the

Classification Society and Maker's standard.

All spares to be placed in steel boxes duly labeled/marked and boxes shall be placed in shelves in spares store room.